# 3GPP TSG-RAN WG4 Meeting #111                                                                R4-2407948

# Fukuoka, Japan, 20 May – 24 May, 2024

Agenda Item:	6.17.2

Source: 	CMCC, Murata Manufacturing Co Ltd., ZTE Corporation

Title: 	(HPUE_FR1_TDD_NR_CADC_SUL_R18) TP for TR 38.899 to introduce PC2 and PC1.5 CA_n3A-n40A

Document for:	Approval

- Introduction

The request of CA_n3A-n40A have been approved.

In this contribution, we will analyze MSD studies for these band combinations.

- Reference

- . TR38.899, High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s), 0.8.0

- TP to TR 38.899

## <<Start of Change for TR 38.899>>

  - 5.x	CA_ n3A-n40A

    - 5.x.1	UE maximum output power

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier |  | NR Band |  | Channel bandwidth (MHz) |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n40A | n408,9 CA_n3A-n40A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 2 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |

5.x.2	Maximum output power

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n40 | CA power class | Band n3 power class | Band n40 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n40A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

    - 5.x.3	Reference sensitivity requirements

5.x.3.1	Power class 2 for case A

As existing requirements for PC3 CA_n3-n40, there is no MSD issue.

5.x.3.2	Power class 2 for case B

Cross band isolation interference of PC2 n40 falls into n3.

The PC2 MSD due to cross band isolation are specified in following table 5.x.3.2-1.

Table 5.x.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n3 | 2350 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 0.6 | >ACLR2 |

5.x.3.3	Power class 1.5 for n40

Cross band isolation interference of PC1.5 n40 falls into n3.

The PC1.5 MSD due to cross band isolation are specified in following table 5.x.3.3-1.

Table 5.x.3.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n3 | 2350 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 1.2 | >ACLR2 |

5.3.4	∆TIB and ∆RIB values

Void

## <<End Change for TR 38.899>>