3GPP TSG-RAN WG4 Meeting #111	R4-2407961

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title:	Intra-band EN-DC channel spacing issue

Source:	Qualcomm Incorporated

Agenda item:	4.1

Document for:	Approval

# 1.	Introduction

In RAN4#110 Athens RAN4 received a proposal [1] to chance the scope of Rel-15 intra-band non-contiguous EN-DC to extend the channel spacing to cover also the case when channel spacing is equal to nominal channel spacing:

The proposal was not agreed due to complexity in procedures of updating new features to the existing fieled devices. This paper discusses technical problems related to the proposal in view of the repeated proposal in [2,3].

# 2. 	Discussion

The clause 5.4B.1 defines the difference between intra-band contiguous and non-contiguous so that the contiguous deployment is when channel spacing is equal to nominal and non-contiguous deployment is when channel spacing is larger than the nominal. Figure 1 explains the current definition left side and also new proposal.

Figure 1. intra-band EN-DC deployments and new proposal

The new proposal would create a superposition of the deployment, when channel spacing is exactly equal to nominal channel spacing, the deployment, and requirements, or both contiguous and non-contiguous intra-band EN0DC would apply.

Observation 1: Defining intra-band non-contiguous EN-DC to include case when channel spacing is equal to nominal creates a superposition of requirements.

The motivation as written in [1] is to make deployed devices that support intra-band non-contiguous attach to the network when the channel spacing is equivalent to contiguous. It should be noted that changing documentation of a closed release in 3GPP do not change the behaviour of the fielded devices automatically but Over The Air modem software update is needed which then is responsibility of the device manufacturers. User can also deny such updates so there are no guarantees the update can reach the intended population of devices.

Changing a closed release invalidates all devices with the support for the corresponding feature. I also makes the 3GPP specification invalid. Devices that have passed conformance for this feature may or may not be compliant with the change, there is no possible way without performing the conformance test again with the updated requirements for those old devices.

The complications described above is why 3GPP frowns upon Non-Backwards Compatible (NBC) changes.

Observation 2: Changing specifications of the closed release with functional changes will create an unrecoverable mismatch between 3GPP specifications and fielded devices

New features can be added to closed releases but then they need to be distinguished clearly with a new capability so that new design can adopt this feature and old devices are still compliant since they do not declare the support for this new feature.

Observation 3: New features can be added to closed releases under new capabilities.

New features without a capability are mandatory for any UE supporting that release and changes to a feature with a capability are also mandatory for UEs supporting that feature. It is not a good idea to do functional changes to existing feature.

Proposal 1: Ran4 will not define new mandatory features to closed releases

## 2.2	How to handle the superposition of requirements

If the RAN4 were to enable the nominal channel spacing for non-contiguous EN-DC, the applicable requirements would need to be defined. Without any indication, UE could choose which requirements apply and this would make defining testable conformance requirements impossible.

Observation 4: Superposition of requirements make conformance testing impossible

RAN4 can not change the contiguous CA or non-contiguous CA since then this would be NBC change. One way to solve this problem is to define the capability with two fields as follows:

Intra-bandEN-DC-NC_with_nom_Ch_spacing:

UE that declares support for [] supports intra-band non-contiguous with nominal channel spacing.

Intra-bandEN-DC-NC_with_nom_Ch_spacing_with_NC_reqs:

UE that supports Intra-bandEN-DC-NC_with_nom_Ch_spacing applies non-contiguous intra-band EN-DC requirements when channel spacing is exactly nominal.

Intra-bandEN-DC-NC_with_nom_Ch_spacing_with_Cont_reqs:

UE that supports Intra-bandEN-DC-NC_with_nom_Ch_spacing applies contiguous intra-band EN-DC requirements when channel spacing is exactly nominal.

Technically solving the problem is possible but it raises the question why does 3GPP need to extend one requirement to cover the deployment of an other requirement. It would be easier to just build device with contiguous intra-band EN-DC.

Observation 5: 3GPP already has a feature to cover the deployment of intra-band EN-DC with nominal channel spacing.

There may be a technical problem with applying non-contiguous requirements for nominal channel spacing, such as a parameter used in MPR or AMPR is not defined when channel spacing is nominal. It is unknow to the author now if there is one but the proponent of this feature should provide an analysis of the requirements.

Proposal 2: Ran4 to carefully analyse impact of the applicable requirements before agreeing to changes

One issue to be considered is that currently the supported bands for contiguous and non-contiguous intra-band EN-DC are not the same. Band n1 is supported in the non-contiguous but not in contiguous side. So this change would enable band n1 to be supported with nominal channel spacing but there are no requirements for contiguous EN-DC for n1.

Observation 6: The change would possible make requirements for contiguous EN-DC redundant

We also notice that n1 has no NS associated, it is not clear a the time of writing this why. If Ran4 were to engage in this activity, the workload maybe significant.

## 2.3	NR-U equivalent case

It was referred in the discussions that the NR-U recently made a similar change, to define that the contiguous CA covers also a case when channel spacing is larger than the nominal channel spacing. This change came from different motivation since NR-U follows Wifi channelization and in NR-U domain, the contiguous CA was not possible at all. The changes therefore enabled contiguous CA. NR-U may nee to also define separately that the non-contiguous CA is only defined for the “the possible next channel” location.

Observation 7: NR-U changes did not create the superposition of the requirements problem as described in section 2.2 due to unique channelization of NR-U.

# Conclusion

We discussed the change proposal in [1] and made seven observations

Observation 1: Defining intra-band non-contiguous EN-DC to include case when channel spacing is equal to nominal creates a superposition of requirements.

Observation 2: Changing specifications of the closed release with functional changes will create an unrecoverable mismatch between 3GPP specifications and fielded devices

Observation 3: New features can be added to closed releases under new capabilities.

Observation 4: Superposition of requirements make conformance testing impossible

Observation 5: 3GPP already has a feature to cover the deployment of intra-band EN-DC with nominal channel spacing.

Observation 6: The change would possible make requirements for contiguous EN-DC redundant

Observation 7: NR-U changes did not create the superposition of the requirements problem as described in section 2.2.

And two proposals

Proposal 1: Ran4 will not define new features to closed releases

Proposal 2: Ran4 to carefully analyse impact of the applicable requirements before agreeing to changes

# Reference

[1]	R4-2400864, (NR_newRAT-Core)R15 Cat-F CR 38.101-3 channel spacing for intra-band EN-DC, Huawei, HiSilicon, 3GPP TSG-WG4 Meeting #110, Athens, Greece, February 26 – March 01, 2024

[2]	R4-2407615, (NR_newRAT-Core) Discussion on the channel spacing for intra-band EN-DC, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

[3]	R4-2407616, “R17 Cat-F CR 38.101-3 channel spacing for intra-band EN-DC”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

[4]	R4-2407617, “R18 Cat-A CR 38.101-3 channel spacing for intra-band EN-DC”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

[5]	R4-2406677, “WF on NR-U Nominal channel spacing”, Skyworks, Nokia, 3GPP RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th, 2024