3GPP TSG-RAN4 Meeting #111	R4-2407985

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source:	LG Electronics

Title:	ATG UE RF requirements for UL-MIMO

Agenda item:	10.8.2.3

Document for:	Discussion

Introduction

In this paper, we provide our views on ATG UE RF requirements for UL-MIMO based on the approved WF[1].

Discussion

Based on the approved WF[1], we would like to discuss the ATG UE RF requirements for UL-MIMO.

Issue 3-4-1: clarification of UL-MIMO Agreement: - Introduce UL MIMO requirements with 2Tx Issue 3-4-2: max layer number for UL-MIMO Agreement: - 2 layers Issue 3-4-3: ULFPTx mode for UL-MIMO Agreement: - FFS on whether ULFPx mode is supported for ATG UL-MIMO Issue 3-4-4: the operation band for UL MIMO Agreement: - UL MIMO apply to all existing ATG operation bands Issue 3-4-5: maximum output power Agreement: Use following as starting point. - For UE maximum output power, reuse the rated maximum output power declared by the ATG UE capability maxOutputPowerATG-r18, and consider the existing NR UL-MIMO requirement as starting point with the capability antennaArrayType-r18 Issue 3-4-6: configured transmitted power - Proposals - Option 1: For configured transmitted power, reuse the existing ATG UE requirement with PCMAX,c tolerance of Table 2.1. (LGE) Table 2.1: ATG PCMAX,c tolerance in closed-loop spatial multiplexing scheme PCMAX,c (dBm); Tolerance TLOW(PCMAX_L,c) (dB); Tolerance THIGH(PCMAX_H,c) (dB). 23 ≤ PCMAX,c ≤ 40; 3.0; 2.0. 22 ≤ PCMAX,c < 23; 5.0; 2.0. 21 ≤ PCMAX,c < 22; 5.0; 3.0. 20 ≤ PCMAX,c < 21; 5.0; 4.0. 16 ≤ PCMAX,c < 20; 5.0. 11 ≤ PCMAX,c < 16; 6.0. -40 ≤ PCMAX,c < 11; 7.0. - Option 2: tolerance of configured power for ATG UE with UL MIMO needs to be specified. (CMCC) - Option 3: Configured transmitted power for UL MIMO is per each UE (ZTE) Agreement: - FFS on the configured transmitted power requirements Issue 3-4-7: MPR/AMPR Agreement: - Do not define MPR/A-MPR requirements Issue 3-4-8: output power dynamic Agreement: - Reusing the existing NR UL-MIMO requirement as starting point. - Further discuss how to modify the requirements with capability antennaArrayType-r18 - Further discuss whether preclude the single antenna port related requirement Issue 3-4-9: transmit signal quality Agreement: - Reusing the existing NR UL-MIMO requirement as starting point. - Further discuss - how to update the requirements with capability antennaArrayType-r18 - whether to preclude the single antenna port related requirement - whether to apply the coherent UL MIMO requirement for ATG UE - how to update with ATG UE transmit signal requirement which is different from normal UE due to some timing and frequency compensation assumption . Issue 3-4-10: Output RF spectrum emissions Agreement: - Reusing the existing NR UL-MIMO requirement as starting point. - Further discuss - how to update the requirements with capability antennaArrayType-r18 - whether to preclude the single antenna port related requirement - how to update with ATG UE SEM requirement which is different from normal UE SEM requirement due to higher transmission power Issue 3-4-11: Rx requirements for supporting UL-MIMO Agreement: - FFS on the Rx requirements for supporting UL-MIMO

UE Tx RF requirement

The view for ATG UL-MIMO with 2Tx is as follows.

UE maximum output power for ATG UL-MIMO with 2Tx

Considering the agreed maximum 2 layers, the following requirements can be defined with the capability antennaArrayType-r18.

For 2-layer UL MIMO operation, the rated maximum output power shall be met using 2-layer UL MIMO codebook-based transmission with precoding matrix of W= . DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

For single layer UL MIMO operation and two antenna-ports,

- the rated maximum output power shall be met to a UE that supports ULFPTx feature and is configured in its declared full power mode.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the rated maximum output power applies for sum of two antenna connectors for the ATG UE not indicating the capability antennaArrayType-r18.

- the rated maximum output power applies for sum of two TAB connectors for the ATG UE indicating the capability antennaArrayType-r18.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity and the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16,

- the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

ULFPTx mode for UL-MIMO

In NR, ULFPTx mode is specified for PC2 and PC1.5 as follows.

- Single layer for 2 Tx port & 4 Tx port

  - PC2/PC1.5

- 2 layers for 4 Tx port

  - PC1.5

For ATG single layer UL MIMO operation and two antenna-ports, whether to support ULFPTx feature needs to be discussed. Power class was not defined to AGT UE, instead of, the rated maximum output power (R-MOP) was introduced. The range is 23dBm to 40dBm with step of 1dB.

If 26dBm + 26dBm is implemented in ATG UE and R-MOP of 26dBm is declared, it can be supported with ULFPTx mode = ModeFull.

Table 2-1 shows the example of ULFPTx mode for R-MOP of 26dBm.

Table 2.1: Example of ATG ULFPTx (R-MOP = 26dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | 26 | 23~25 |
| Mode-Full | 26 | 26 |
| Mode1 | 23~25 | 23~25 |

Proposal 1: Consider ULFPTx mode for ATG UL-MIMO.

Configured transmitted power for ATG UL-MIMO with 2Tx

The existing ATG UE configured transmitted power can be reused with Table 2.1 tolerance.

For PCMAX,c tolerance, max PCMAX,C of 40 dBm and min PCMAX,C of -19 dBm are proposed. -19dBm is based on the endorsed [3].

Table 2.1: ATG PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 40 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -19 ≤ PCMAX,c < 11 | 7.0 |  |

Proposal 2: For configured transmitted power, reuse the existing ATG UE requirement with Table 2.1 PCMAX,c tolerance.

Output power dynamics for ATG UL-MIMO with 2Tx

Minimum output power for ATG UL MIMO

Generally, NR UL-MIMO can be reused.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.3J.1 apply.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the requirements in clause 6.3J.1 apply as the sum of the mean power at each transmit antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.3J.1 apply as the sum of the mean power at each transmit TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

Transmit OFF power for ATG UL MIMO

Generally, NR UL-MIMO can be reused.

Transmit ON/OFF time mask for ATG UL MIMO

Generally, NR UL-MIMO can be reused.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.3J.3 apply.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the requirements in clause 6.3J.3 apply at each transmit antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.3J.3 apply at each transmit TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

Power control for ATG UL MIMO

Generally, NR UL-MIMO can be reused.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.3J.4 apply.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the requirements in clause 6.3J.4 apply to the sum of output power at each transmit antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.3J.4 apply to the sum of output power at each transmit TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

Proposal 3: For output power dynamics, define the requirement for single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity, and with indicating txDiversity with the capability antennaArrayType-r18.

Transmit signal quality for ATG UL-MIMO with 2Tx

Frequency error for ATG UL MIMO

For ATG UE(s) supporting UL MIMO, the basic measurement interval of modulated carrier frequency is 1 UL slot.  The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift.

The mean value of basic measurements of ATG UE modulated carrier frequency per layer shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

Transmit modulation quality for ATG UL MIMO

Generally, NR UL-MIMO can be reused.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.4J.2 apply.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the requirements in clause 6.4J.2 apply at each transmit antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.4J.2 apply at each transmit TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

Proposal 4: For transmit signal quality, define the requirement for single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity, and with indicating txDiversity with the capability antennaArrayType-r18.

Output RF spectrum emissions for ATG UL-MIMO with 2Tx

Occupied bandwidth for ATG UL MIMO

Generally, NR UL-MIMO can be reused.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.5J.1 apply.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the requirements in clause 6.5J.1 apply at each transmit antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.5J.1 apply at each transmit TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

Out of band emission for ATG UL MIMO

Generally, NR UL-MIMO can be reused.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.5J.2 apply.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the requirements in clause 6.5J.2 apply to the sum of the emissions from all UE transmit antenna connectors for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.5J.2 apply to the sum of the emissions from all UE transmit TAB connectors for the ATG UE indicating the capability antennaArrayType-r18.

Spurious emissions for ATG UL MIMO

Generally, NR UL-MIMO can be reused.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.5J.3 apply.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the requirements in clause 6.5J.3 apply to the sum of the emissions from all UE transmit antenna connectors for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.5J.3 apply to the sum of the emissions from all UE transmit TAB connectors for the ATG UE indicating the capability antennaArrayType-r18.

Proposal 5: For output RF spectrum emissions, define the requirement for single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity, and with indicating txDiversity with the capability antennaArrayType-r18.

Conclusion

In this contribution, we provided our view on ATG UE RF requirements for UL-MIMO. Based on those, we propose as follows.

Proposal 1: Consider ULFPTx mode for ATG UL-MIMO.

Proposal 2: For configured transmitted power, reuse the existing ATG UE requirement with Table 2.1 PCMAX,c tolerance.

Table 2.1: ATG PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 40 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -19 ≤ PCMAX,c < 11 | 7.0 |  |

Proposal 3/4/5: Define the following requirements for single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity, and with indicating txDiversity with the capability antennaArrayType-r18.

- output power dynamics

- transmit signal quality

- output RF spectrum emissions

- References

[1] R4-2406954, “WF on Rel-19 ATG UE requirements”, CMCC

[2] RP-240839, “New WID on Enhancements for Air-to-ground network for NR”, Moderator(Apple)

[3] R4-2406593, “draft CR on MOP and configured transmitted power for ATG UE”, LG Electronics, Huawei