3GPP TSG RAN WG4 #111 meeting 	 	R4-2408045

Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	10.9.2.2 & 10.9.2.3

Source:	Meta Ireland

Title:	UE RF requirements for NR SL Intra-band non-contiguous CA

Document for:	Approval

- Introduction

In RAN #110bis meeting, RAN4 agreed WF on NR sidelink CA in ITS spectrum in Rel-19.

Based on the following WF, we suggest how to define RF core requirements for the intra-band non-contiguous SL CA in ITS spectrums in Rel-19. The following contents are agreed items and further discussion points for the SL-CA in ITS spectrum in Rel-19.

WF on NR SL CA WI in ITS spectrum in Rel-19 [1] Issue 2-2-1: UE RF architecture Similarly, for Rel-19 sidelink CA, based on the assumption of both 1TX and 2TX for intra-band non-contiguous NR CA, with different implementation of 1LO and 2LO for 2TX cases, below different cases are listed: - Option 1: 1PA + 1LO Figure 2-a PC2/PC3 intra-band non-contiguous CA assumption of 1PA+1LO - Option 2: 2PA + 2LO, Figure 2-b PC2/PC3 intra-band non-contiguous CA assumption of 2PA+2LO - Option 3: 2PA + 1LO Figure 2-c PC2/PC3 intra-band non-contiguous CA assumption of 2PA+1LO Similar as previous issue, as only 70MHz needs to be supported, hence the following WF is proposed: WF: - For PC3 and PC2 intra-band non-contiguous CA, it is proposed to use Option1 and Option 2 as a starting point assumption for MPR/A-MPR simulation. - RAN4 can consider all candidate RF architectures to derive PC3/PC2 MPR/A-MPR requirements for intra-band non-contiguous CA. - Further discuss these options in the next meeting. Issue 2-2-2: Simulation assumption - Agree on below table for simulation assumption. - The carrier leakage can be further discussed. Center frequency; 5.9GHz. Bandwidth; per CC: 10/20MHz aggregated CBW: 10+10MHz/ 10+20MHz. Maximum output power for aggregated CBW; 23dBm/26dBm. Numerology; 15 kHz/30kHz/60kHz. Modulation per CC; QPSK/16QAM/64QAM/256QAM. Waveform; CP-OFDM. ACLR; 30dBc(PC3) / 31dBc(PC2). Carrier leakage; 25dBc. IQ image; 34dBc. CIM3; 60dBc. PA calibration; PA calibrated to deliver 30dB ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case.. Issue 2-2-3: Frequency gap - Moderator WF: - for the intra-band non-contiguous SL CA, the frequency gap should be determined based on the real deployment scenarios in global countries. The detail frequency gap is FFS.

In this paper, we provide our preference and how to define the UE RF requirements for NR SL intra-band non-contiguous CA UE.

- UE RF requirements for NR Sidelink CA

  - Consideration on MPR/A-MPR requirements for NR SL intra-band non-contiguous CA in ITS spectrum

- Frequency gap for NR SL intra-band non-contiguous CA

To derive MPR/A-MPR requirements in the ITS spectrum, one important point is how much frequency gap between each sub-block is assumed.

In US, the total 30MHz CBWs are available for C-V2X operation from 5895 to 5925MHz as follows:

Figure 1. ITS spectrum allocation after the decision by US Court of Appeals [5][6]

In Europe, the candidate spectrum will be considered for both DSRC system and C-V2X system as follows:

Based on the spectrum designation plan, the Road C-V2X UE can be configured from 5875 MHz to 5915MHz. The total allowed channel bandwidth for road ITS service is 40MHz in n47.

Figure 2. Spectrum designations at 5.9Ghz in Europe [7]

Also, the Ministry of Science and ICT and the Ministry of Land in Korea has announced that cellular-V2X communication technologies will be used for next generation vehicle communication systems for the whole 70MHz in n47 [8]. Therefore, the maximum frequency gap (WGap) is 50MHz as worst case to derive MPR/A-MPR requirements for intra-band non-contiguous SL CA operation as follows:

Figure 3. Candidate frequency gap as worst cases for MPR/A-MPR simulation in ITS spectrum

Observation 1: The maximum frequency gap between each sub-block is about 50MHz in South Korea. The frequency gap in other countries is less than 50MHz.

Based on above observation, we propose as follows:

Proposal 1: On top of above MPR simulation assumptions in Table in issue 2-2-2, RAN4 should consider the frequency gap between sub-block1 and sub-block2 to derive MPR/A-MPR requirements for NR SL intra-band non-contiguous CA UE in ITS spectrum.

- A-MPR requirements

Based on the agreed MPR simulation assumptions Table in session 1, RAN4 can derive A-MPR requirements to comply with European regulation and US regulatory requirements.

On top of the MPR simulation assumption, RAN4 need to analyse the A-MPR requirements for both PC2 and PC3 SL intra-band non-contiguous CA UE, to comply European regulation or US FCC requirements which were specified in the additional spectrum emission mask in Table 6.5E.2.3.1-1 for ETSI and additional spurious emission requirements in Table 6.5E.3.4.2-1 and Table 6.5E.3.4.2-2  in TS38.101-1.

6.5E.2.3.1	Requirements for network signalling value "NS_33"

The additional spectrum mask in Table 6.5E.2.3.1-1 applies for NR V2X UE within 5 855 MHz to 5 950 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.1-1.

Table 6.5E.2.3.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

========== Unchanged sections are omitted ============

Table 6.5E.3.4.2-1: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5950 | -30 | 1 | 1 |
| Frequency range | 5815 |  | - | 5855 | -30 | 1 | 3 |
| NOTE 1: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy. |  |  |  |  |  |  |  |

When "NS_33" is configured from pre-configured radio parameters or the cell, and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any NR V2X UE emission shall fulfil either one of the two sets of conditions.

Table 6.5E.3.4.2-2: Requirements for spurious emissions to protect CEN DSRC for V2X UE

|  | Maximum Transmission Power (dBm EIRP1) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP1) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |
| NOTE 1: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. |  |  |

For the US regulation limits, RAN4 can wait to formally release the additional emission limits by FCC. After their final announcement, RAN4 can study and update the A-MPR requirements with NS_52 for both SL single carrier and SL intra-band CA to reflect the latest emission limits in the US.

Proposal 2: The above MPR simulation assumptions in Table in issue 2-2-2 1, frequency gap with 20MHz in Euro regions and additional spectrum emission mask in Table 6.5E.2.3.1-1 in TS38.101-1 and additional spurious emission requirements in Table 6.5E.3.4.2-1 and Table 6.5E.3.4.2-2 in TS38.101-1 will be considered to derive A-MPR requirements to comply European regulation for NR SL intra-band non-contiguous CA for both power class 3 and power class 2 in ITS spectrum.

Proposal 3: For US regulation requirements, RAN4 will study and specify the related A-MPR requirements with NS_52 for NR SL intra-band non-contiguous CA UE after final FCC announcement for the additional emission limits in US.

  - Other Tx RF requirements for NR SL intra-band non-contiguous CA UE

The following points are considered to define the Tx requirements for PC3/PC2 NR SL intra-band non-contiguous CA UE in ITS spectrum.

| Tx requirements for SL intra-band non-contiguous CA in ITS band | Comments for Requirements |
| --- | --- |
| 6.2E.1A MOP | Define SL intra-band non-contiguous CA MOP as total transmitted power (Per UE) for both PC3/PC2 SL CA UE |
| 6.2E.2A MPR | Need to MPR simulation campaign based on the agreed MPR simulation Table and frequency gap in section 2.1 |
| 6.2E.3A A-MPR | Need to A-MPR simulation campaign for Europe based on the agreed MPR simulation Table and worst frequency gap conditions (up to 20MHz). FFS in the US until the FCC finally release the additional emission limits. |
| 6.2E.4A Configured Tx power | Follow the configured Tx power of NR intra-band non-contiguous CA UE. It can be referred to define the configured Tx power for SL intra-band non-contiguous CA UE. |
| 6.3E.1A minimum output power | The min power -30dBm /10MHz, -30dBm/20MHz per sub-block will be applied |
| 6.3E.2A Transmit OFF power | The transmit off power (-50dBm/measurement BW) of NR SL in single carrier will be applied to each SL sub-block |
| 6.3E.3A Transmit On/OFF time mask | The transit on/off time mask of NR SL for single carrier will be applied to each SL sub-block. |
| 6.3E.4A Power control | Absolute power tolerance of NR SL for single carrier will be applied to each SL sub-block. |
| 6.4E.1A Frequency error | ±0.1 PPM observed over a period of 1 ms will be applied for the primary sub-block for all SL synchronous reference sources. |
| 6.4E.2A Transmit modulation quality | The EVM of NR SL in single carrier will be applied to each SL sub-block. The carrier leakage requirement is defined for each component carrier. Only one uplink carrier is activated in a time, the applicable LO leakage (clause 6.4.2.2) in NR UE will be applied per the activated carrier. |
| 6.5E.1A Occupied bandwidth | The occupied bandwidth requirement is met when the ratio of the transmitted power in all sub-blocks of the uplink CA configuration to the total integrated power of the transmitted spectrum is greater than 99%. |
| 6.5E.2A.2 out-of-band emission: General SEM | Composite spectrum emission mask applies to frequencies up to ΔfOOB starting from the edges of the sub-blocks. - Wider frequency gap: The general NR spectrum emission mask specified in clause 6.5.2.2.1 shall be applied to each SL sub-block. - If some frequency sub-block spectrum emission masks overlap, then a spectrum emission mask allowing higher power spectral density applies for that frequency. - If for some frequency a sub-block spectrum emission mask overlaps with the sub-block bandwidth of another sub-block, then the emission mask does not apply for that frequency. |
| 6.5E.2A.3 out-of-band emission: A-SEM | Need to consider the A-SEM in Europe since the A-SEM in Table 6.5E.2.3.1-1 is applied for the 10MHz CBW in Europe. For the US, RAN4 can reuse NS_52, the details can be further discussed with the final A-SEM or A-SE requirements from FCC. |
| 6.5E.2A.4 ACLR | The general NR CA ACLR requirements in subclause 6.5A.2.4.1.2 shall be applied to each SL sub-block. - In case the gap bandwidth Wgap between 2 uplink CCs is smaller than maximum of the 2 uplink channel bandwidths then no CA ACLR requirement is set for the gap. |
| 6.5E.3A.1 General SE | For intra-band non-contiguous SL CA, the spurious emissions are measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE. Composite spurious emission is applied to frequencies up to ΔfOOB starting from the edges of the sub-blocks. - Wider frequency gap: The general NR spurious emission specified in clause 6.5.3.1 shall be applied to each SL sub-block. - If for some frequency a sub-block sub-block spurious emission requirement overlaps with the general spectrum emission mask or the sub-block bandwidth of another sub-block, then it does not apply. |
| 6.5E.3A.2 UE coexistence | The protection operating band lists for n47 transmission was defined in Table 6.5.3.2-1 which will be reconsidered the UE coexistence requirements for SL CA. |
| 6.5E.3A.3 A-SE | The A-SE in Table 6.5E.3.4.2-1 and CEN DSRC protection requirements in Table 6.5E.3.4.2-2 will be reconsidered for NR CA UE in Europe. |

  - Rx RF requirements for NR SL intra-band non-contiguous CA UE

The following points are considered to define the Rx requirements for PC3/PC2 NR SL intra-band non-contiguous CA UE in ITS spectrum.

| Rx requirements for SL intra-band non-contiguous CA in ITS band | Comments for Requirements |
| --- | --- |
| 7.2E.2A REFSENS | For intra-band non-contiguous CA UE in n47, the reference sensitivity requirement specified in Table 7.3E.2-1 shall apply for each sub-block with all carriers active. The requirement is applied for each sub-block reception when 2 sub-block transmissions are activated at the same time. |
| 7.4E.1A Maximum input level | For intra-band non-contiguous CA, the maximum input level requirements (Section 7.4E.1) will be applied to each sub-block. The throughput (>= 95% T-put) of each DL CC shall meet or exceed the minimum requirements for the specified reference measurement channel in A.7.3 and A.7.4. |
| 7.5E.1A ACS | For intra-band non-contiguous CA, the UE shall fulfil the minimum requirement specified in Table 7.5E.1-1 to Table 7.5E.1-3 per sub-block where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 while all DL carriers are active. |
| 7.6E.2A In-band blocking | For intra-band non-contiguous CA, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with the test parameters defined in clause 7.6E.2.1 per sub-block while all downlink carriers are active. |
| 7.6E.3A Out-of-band blocking | For intra-band non-contiguous CA, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.6E.3.1 per sub-block while all downlink carriers are active. |
| 7.7E.1A Spurious response | For intra-band non-contiguous CA, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.7E.1 per sub-block while all downlink carriers are active. |
| 7.8E.1A Wideband intermodulation | For intra-band non-contiguous CA, the UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with test parameters defined in clause 7.8E.2.1 per sub-block while all downlink carriers are active. |

Proposal 4: The above UE Tx/Rx requirements for NR SL intra-band non-contiguous CA shall be considered to specify the UE RF requirements in TS38.101-1.

- Conclusions

In this contribution, we propose the detailed A-MPR simulation assumptions including frequency gap between sub-block 1 and sub-block 2. Also we share the detailed intra-band non-contiguous SL CA UE RF requirements in section 2. Based on our analysis for the intra-band non-contiguous SL CA UE in Rel-19, we proposed as follows:

Observation 1: The maximum frequency gap between each sub-block is about 50MHz in South Korea. The frequency gap in other countries is less than 50MHz.

Proposal 1: On top of above MPR simulation assumptions in Table in issue 2-2-2, RAN4 should consider the frequency gap between sub-block1 and sub-block2 to derive MPR/A-MPR requirements for NR SL intra-band non-contiguous CA UE in ITS spectrum.

Proposal 2: The above MPR simulation assumptions in Table in issue 2-2-2 1, frequency gap with 20MHz in Euro regions and additional spectrum emission mask in Table 6.5E.2.3.1-1 in TS38.101-1 and additional spurious emission requirements in Table 6.5E.3.4.2-1 and Table 6.5E.3.4.2-2 in TS38.101-1 will be considered to derive A-MPR requirements to comply European regulation for NR SL intra-band non-contiguous CA for both power class 3 and power class 2 in ITS spectrum.

Proposal 3: For US regulation requirements, RAN4 will study and specify the related A-MPR requirements with NS_52 for NR SL intra-band non-contiguous CA UE after final FCC announcement for the additional emission limits in US.

Proposal 4: The above UE Tx/Rx requirements for NR SL intra-band non-contiguous CA shall be considered to specify the UE RF requirements in TS38.101-1.

Reference

[1] R4-2406611, “WF on NR_SL_ intraB_CA_ITS,” OPPO, Meta Ireland

[2] RP-240840, “New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band,” Apple

[3] TS38.101-1

[4] TR38.786

[5] FCC-20-164A1, “First Report and Order, Further Notice of Proposed Rulemaking, and Order of Proposed Modification.,” FCC, Nov. 2020

[6] No. 21-1130, Intelligent Transportation Soc, et al v. FCC, et al, US Court of Appeals

[7] 5GAA White paper, “Development band configuration for C-V2X at 5.9GHz in Europe”

[8] The Republic of Korea Picks C-V2X as its Technology of Choice - 5GAA