3GPP TSG-RAN WG4 Meeting #111	R4-2408050

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	Testability aspects of LP-WUR

Agenda item:	10.14.2.4

Document for:	Discussion

Introduction

Regarding the testability aspects of LP-WUR, following agreements were made in RAN4#110bis [1].

Issue 2-1-4: Performance metric for Rx requirements Way forward: - Agreements have been reached in issue 2-2-1. Issue 2-2-1: Performance metric for REFSENS Agreement: - Use X% missed detection rate as the starting point for performance metric for LP-WUS RF requirements - FFS on X values - FFS on whether to have false alarm rate Issue 2-6-1: Test metric for LP-WUR receiver Way forward: Test metric should be aligned with performance metric for Rx requirements

Discussion

Specification vs testing

As discussed in the last RAN4#110bis meeting, the test metric used for LP-WUR should be aligned with performance metric for receiver requirements. However, there are two separate questions we need to answer,

- How to derive the specifications for the LP-WUR?

- How to test the LP-WUR performance?

The specifications give the vendors a guideline on how to build the LP-WUR and answering the first question helps manufactures. The second question is related to the conformance tests. The motivation to separate the discussion in two parts is because the LP-WUR is going to be a receiver which has no direct external interface to the test equipment (TE) as it can only talk with TE via the main radio (MR) unless there is a test mode to enable LP-WUR to have direct external interface to the TE. Thus, it is not possible to only test the performance of just the LP-WUR. From outside world, we can only test the combined performance of both LP-WUR and MR. Only the manufacturer will have insight to the standalone performance of the LP-WUR.

Further, if there is no sleeping (DRX) cycle configured for the MR during the test, then MR can also listen to the LP-WUS and respond on behalf of LP-WUR, thus passing the tests. In this way, in a test mode we can never be sure if we are testing the LP-WUR performance or not.

Specifications are used by vendors as design guidelines.

Conformance testing is used to make sure the final device is performing well enough in the field.

It is not possible to test just the performance of the LP-WUR without the MR unless there is a test mode to enable LP-WUR to have direct external interface to the TE.

If a no sleeping (DRX) cycle is configured, then MR can just stay awake and pass the tests cases meant for testing LP-WUR performance.

For deriving the requirements such as guard RBs for ACS, ASCS etc., link level simulations (LLS) use has been agreed. For LLS 1% missed detection rate can be used.

Link level simulations with ≤ 1% missed detection rate will be used for deriving the LP-WUR requirements.

Miss detection rate

To achieve a ≤1% miss detection rate (MDR) of LP-WUS in a statistically confident way is going to take long time. For conformance testing, a higher MDR can be used to speed up the conformance tests. The total time taken to have a statistically significant value depends on multiple factors such as test case execution time, TE setup and response time, UE feedback time, repetition cycle, wait times between repetition, etc. As the motivation to have a higher MDR (≥1%) is to save on conformance test time, it is best for RAN5 to decide the detection rate that can be achieved in a reasonable time while being statistically significant too.

Only motivation to have a higher (≥1%) MDR is to save conformance test time.

Lot of factors impacting the time taken for test case execution are under the scope of RAN5 work.

RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

Feedback from UE

In normal operating mode, it is possible to get feedback from only the MR as LP-WUR can only receive. That can be received in both RRC_IDLE or RRC_CONNECTED mode. However, in RRC_CONNECTED mode, lot more data can be collected in shorter time frame. Further, as RAN5 test are usually performed under RRC_CONNECTED mode, we think feedback regarding LP_WUR performance should be gathered in the RRC_CONNECTED mode.

As the LP_WUR is receive only, the MR can be used for uplink transferring of data required for evaluating the LP-WUR performance.

Feedback from the UE regarding LP-WUR performance should be gathered in the RRC_CONNECTED mode.

One important parameter that must come from the LP-WUR is false alarm rate (FAR), otherwise the LP-WUS detection threshold can be set to extremely low level to pass the test on MDR while causing a lot of false-alarm, which means reducing power saving as MR is waken up unnecessarily. By monitoring FAR along with MDR, it is possible to have a high degree of confidence in the performance of LP-WUR. This is especially important when a higher MDR is used.

False alarm rate is required to have a higher confidence on the calculated miss detection rate.

False alarm rate should be reported by the LP_WUR.

Test procedure

The test procedure must make sure that it is testing only the LP-WUR performance and not the combined performance of MR and LP-WUR. This might require using high SINR for the MR whenever there is a communication between TE and MR. However, designing detailed test cases comes under RAN5 responsibility. In RAN4 we can discuss if a LS to RAN5 is required or a note in RAN4 specification is sufficient to convey the intent.

RAN5 defines the detailed test procedure for the conformance tests.

A higher SNR for the main radio signals can be used, so that we just test the performance of the LP-WUR.

Discuss if a LS to RAN5 or a note in RAN4 specification is sufficient.

Conclusion

In this paper we have presented our initial views on the testability aspects of LP_WUR. The following Observations and Proposals were made:

Observation 1: Specifications are used by vendors as design guidelines.

Observation 2: Conformance testing is used to make sure the final device is performing well enough in the field.

Observation 3: It is not possible to test just the performance of the LP-WUR without the MR unless there is a test mode to enable LP-WUR to have direct external interface to the TE.

Observation 4: If a no sleeping (DRX) cycle is configured, then MR can just stay awake and pass the tests cases meant for testing LP-WUR performance.

Proposal 1: Link level simulations with ≤ 1% missed detection rate will be used for deriving the LP-WUR requirements.

Observation 5: Only motivation to have a higher (≥1%) MDR is to save conformance test time.

Observation 6: Lot of factors impacting the time taken for test case execution are under the scope of RAN5 work.

Proposal 2: RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

Observation 7: As the LP_WUR is receive only, the MR can be used for uplink transferring of data required for evaluating the LP-WUR performance.

Proposal 3: Feedback from the UE regarding LP-WUR performance should be gathered in the RRC_CONNECTED mode.

Observation 8: False alarm rate is required to have a higher confidence on the calculated miss detection rate.

Proposal 4: False alarm rate should be reported by the LP_WUR.

Proposal 5: RAN5 defines the detailed test procedure for the conformance tests.

Proposal 6: A higher SNR for the main radio signals can be used, so that we just test the performance of the LP-WUR.

Proposal 7: Discuss if a LS to RAN5 or a note in RAN4 specification is sufficient.

- References

- R4-2406619, WF on NR LP-WUS UE requirements, Vivo, RAN4#110bis, Changsha, China, 15 – 19 April 2024.