3GPP TSG-RAN WG4 Meeting # 111 	R4-2408088

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	vivo

Title: 	Discussion on the co-existence assumption for 14800 to 15350 MHz

Agenda Item:	10.3.4.1

Document for:	Approval

- Introduction

In the last meeting, the parameter for 14800 to 15350 MHz was discussed but there is no agreement was achieved, and related discussion point are recorded in [1]. Since this is a new frequency range and RAN4 never perform any co-existence evaluation before, the co-existence simulation is necessary to derive related requirement, e.g., ACLR/ACS. In this contribution, we provide our views on the simulation assumptions.

- Discussion

- Co-existence simulation scenario

The following cases can be considered:

Table 5.1: Summary of initial simulation scenarios for 15GHz

| No. | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | NR, 200MHz | NR, 200MHz | DL to DL | 15 GHz | Indoor hotspot |
| 2 | NR, 200MHz | NR, 200MHz | DL to DL | 15 GHz | Urban macro |
| 3 | NR, 200MHz | NR, 200MHz | UL to UL | 15 GHz | Indoor hotspot |
| 4 | NR, 200MHz | NR, 200MHz | UL to UL | 15 GHz | Urban macro |

According to the results in TR38.803, it seems the interference level for dense urban is lower than Urban macro and indoor, to save efforts, this scenario is removed from the table.

- Network layout model

For Uma, the assumptions in TR38.803 can be the baseline. One change here is the ISD, normally this value depends on the cell size. For 10GHz, the ISD used in the simulation is 450m and for FR2, 200m is used, so for 15GHz, we think the 350m can be possible option.

Table 2: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 350m | 200m is used for FR2 and 450m is used for 10GHz |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 3: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) |  |

For indoor office, the assumption in TR 38.803 can be reused.

Table 4: Single operator layout for indoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | 50m x 120m, 12BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office |  |
| Shadowing correlation |  | NA |  |

Table 5.: Multi operators layout for indoor

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Coordinated operation (0% Grid Shift) |  |

- Propagation model

The models in TR 38.803 are used.

- Antenna and beamforming

For the BS, the antenna model in TR 38.921 is used.

For the UE, isotropic radiation pattern antenna and no beamforming are assumed

- Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink, the classic CLx-ile based formula still can be used. This method is based on the target SNR@BS = 15 dB, considering the BW and NF is reused from TR 38.803, the following parameter can also be reused:

-	CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz)

-	γ = 1

- Link level performance for 5G NR coexistence

The mapping between SINR and throughput in TR 38.803 can be used.

- Other simulation parameters

Table 5.2.8-1: Other simulation parameters

| Parameters | Indoor | Urban macro |
| --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz | 200MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | YES | YES |
| BS max TX power in dBm | 23dBm | 43dBm |
| UE max TX power in dBm | 26dBm | 26dBm |
| UE min TX power in dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 11 | 11 |
| UE Noise figure in dB | 11 | 11 |
| Handover margin | 3dB | 3dB |

For the maximum output power, we assume the UE in this frequency range can achieve similar maximum peak EIRP as FR2 UE which is about 26~28 dBm. Since no beamforming is assumed, 26 dBm is used.

Proposal: Considering the simulation assumption in this contribution as starting point for 14800 to 15350 MHz.

- Conclusion

In this contribution, we provide our views on the simulation assumption for co-existence.

Proposal: Considering the simulation assumption in this contribution as starting point for 14800 to 15350 MHz.

- References

- R4-2406615, WF on IMT parameters for other frequency ranges, CATT, RAN4#110bis