3GPP TSG-RAN WG4 Meeting # 111 	R4-2408093

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for AIoT BS

Agenda Item:	10.13.3.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on the AIoT BS RF requirement.

- Discussion

  - System parameters

- Operating band

This depends on operator request, same as AIoT device, maybe we can take band n8 as the example band.

- Channel bandwidth

For the BS, since the in-band operation is prioritized, the channel bandwidth can be same as NR.

- Minimum receiver bandwidth

This requirement is also mentioned in [1]. Traditionally, we assume the bandwidth is same for Tx and Rx, but the spectrum for backscattering signal may different from OFDM, e.g., single sideband or double sideband, so RAN4 also need to discuss whether this requirement is needed or not.

  - Tx requirements

- Transmit output power

In [2], the MOP (maximum output power) for reader is defined as below:

- Output power dynamic

In [3], we provide the initial link budget based analysis for AIoT. Due to the option 1-1 and option 1-2 are prioritized, the interference is mainly between indoor and outdoor, and only when the R2D uses the UL spectrum, the NR signal have significant interference on the R2D. RAN4 needs to discuss whether the power boosting is needed and what is the feasible value.

- Transmit ON/OFF power

These requirements are only applied to TDD operation BS, so it is not needed for AIoT BS which is working in FDD band.

- Transmit signal quality

The frequency error for Reader is defined in [2] as ±10 ppm which can be the starting point. The EVM also needs to be defined, but the details need more discussion.

- Unwanted emissions

For the SEM, the unwanted emission for RFID in [2] can be starting point.

The definition of occupied bandwidth can be same as NR and the ACLR depends on co-existence study.

The SE is also defined in [2]

  - Rx requirements

In [2], for the Reader, 50% success rate is used as metric for the Rx requirement. RAN4 should first discuss what metric will be used for Rx requirement definition. All the discussion seems to be inefficient before we get the conclusion on this issue.

Proposal 1: It is suggested that success rate is used as performance metric for Rx requirement definition for AIoT BS.

Based on the analysis above, we have following proposal:

Proposal 2: The following table can be the starting point for AIoT BS RF requirement discussion:

| RF Requirement for AIoT BS |  |  |  |
| --- | --- | --- | --- |
| System parameter | Operating band |  | Band n8 as example band in SI |
|  | Channel bandwidth | Transmission bandwidth configuration | Same as NR for in-band operation |
|  |  | Guard band | Same as NR for in-band operation |
|  | Minimum receiver bandwidth |  | RAN4 discuss whether this requirement is needed considering the spectrum of backscattering signal |
| Tx requirement | Transmit output power | Maximum output power | 33 dBm/200kHz for frequency <900MHz; 36 dBm/400kHz for frequency >900MHz; |
|  | Output power dynamic |  | RAN4 need to discuss whether the power boosting is necessary and what is the feasible boosting value. |
|  | Transmit signal quality | Frequency error | ±10 ppm |
|  |  | EVM | FFS, ASK/PSK are considered |
|  | Output RF spectrum emissions | Occupied bandwidth | Same as NR BS |
|  |  | SEM | SEM in [2] as starting point |
|  |  | ACLR | Depends on co-existence study |
|  |  | SE | SE in [2] as starting point |
| Rx requirement | RAN4 should first discuss and decide what metric will be used to represent the Rx performance, e.g., 10% BLER, 50% success rate, etc. |  |  |

- Conclusion

In this contribution, we provide our views on the RF requirement for AIoT BS.

Proposal 1: It is suggested that success rate is used as performance metric for Rx requirement definition for AIoT BS.

Proposal 2: The following table can be the starting point for AIoT BS RF requirement discussion:

| RF Requirement for AIoT BS |  |  |  |
| --- | --- | --- | --- |
| System parameter | Operating band |  | Band n8 as example band in SI |
|  | Channel bandwidth | Transmission bandwidth configuration | Same as NR for in-band operation |
|  |  | Guard band | Same as NR for in-band operation |
|  | Minimum receiver bandwidth |  | RAN4 discuss whether this requirement is needed considering the spectrum of backscattering signal |
| Tx requirement | Transmit output power | Maximum output power | 33 dBm/200kHz for frequency <900MHz; 36 dBm/400kHz for frequency >900MHz; |
|  | Output power dynamic |  | RAN4 need to discuss whether the power boosting is necessary and what is the feasible boosting value. |
|  | Transmit signal quality | Frequency error | ±10 ppm |
|  |  | EVM | FFS, ASK/PSK are considered |
|  | Output RF spectrum emissions | Occupied bandwidth | Same as NR BS |
|  |  | SEM | SEM in [2] as starting point |
|  |  | ACLR | Depends on co-existence study |
|  |  | SE | SE in [2] as starting point |
| Rx requirement | RAN4 should first discuss and decide what metric will be used to represent the Rx performance, e.g., 10% BLER, 90% success rate, etc. |  |  |

- References

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard

- ETSI EN 302 208 V3.4.1

- R4-2408092, Preliminary evaluation on AIoT coexistence, vivo, RAN4#111