3GPP TSG-RAN WG4 Meeting #111		R4-2408110

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	 10.14.2.3

Source:	 vivo

Title:	Views on LP-WUS IBB, OBB, intermodulation, spurious emissions

Document for:	 Approval

- Introduction

This contribution shares further views on LP-WUS other UE RF requirements.

- LP-WUS UE RF

  - General for IBB/OBB and intermodulation requirements

It was agreed that IBB/OBB and intermodulation requirements should be defined for LP-WUR, current requirement in TS 38.101-1 can be a starting point.

Table 7.6.2-1 in TS 38.101-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB |  | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 3 | 5 |  |  |
| FIoffset, case 1 | MHz | 4.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 7.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

However, given the performance metric has been changed from 5% throughout loss to X% MDR, and the REFSENS value will be redefined, thus whether the same DL power level can be reused or not should further analyzed.

Besides, given the LP-WUS is a dedicated signal bandwidth within a large NR channel, the IBB/OBB/ intermodulation performance is highly related to LP-WUS location within the DL channel, e.g., 5MHz LP-WUS at center of 100MHz channel then many current requirements do not apply, which means the traditional definition can not be adopted directly.

Overall, we think RAN4 should re-define the IBB/OBB/intermodulation requirement (and test cases) for LP-WUS. Simulation evaluation can be performed if needed.

Proposal 1: The IBB/OBB/intermodulation requirements for LP-WUS is related to the allocated position of LP-WUS signal within NR channel, the definition and test cases should be modified.

Proposal 2: The DL power level for IBB/OBB/intermodulation requirements should also be re-evaluated for LP-WUS at different bandwidth and different location.

For the above requirements, RAN4 should assume that the guard RBs for ACS and ASCS should always be reserved.

Proposal 3: The Guard RBs for ACS/ASCS should be configured for the above requirements.

  - Spurious emissions

For LP-WUS, the general receiver spurious emission requirements defined in TS 38.101-1 should apply.

Table 7.9-1 in TS 38.101-1: General receiver spurious emission requirements

| Frequency range | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30 MHz f < 1 GHz | 100 kHz | -57 dBm |  |
| 1 GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 2 |
| 12.75 GHz – 26 GHz | 1 MHz | -47 dBm | 3 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH as defined in Annex C.3.1. NOTE 2: Applies for Band that the upper frequency edge of the DL Band more than 2.69 GHz. NOTE 3: Applies for Band that the upper frequency edge of the DL Band more than 5.2 GHz. |  |  |  |

Proposal 4: LP-WUR can reuse the general spurious emissions requirements directly.

- Conclusion

In this contribution, we share our views on other LP-WUR RF requirements:

Proposal 1: The IBB/OBB/intermodulation requirements for LP-WUS is related to the allocated position of LP-WUS signal within NR channel, the definition and test cases should be modified.

Proposal 2: The DL power level for IBB/OBB/intermodulation requirements should also be re-evaluated for LP-WUS at different bandwidth and different location.

Proposal 3: The Guard RBs for ACS/ASCS should be configured for the above requirements.

Proposal 4: LP-WUR can reuse the general spurious emissions requirements directly.

# References

- R4-2406619, WF on NR LP-WUS UE requirements, vivo, RAN4#110bis