3GPP TSG-WG4 Meeting #111	R4-2408116

# Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2776 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.101-1 for cleanup of Delta_powerclass related requirements for HPUE(Alt1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There were duplicate definitions for duty cycle and the ΔPPowerClass definition in single carrier case, while for CA case there were no duplication. The situtation is as following: Single carrier case: Defined in both MOP section, and also in configured transmitted power section. Intra-band contiguous CA / Intra-band non-contigous CA / inter-band CA Only define in MOP section, using reference in configured transmitted power sections; It should be noted that the wording of duty cycle related ΔPPowerClass calculation in the Pcmax section is squeezed in a few paragraphs, and not as clear as in the MOP part. Keeping the Pcmax part definition is not applicatable to CA cases. - The requirements defined in MOP which is related to ΔPPowerClass in configured transmitted power, would potentially impact other Tx requirements such as MPR/A-MPR, make the requirements not clear, and make the power class concept ambiguous. - Currently, P-max is included in the ΔPPowerClass concept in that it was included in the SAR scheme description. However, since it is already included in Pcmax equations seperately, remove it from definition and would bring more conciseness and not impact the Pcmax calculation. - If ΔPPowerClass is defined as a power backoff parameter only impact MOP, this would be similar to P-MPR, and may need not to be counted when sufficient MPR/A-MPR or P-MPR is available for lower bound of NR configurated transmitted power. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the duplicate part in ΔPPowerClass calculation in the Pcmax part for single carrier case, and make it aligned with CA cases. Note: In Rel-18, Inter-band CA+MIMO already aligned with other CA part while no more duplication, only minor correction needed. - In MOP requirements for single carreir, make clear only MOP is impacted by the dutycycle related behavior. i.e. “all requirements for” “maximum output power of”, “requirements” “maximum output power” in different parts. - Remove P-max related concept from the duty cycle based SAR scheme description. - Change the position of NR ΔPPowerClass to similar position of P-MPR for lower bound of NR configurated transmitted power to aviod duplicate count with MPR/A-MPR, if this concept is just for power backoff. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The duplication remains for dutycycle related ΔPPowerClass definitions in the Pcmax section, which is not only redundant and not aligned with others, but also adding the possibility for more errors. The impacted scope of SAR descriptions in MOP part is not clear. Some redundancy for P-Max, and some duplication may occur for power back off of lower bound. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.4, 6.2A.1, 6.2A.4, 6.2B.4.1, 6.2C.1, 6.2H.3.1, 6.2H.3.4, 6.2L.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

<< Unchanged parts omitted >>

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	shall apply maximum output power of the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	shall apply maximum output power of power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply maximum output power of the supported power class and set the configured transmitted power as specified in clause 6.2.4.

<< Unchanged parts omitted >>

6.2.4	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass –+ ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc, ΔPPowerClass) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation, without taking into account the tolerance specified in the Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation;

When  the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less symbols in certain evaluation period are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass =

-	3 dB for a power class 2 UE when the maximum output power of default power class is applied as specified in sub-clause 6.2.1;

-	3 dB for a power class 1.5 UE when the maximum output power of power class 2 is applied as specified in sub-clause 6.2.1;

-	6 dB for a power class 1.5 UE when the maximum output power of default power class is applied as specified in sub-clause 6.2.1;

-	3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1 on the band where UE indicates power class 2;

-	3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 or txDiversity2Tx-r18 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r8’;

-	0 dB otherwise;

<< Unchanged parts omitted >>

6.2A	Transmitter power for CA

6.2A.1	UE maximum output power for CA

6.2A.1.1	UE maximum output power for Intra-band contiguous CA

For uplink intra-band contiguous carrier aggregation, the maximum output power is specified in Table 6.2A.1.1-1. For downlink intra-band contiguous carrier aggregation with a single uplink component carrier configured in the NR band, the maximum output power is specified in Table 6.2.1-1 for power class 3 and other power classes if indicated in clause 5.5A.1.

<< Unchanged parts omitted >>

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-1 and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	shall apply maximum output power of the default power class and set the configured transmitted power as specified in clause 6.2A.4.1.1;

-	else shall applymaximum output power of the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1.

6.2A.1.2	UE maximum output power for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements in clause 6.2.1 apply for power class 3 and other power classes if indicated in clause 5.5A.2. For intra-band non-contiguous carrier aggregation with two uplink carriers the maximum output power is specified in Table 6.2A.1.2-1.

<< Unchanged parts omitted >>

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-2 and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	shall apply maximum output power of the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.2;

-	else shall apply maximum output power of the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.2.

6.2A.1.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in Table 6.2.1-1 apply for power class 3 and other power classes if indicated in clause 5.5A.3.

<< Unchanged parts omitted >>

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.3-1 and the supported power class enables the higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	shall apply maximum output power of the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3;

–	else;

–	shall apply maximum output power of the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 50%  ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

<< Unchanged parts omitted >>

6.2A.4	Configured output power for CA

6.2A.4.1	Configured transmitted power level

6.2A.4.1.1	Configured transmitted power for Intra-band contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in clause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,PPowerClass,CA – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc , ΔPPowerClass,CA) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA }

<< Unchanged parts omitted >>

6.2A.4.1.2	Configured transmitted power for Intra-band non-contiguous CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.4, but with MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR as determined by subclause 6.2A.2 and 6.2A.3, respectively. For PH reporting the following exception applies: if the UE is configured with multiple uplink serving cells, the power PCMAX,c  used for the purpose of PH reporting on first serving cell c = c1 does not consider for computation of the PH report transmissions on a second serving cell c2 as exempted  in subclause 7.7.1 in [8]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink intra-band non-contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA,PPowerClass,CA) – MAX(MAX(MPRc, A-MPRc) + ΔTIB,c + TC + DTRxSRS, P-MPR, ΔPPowerClass,CA ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA)}

<< Unchanged parts omitted >>

6.2A.4.1.3	Configured transmitted power for Inter-band CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in clause 6.2.4, except that the UE power class for serving cell c on the specific operating band shall be determined by the ue-PowerClassPerBandPerBC-r17 IE [7] as indicated for the band combination if signalled.

For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in clause 6.2.2 and clause 6.2.3, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink inter-band carrier aggregation with one serving cell c per operating band when same slot symbol pattern is used in all aggregated serving cells,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (tC,c),  pPowerClass.c/(MAX(mprc·∆mprc, a-mprc)·tC,c ·tIB,c·tRxSRS,c) , pPowerClass,c/pmprc], PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,CA-ΔPPowerClass, CA}

<< Unchanged parts omitted >>

<< Unchanged parts omitted >>

6.2B.4.1	Configured transmitted power level for NR-DC

The UE is allowed to set its configured maximum output power PCMAX,f,c,MCG and PCMAX,f,c,SCG for the respective MCG and SCG and its total configured maximum output power for NR-DC operation  PTotalNR-DC=10log10(PTotalNR-DC) with PTotalNR-DC as specified in clause 7.6.2 of [8]. The UE is configured with an inter-CG power sharing mode by NR-DC-PC-mode.The requirements apply for one uplink serving cell configured per CG and for asynchronous and synchronous NR-DC if not otherwise stated.

Unless otherwise stated, the configured maximum output power PCMAX,f,c,MCG (q) in physical-channel q for carrier f of serving cell c shall be set within the bounds if contained in the MCG,

PCMAX_L,f,c,MCG (q) ≤  PCMAX,f,c,MCG (q) ≤  PCMAX_H,f,c,MCG (q)

and the corresponding PCMAX_L,f,c,SCG (q) for a serving cell contained in the SCG,

PCMAX_L,f,c,SCG (q) ≤  PCMAX,f,c,SCG (q) ≤  PCMAX_H,f,c,SCG (q)

where PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,SCG and PCMAX_H,f,c,SCG are the limits for a serving cell c as specified in clause 6.2.4 modified as follows:

PCMAX_L,f,c,MCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, PPowerClass,NR-DC – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc, ΔPPowerClass,NR-DC)}

PCMAX_H,f,c,MCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass, NR-DC – ΔPPowerClass,NR-DC}

for the MCG and

PCMAX_L,f,c,SCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, PPowerClass,NR-DC  – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc, ΔPPowerClass,NR-DC)}

PCMAX_H,f,c,SCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass,NR-DC – ΔPPowerClass,NR-DC}

for the SCG, where

-	PEMAX,NR-DC is the value given by the field p-UE-FR1 of the PhysicalCellGroupConfig IE for the MCG as defined in [7];

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in [7];

-	PPowerClass,NR-DC is the maximum UE power specified in Table 6.2B.1.3-1 without taking into account the tolerance specified in the Table 6.2B.1.3-1;

-	∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2B.4.2 for NR-DC; ∆TIB,c = 0 dB otherwise;

-	∆TC,c = 1.5dB when NOTE 2 in Table 6.2B.1.3-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

-	∆MPRc for serving cell c is specified in clause 6.2.2.

-	ΔPPowerClass,NR-DC = 0 dB for a power class 3 UE.

When MSG or SCG are configured with intra-band contiguous carrier aggregation, then intra-band carrier aggregation PCMAX,CA,MCG (q) and/or PCMAX,CA,SCG (q) in physical-channel q shall be set within the bounds:

PCMAX_L,CA, MCG (q) ≤  PCMAX,CA,MCG (q) ≤  PCMAX_H,CA,MCG (q)

for MSG, and/or

PCMAX_L,CA,SCG (q) ≤  PCMAX,CA,SCG (q) ≤  PCMAX_H,CA,SCG (q)

for SCG, where PCMAX_L,CA,MCG, PCMAX_H,CA,MCG, PCMAX,CA,SCG and PCMAX_H,CA,SCG are the limits for a carrier aggregation uplink as specified in clause 6.2A.4.1.1 modified as follows:

PCMAX_L,CA,MCG = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA, PEMAX,NR-DC, PNR,MCG, PPowerClass,NR-DC  – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc, ΔPPowerClass,NR-DC ) }

PCMAX_H,CA,MCG  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA , PEMAX,NR-DC, PNR,MCG, PPowerClass,NR-DC – ΔPPowerClass,NR-DC }

for the MCG, and

PCMAX_L,CA,SCG = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA, PEMAX,NR-DC, PNR,SCG, PPowerClass,NR-DC  – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC + TRxSRS, P-MPRc , ΔPPowerClass,NR-DC) }

PCMAX_H,CA,SCG  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA , PEMAX,NR-DC, PNR,SCG, PPowerClass,NR-DC – ΔPPowerClass,NR-DC}

for SCG.

<< Unchanged parts omitted >>

6.2C	Transmitter power for SUL

6.2C.1	Configured transmitted power for SUL

When a UE is configured with both NR UL and NR SUL carriers in a serving cell with active transmission either on the UL carrier(s) or SUL carrier, the configured transmit power requirements specified in clause 6.2.4 and 6.2A.4 are applicable for the UL carrier(s) and the SUL carrier, respectively.

If a UE supports a different power class than the default UE power class for NR UL band of SUL combination and the supported power class enables the higher maximum output power for SUL combination than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle- SULcombination-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than the maximum percentage of uplink symbols that the UE indicates by maxUplinkDutyCycle- SULcombination-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	shall apply maximum output power of the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

–	else;

–	shall apply maximum output power of the supported power class and set the configured transmitted power as specified in clause 6.2.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 50%  ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

<< Unchanged parts omitted >>

6.2H.3	Transmitter power for inter-band UL CA with UL MIMO

6.2H.3.1	UE maximum output power for inter-band UL CA with UL MIMO

For inter-band UL CA with UL MIMO in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2H.3.1-1. The period of measurement shall be at least one sub frame (1 ms). The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2 and 6.2D.1-3 for 2-layer configuration and ULFPTx configuration respectively for the component carrier configured with UL MIMO.

<< Unchanged parts omitted >>

If a UE supports power class 2 for the band combination listed in Table 6.2H.3.1-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	shall apply maximum output power of the default power class and set the configured transmitted power as specified in clause 6.2H.3.4;

–	else;

–	shall apply maximum output power of the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

If a UE supports power class 1.5 for the band combination listed in Table 6.2H.3.1-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and

–	if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or

–	if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 23dBm or lower;

–	shall apply maximum output power of the default power class and set the configured transmitted power as specified in clause 6.2H.3.4;

–	if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or

–	shall apply maximum output power of the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4;

–	else;

–	shall apply maximum output power of the power class 1.5 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability 0.5maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one band within the band combination is power class 1.5

–	if the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 are both absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

–	else if only one of the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported;

–	the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability;

–	else

–	the corresponding maxDutyNR,x or maxDutyNR,y is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 and 0.5*maxUplinkDutyCycle-PC2-FR1;

–	if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

<< Unchanged parts omitted >>

6.2H.3.4	Configured transmitted power for inter-band UL CA with UL MIMO

For inter-band UL CA with UL MIMO in one of the two frequency bands, the requirements in clause 6.2A.4.1.3 apply except that:

-	PPowerClass,CA is the maximum UE power specified in Table 6.2H.3.1-1 without taking into account the tolerance;

-	MPRc and A-MPRc are specified in clause 6.2D.2 and clause 6.2D.3 respectively for the component carrier configured with UL MIMO.

-	ΔPPowerClass,CA:

–	For a power class 2 capable UE, it is 3dB when the maximum output power of default power class are applied as specified in sub-clause 6.2.H.3.1, otherwise ΔPPowerClass, CA = 0 dB;

–	For a power class 1.5 capable UE, it is 6dB when the maximum output power of default power class are applied as specified in sub-clause 6.2.H.3.1; and it is 3dB when the maximum output power of power class 2 are applied as specified in sub-clause 6.2.H.3.1; otherwise ΔPPowerClass, CA = 0 dB;

<< Unchanged parts omitted >>

6.2L	Transmitter power for CA with Tx Diversity

6.2L.1	Void

6.2L.2	Void

6.2L.3	Transmitter power for inter-band UL CA with Tx Diversity

6.2L.3.1	UE maximum output power for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors and all UL CCs, as specified in Table 6.2L.3.1-1. The period of measurement shall be at least one sub frame (1 ms).

<< Unchanged parts omitted >>

If a UE supports power class 2 for the band combination listed in Table 6.2L.3.1-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	shall apply maximum output power of the default power class and set the configured transmitted power as specified in clause 6.2L.3.4;

–	else;

–	shall apply maximum output power of the power class 2 and set the configured transmitted power as specified in clause 6.2L.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

If a UE supports power class 1.5 for the band combination listed in Table 6.2L.3.1-1:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and

–	if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or

–	shall apply maximum output power of the default power class and set the configured transmitted power as specified in clause 6.2L.3.4;

–	if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or

–	shall apply maximum output power of the power class 2 and set the configured transmitted power as specified in clause 6.2L.3.4;

–	else;

–	shall apply maximum output power of the power class 1.5 and set the configured transmitted power as specified in clause 6.2L.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 0.5*(DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 or maxUplinkDutyCycle-PC1dot5-MPE-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one band within the band combination is power class 1.5

–	if the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 are both absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 25%;

–	else if only one of the corresponding UE capability 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC1dot5-MPE-FR1 is reported;

–	the corresponding maxDutyNR,x or maxDutyNR,y is according to the reported capability;

–	else

–	the corresponding maxDutyNR,x or maxDutyNR,y is the smaller of maxUplinkDutyCycle-PC1dot5-MPE-FR1 and 0.5*maxUplinkDutyCycle-PC2-FR1;

–	if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

<< Unchanged parts omitted >>

6.2L.3.4	Configured transmitted power for inter-band UL CA with Tx Diversity

For inter-band UL CA with Tx Diversity in one of the two frequency bands, the requirements in clause 6.2A.4.1.3 apply except that:

-	PPowerClass,CA is the maximum UE power specified in Table 6.2L.3.1-1 without taking into account the tolerance;

-	MPRc and A-MPRc are specified in clause 6.2G.2 and clause 6.2G.3 respectively for the component carrier configured with Tx Diversity.

-	ΔPPowerClass,CA:

–	For a power class 2 capable UE, it is 3dB when the maximum output power of default power class are applied as specified in sub-clause 6.2.J.3.1, otherwise ΔPPowerClass, CA = 0 dB;

–	For a power class 1.5 capable UE, it is 6dB when the maximum output power of default power class are applied as specified in sub-clause 6.2.J.3.1; and it is 3dB when the maximum output power of power class 2 are applied as specified in sub-clause 6.2.J.3.1; otherwise ΔPPowerClass, CA = 0 dB;