3GPP TSG-RAN WG4 Meeting #111	R4-2408297

# Fukuoka, JP, 20 May - 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on measurement delay test cases for SL positioning, Sets 10-3, 10-4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Reason for change#1: Measurement delay test cases for SL AoA and SL RTOA needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change#1: Define the measurement delay test cases for SL AoA and SL RTOA, including the test purpose and environment, and the test requirements, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no measurement delay test cases for SL AoA and SL RTOA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9A.1.1.3, A.9A.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of Change #1 ---

A.9A.1.1.3	SL AoA measurements

A.9A.1.1.3.1	SL AoA measurement reporting delay test case for single positioning frequency layer in FR1

A.9A.1.1.3.1.1	Test Purpose and Environment

The purpose of the test is to verify that the SL AoA measurement meets the requirements specified in Clause 12A.6 in an environment with AWGN propagation conditions in FR1 scenario when the downlink timing of the serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) on a non-V2X sidelink carrier is used as timing reference and single SL BWP is configured.

In the test, all the UEs are in coverage and synchronizaed to the same gNB.

This test is applicable for V2X sidelink communication capable UEs that support NR Uu and sidelink operation.

The supported test configurstions for FR1 NR cell are shown in Table A.9A.1.1.3.1.1-1. There is one active cell (PCell) in this test.

Table A.9A.1.1.3.1.1-1: Supported Test Configurations for FR1 NR cell as synchronization reference source

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to pass in one of the supported test configurations in FR1. |  |

The supported test configurations for sidelink positioning are specified in Table A.9A.1.1.3.1.1-2.

Table A.9A.1.1.3.1.1-2: Supported test configurations for FR1 sidelink

| Configuration | Description |
| --- | --- |
| 1 | NR SL: TDD, SL PRS and data SCS 15 kHz, BW 10 MHz |
| 2 | NR SL: TDD, SL PRS and data SCS 15 kHz, BW 20 MHz |
| 3 | NR SL: TDD, SL PRS and data SCS 30 kHz, BW 20 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous anchor UEs: UE 1, UE 2. Both UEs are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Both UEs transmit SL-PRS during T2.

The SL-AOA-ProvideAssistanceData and SL-AOA-RequestLocationInformation as defined in TS 38.355 [37], shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the SL-AOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first slot containing the SL PRS resources. In T2, UE 1 transmit SL PRS in the first slot n, and UE 2 transmit SL PRS in slot n+1 and slot n+100ms.

The general test parameters are listed in Table A.9A.1.1.3.1.1-3, and UE specific test parameters and cell specific test parameters are listed in Table A.9A.1.1.3.1.1-4 and Table A.9A.1.1.3.1.1-5, respectively.

Table A.9A.1.1.3.1.1-3: General test parameters for SL AoA measurement reporting delay

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| UE 1 |  | 1, 2, 3 | UE 1 |  |
| UE 2 |  | 1, 2, 3 | UE 2 |  |
| RF Channel Number |  | 1 | 1: UE 1 and UE 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 48 |  |
|  |  | 2 | 20: NRB,c = 96 |  |
|  |  | 3 | 20: NRB,c = 48 |  |
| SL configuration |  | 1 | SL.1 FR1 |  |
|  |  | 2 | SL.2 FR1 |  |
|  |  | 3 | SL.3 FR1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| SL-DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between UEs | s | 1, 2, 3 | 3 | Synchronous UEs |
| Expected SL RSTD | s | 1, 2, 3 | 3 |  |
| Expected SL RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.9A.1.1.3.1.1-4: UE specific test parameters

| Parameter |  | Unit | Test configuration | Value |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | UE 1 |  |  | UE 2 |  |  |
|  |  |  |  | T1 |  | T2 | T1 | T2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |  |  |  |  |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 1 | 10 (NRB,c = 48) |  |  |  |  |  |
|  |  |  | 2 | 20 (NRB,c = 96) |  |  |  |  |  |
|  |  |  | 3 | 20 (NRB,c = 48) |  |  |  |  |  |
| SCS |  | kHz | 1 | 15 |  |  |  |  |  |
|  |  |  | 2 | 15 |  |  |  |  |  |
|  |  |  | 3 | 30 |  |  |  |  |  |
| Active cell |  |  | 1, 2, 3 | Cell 1 |  |  |  |  |  |
| Active SyncRef UE |  |  | 1, 2, 3 | None |  |  |  |  |  |
| PSCCH Reference Measurement Channel |  |  | 1, 2, 3 | CC.1A HD |  |  |  |  | As specified in Table A.3.21.3-1 |
| PSSCH Reference Measurement Channel |  |  | 1, 2, 3 | CD.1A HD |  |  |  |  | As specified in Table A.3.21.3-2 |
| Resource Pool Type |  |  | 1, 2 | 1: Dedicated RP and 2: Shared RP |  |  |  |  |  |
| SL-PRS configuration |  |  | 1, 2, 3 | As specified in section A.3.X.2.1 |  |  |  |  | IE values unless specified otherwise in this test. |
| Note 2 |  | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  |  | 2 | -98 |  |  |  |  |  |
|  |  |  | 3 | -95 |  |  |  |  |  |
| Note 2 |  | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  |  | 2 |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |
| SL-PRS |  | dB | 1 | -Infinity | 3 |  | -Infinity | -6 |  |
|  |  |  | 2 |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |
| SL-PRS |  | dB | 1 | -Infinity | 6.99 |  | -Infinity | 1.78 |  |
|  |  |  | 2 |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |
| SL-PRP Note 3 |  | dBm/SCS kHz | 1 | -Infinity -Infinity -Infinity | -91 |  | -Infinity -Infinity -Infinity | -96 |  |
|  |  |  | 2 |  | -91 |  |  | -96 |  |
|  |  |  | 3 |  | -88 |  |  | -93 |  |
| Io |  | dBm/9.36 MHz | 1 | N/A | -62.26 |  | N/A | -65.92 |  |
|  |  | dBm/19.08 MHz | 2 |  | -59.17 |  |  | -62.83 |  |
|  |  | dBm/18.36 MHz | 3 |  | -59.34 |  |  | -63.01 |  |
| Propagation Condition |  |  | 1, 2, 3 | AWGN |  |  |  |  |  |
|  | Note 1: The resources for SL transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SL-PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.9A.1.1.3.1.1-5: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| RF Channel Number |  |  | 2 |
| Duplex Mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 40: NRB,c = 106 |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| DRX Cycle |  |  | N/A |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.2 |
| OCNG Patterns |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -98 |
| Note2 | Config 1,2 | dBm/SCS | -98 |
|  | Config 3 |  | -95 |
|  |  | dB | 3 |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -95 |
|  | Config 3 |  | -92 |
| Io Note 3 | Config 1,2 | dBm/9.36 MHz | -65.2 |
|  | Config 3 | dBm/38.1 MHz | -59.2 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

A.9A.1.1.3.1.2	Test Requirements

The UE shall perform and report the SL AoA measurements for UE 1 and UE 2, within the time limit specified in clause 12A.6, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time limit above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

--- End of Change #1 ---

--- Start of Change #2---

A.9A.1.1.4	SL RTOA measurements

A.9A.1.1.4.1	SL RTOA measurement reporting delay test case for single positioning frequency layer in FR1 SA

A.9A.1.1.4.1.1	Test Purpose and Environment

The purpose of the test is to verify that the SL RTOA measurement meets the requirements specified in Clause 12A.7 in an environment with AWGN propagation conditions in FR1 scenario when the downlink timing of the serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) on a non-V2X sidelink carrier is used as timing reference and single SL BWP is configured.

In the test, all the UEs are in coverage and synchronizaed to the same gNB.

This test is applicable for V2X sidelink communication capable UEs that support NR Uu and sidelink operation. There is one active cell (PCell) in this test.

The supported test configurstions for FR1 NR cell are shown in Table A.9A.1.1.4.1.1-1.

Table A.9A.1.1.4.1.1-1: Supported Test Configurations for FR1 NR cell as synchronization reference source

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to pass in one of the supported test configurations in FR1. |  |

The supported test configurations for sidelink positioning are specified in Table A.9A.1.1.4.1.1-2.

Table A.9A.1.1.4.1.1-2: Supported test configurations for FR1 sidelink

| Configuration | Description |
| --- | --- |
| 1 | NR SL: TDD, SL PRS and data SCS 15 kHz, BW 10 MHz |
| 2 | NR SL: TDD, SL PRS and data SCS 15 kHz, BW 20 MHz |
| 3 | NR SL: TDD, SL PRS and data SCS 30 kHz, BW 20 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous anchor UEs: UE 1, UE 2. Both UEs are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Both UEs transmit SL-PRS during T2.

The SL-TOA-ProvideAssistanceData and SL-TOA-RequestLocationInformation as defined in TS 38.355 [37], shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the SL-TOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first slot containing the SL PRS resources. In T2, UE 1 transmit SL PRS in the first slot n, and UE 2 transmit SL PRS in slot n+1 and slot n+100ms.

The general test parameters are listed in Table A.9A.1.1.4.1.1-3, and UE specific test parameters and cell specific test parameters are listed in Table A.9A.1.1.4.1.1-4 and Table A.9A.1.1.4.1.1-5, respectively.

Table A. 9A.1.1.4.1.1-3: General test parameters for SL RTOA measurement reporting delay

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| UE 1 |  | 1, 2, 3 | UE 1 |  |
| UE 2 |  | 1, 2, 3 | UE 2 |  |
| RF Channel Number |  | 1 | 1: UE 1 and UE 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 48 |  |
|  |  | 2 | 20: NRB,c = 96 |  |
|  |  | 3 | 20: NRB,c = 48 |  |
| Sl configuration |  | 1 | SL.1 FR1 |  |
|  |  | 2 | SL.2 FR1 |  |
|  |  | 3 | SL.3 FR1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| SL-DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between UEs | s | 1, 2, 3 | 3 | Synchronous UEs |
| Expected SL RSTD | s | 1, 2, 3 | 3 |  |
| Expected SL RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.9A.1.1.4.1.1-4: UE specific test parameters

| Parameter |  | Unit | Test configuration | Value |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | UE 1 |  |  | UE 2 |  |  |
|  |  |  |  | T1 |  | T2 | T1 | T2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |  |  |  |  |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 1 | 10 (NRB,c = 48) |  |  |  |  |  |
|  |  |  | 2 | 20 (NRB,c = 48) |  |  |  |  |  |
|  |  |  | 3 | 40 (NRB,c = 48) |  |  |  |  |  |
| SCS |  | kHz | 1 | 15 |  |  |  |  |  |
|  |  |  | 2 | 15 |  |  |  |  |  |
|  |  |  | 3 | 30 |  |  |  |  |  |
| Active cell |  |  | 1, 2, 3 | Cell 1 |  |  |  |  |  |
| Active SyncRef UE |  |  | 1, 2, 3 | None |  |  |  |  |  |
| PSCCH Reference Measurement Channel |  |  | 1, 2, 3 | CC.1A HD |  |  |  |  | As specified in Table A.3.21.3-1 |
| PSSCH Reference Measurement Channel |  |  | 1, 2, 3 | CD.1A HD |  |  |  |  | As specified in Table A.3.21.3-2 |
| Resource Pool Type |  |  | 1, 2 | 1: Dedicated RP and 2: Shared RP |  |  |  |  |  |
| SL-PRS configuration |  |  | 1, 2, 3 | As specified in section A.3.X.2.1 |  |  |  |  | IE values unless specified otherwise in this test. |
| Note 2 |  | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  |  | 2 | -98 |  |  |  |  |  |
|  |  |  | 3 | -95 |  |  |  |  |  |
| Note 2 |  | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  |  | 2 |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |
| SL-PRS |  | dB | 1 | -Infinity | 3 |  | -Infinity | -6 |  |
|  |  |  | 2 |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |
| SL-PRS |  | dB | 1 | -Infinity | 6.99 |  | -Infinity | 1.78 |  |
|  |  |  | 2 |  |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |  |
| SL-PRP Note 3 |  | dBm/SCS kHz | 1 | -Infinity -Infinity -Infinity | -91 |  | -Infinity -Infinity -Infinity | -96 |  |
|  |  |  | 2 |  | -91 |  |  | -96 |  |
|  |  |  | 3 |  | -88 |  |  | -93 |  |
| Io |  | dBm/9.36 MHz | 1 | N/A | -62.26 |  | N/A | -65.92 |  |
|  |  | dBm/19.08 MHz | 2 |  | -59.17 |  |  | -62.83 |  |
|  |  | dBm/18.36 MHz | 3 |  | -59.34 |  |  | -63.01 |  |
| Propagation Condition |  |  | 1, 2, 3 | AWGN |  |  |  |  |  |
|  | Note 1: The resources for SL transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SL-PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.9A.1.1.4.1.1-5: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| RF Channel Number |  |  | 2 |
| Duplex Mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 40: NRB,c = 106 |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| DRX Cycle |  |  | N/A |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.2 |
| OCNG Patterns |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -98 |
| Note2 | Config 1,2 | dBm/SCS | -98 |
|  | Config 3 |  | -95 |
|  |  | dB | 3 |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -95 |
|  | Config 3 |  | -92 |
| Io Note 3 | Config 1,2 | dBm/9.36 MHz | -65.2 |
|  | Config 3 | dBm/38.1 MHz | -59.2 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

A.9A.1.1.4.1.2	Test Requirements

The UE shall perform and report the SL RTOA measurements for UE 1 and UE 2, within the time limit specified in clause 12A.7, starting from the beginning of time interval T2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the time limit above because of TTI insertion uncertainty of the measurement report in DCCH.

The rate of correct events observed during repeated tests shall be at least 90%.

--- End of Change #2 ---