# 3GPP TSG-RAN WG4 Meeting # 111	R4-2408326

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	7.6.1

Source: 	Ericsson

Title:	Remaining issues on BWP operation without restriction

Document for:	Discussion

# Introduction

In this paper we have discussed the remaining issues on BWP operation without restriction requirements [1].

# Requirements for joint options

One of the remaining issues is the requirement for joint options as follow.

Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1 - FFS following options - Option 1: - LS RAN2 and RAN1 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1. - Option 2a: - RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1. - Option 2b: - Configuration of SSB resources outside active BWP is allowed within CSI-ResourceConfig settings for UEs supporting option B-1-1 - No need to send an LS to RAN1/RAN2. - Option 2c: - RAN4 should further clarify what is the ambiguity part of the other WG spec before sending an LS requesting for further details. - Option 3: - Update FG53-1 related clarification in L1 RSRP measurement requirements.

When UE supports option B-1-1, UE is configured to measure SSB within DL BWP where the BWP is indicated by higher layer parameter BWP-id. However, the configured BWP-id is not mandadated to be the active BWP index which means it’s already supported to configure a SSB outside active BWP for measurement. Therefore, configuration of SSB resources outside active BWP is allowed within CSI-ResourceConfig settings for UEs supporting option B-1-1.

Proposal 1: From RAN4’s understanding, configuration of SSB resources outside active BWP is allowed within CSI-ResourceConfig settings for UEs supporting option B-1-1. No need to send an LS to RAN1/RAN2.

# Summary

In this contribution, we have discussed the remaining issues on BWP operation without restriction requirements. Based on the discussions, the following are the proposals:

Proposal 1: From RAN4’s understanding, configuration of SSB resources outside active BWP is allowed within CSI-ResourceConfig settings for UEs supporting option B-1-1. No need to send an LS to RAN1/RAN2.

# References

- R4-2406364, WF on core maintenance and performance part for BWP without restriction, vivo