3GPP TSG-RAN4 Meeting #111	R4-2408403

Fukuoka, Japan, 20th – 24th May, 2024

Title:	Views on IMT parameters for 7125 - 8400 MHz

Source:	Qualcomm Incorporated

Agenda item:	10.3.3

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. In RAN4#110-bis, a WF was agreed in [2] where several agreements were captured for the 7125 – 8400 MHz and 14800 – 15350 MHz frequency range. In this contribution, we provide our views on the different RF parameters for BS and UE items for the 7125 – 8400 MHz frequency range.

# 2 	Discussion

## 2.1	System parameters

### 2.1.1	Duplexing

During RAN4#110-bis, extensive discussion was held on the incorporation of SBFD as a duplex method for the 4400-4800MHz frequency range and the following was captured in [2] regarding the 7125 – 8400 MHz frequency range:

Issue 2-1 Duplex Method - Option 1: Align with 4GHz outcome, i.e., TDD as baseline, check SBFD - Option 2: Follow n104

The existing n104 band is TDD and for 10GHz study in [3], it was agreed that even if FDD is not precluded, it is likely that TDD will be deployed. On the other hand, 3GPP has been studying since Rel-18 the feasibility of allowing the simultaneous existence of downlink and uplink at the gNB side within a conventional TDD band (i.e., SBFD). The outcome of the Rel-18 study has been captured in TR 38.858. Currently 3GPP is specifying in Rel-19 SBFD operation at the gNB side within a TDD carrier, gNB-to-gNB CLI handling schemes and UE-to-UE CLI handling schemes for SBFD operation, and RF requirements for SBFD operation at gNB [4]. It is thus important to document in this SI the ongoing progress within 3GPP on duplex evolution, specifically, SBFD.

Observation 1: 3GPP has been studying flexible duplexing (i.e., SBFD) at the gNB in Rel-18 and Rel-19 and it is desired to document such progress in this SI TR (i.e., TR 38.922).

Since the core requirements for SBFD-capable gNB is still under development within Rel-19 package [4], it is important to progress the sharing studies in WP5D via timely providing WP5D with the requested RF parameters. Accordingly, it is proposed that RAN4 adopts TDD duplexing as a baseline for the 7125 - 8400 MHz and capture in the TR text reflecting the ongoing work on SBFD within 3GPP.

Proposal 1: RAN4 to agree on TDD as the current duplexing candidate for 7125 – 8400 MHz frequency range and capture more text on SBFD in TR 38.922.

### 2.1.2	Channel bandwidth

In [2], it was agreed to have 100MHz as a baseline and further investigate higher CHBWs. In general, wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB width and availability of the spectrum. Since the 71425 – 8400 MHz is not in the FR1 frequency range, higher channel bandwidths compared to 100MHz should not be precluded. Such channel bandwidths can be achieved via higher SCS or intra-band CA employment. Thus, we propose to consider 200MHz while mentioning that other channel bandwidths are not precluded.

Proposal 2: RAN4 to reply in the LS to WP5D that 200MHz is a typical value and that other channel bandwidths are not precluded for the 7125 – 8400 MHz frequency range.

### 2.1.3	Signal bandwidth

Signal bandwidth depends on the channel bandwidth as well as on the sub-carrier spacing (SCS). The transmission bandwidth configuration defined in RAN4 core spec for BS and UE can be reused to derive the necessary signal bandwidth, where the signal bandwidth is calculated based on the NR spectrum utilization for each SCS as follows:

Signal bandwidth = NRB x SCS x 12

For the targeted bandwidth, 30 and 60 KHz might be possible candidates for this frequency range. Since it will be difficult to agree within RAN4 on a given SCS till RAN4#112 (i.e., RAN4 target deadline for 7GHz parameters), it is proposed to mention the dependency of signal bandwidth on the SCS and number of RBs. ,

Proposal 3: As signal bandwidth depends on CHBW and SCS, no need to specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs.

## 2.2	BS parameters

As several BS RF requirements were studied for 6.425-7.025 GHz and 7.025-7.125 GHz in [3], it is natural that some of those parameters will be reused for the 7125 - 8400 MHz frequency range. For the power dynamic range, spectral mask (i.e., OBUE), ACLR, ACS, spurious emission, noise figure, and blocking response, we propose to reuse the parameters specified in [3].

Proposal 4: RAN4 to reuse the existing BS RF parameters, captured in TR 38.921 for the power dynamic range, ACLR, ACS, spectral mask (i.e., OBUE), spurious emission, noise figure, and blocking response.

For the BS AAS parameters, the number of elements considered for the 4400 - 4800 MHz is based on TR 38.803, subclause 5.2.3.2.4, where the total number of antenna elements for macro suburban and macro urban is set to be 96 elements (4x8 subarrays with 3 elements per subarray). Assuming the same coverage metrics for the 7125 - 8400 MHz frequency range as FR1 deployments apply, it is anticipated that the number of antenna elements scale four times with doubling the frequency. In [3], the number of antenna elements considered for the same deployment scenarios is 128 elements (16x8 array).  Accordingly, RAN4 will need to further discuss how to scale the number of elements from 4 GHz

2.3	UE parameters

### 2.3.1	Maximum Output power

PC3 was assumed during Rel-17 work on 7GHz and 10GHz as captured in [3]. On the other hand, n104 considers both PC2 and PC3. Additionally, 3GPP has studied high power UEs during Rel-17 covering high-power UE (power class 1.5) operation in NR bands n77 and n78 (RP-202912), high power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 (RP-221774), and high-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands (RP-221795).

To capture the trend of higher UE power classes, envisaged for new applications and deployment use cases, we propose to communicate with WP5D PC2 (i.e., 26 dBm) as the UE maximum output power. In order to ensure that the ACLR/ACS derived in Rel-17 TR 38.921 are still applicable for higher UE output power (note that TR 38.921 assumed 23 dBm as UE output power), we plot in Figure 1 the SNR and SINR (co-channel and adjacent channel) for PC2 and PC3. It can be observed from the below figure that the system is dominated by the co-channel interference and thus, increasing the UE maximum output power from 23 dBm to 26 dBm will not impact the derived ACLR/ACS values from adjacent channel coexistence point of view.

Observation 2: From adjacent channel coexistence point of view, increasing UE maximum output power from 23 dBm (TR 38.921 assumptions) to 26 dBm (proposed for 74125-8400 MHz) does not impact derived ACLR/ACS values from TR 38.921.

Figure 1 SNR and SINR statistics for PC2 and PC3 for UMa @ 7125 MHz

Additionally, given that Rel-17 TR 38.921 studies for 6425-7125 MHz assumed 23 dBm, moving to the higher frequency band (7125 – 8400 MHz), the encountered propagation losses should be compensated with the higher UE maximum output power. Accordingly, it is proposed that RAN4 agrees to 26 dBm as UE maximum output power.

Proposal 6: RAN4 to agree on 26 dBm (i.e., PC3) as UE maximum output power.

### 2.3.2	UE power dynamic range

To derive the power dynamic range, RAN4 needs to agree on the UE minimum and maximum output power. We propose to reuse FR1 UE minimum output power of -33 dBm while 26 dBm as maximum output power. Accordingly, we propose 59 dBm for the UE power dynamic range.

Proposal 7: RAN4 to agree on 59 dBm as UE power dynamic range.

### 2.3.3	UE ACLR

In [2], the following was agreed:

- UE - Option 1: 26dB, 27dB (study) for PC3 - Option 2: 30dB (n104) for PC3, 31dB (n104) for PC2

As we are proposing PC2 and PC3 for this frequency range, we propose reusing existing n104 UE ACLR requirements given in TS 38.101-1.

Proposal 8: For UE ACLR, RAN4 to agree on 31 dB value for PC2.

### 2.3.4 	UE Remaining parameters

Similar to the BS discussion in Section 2.1, remaining UE parameters following n104 can be reused.

Proposal 9: RAN4 to reuse the existing UE RF parameters for n104 for the spectral mask (i.e., OBUE), ACS, spurious emission, noise figure, and blocking response for the 7125 - 8400 MHz frequency range.

# Conclusion

In this paper we have shared our views on the RF parameters for 7125 – 8400 Frequency range. Our observations and proposals can be summarized for that frequency range as follows:

Observation 1: 3GPP has been studying flexible duplexing (i.e., SBFD) at the gNB in Rel-18 and Rel-19 and it is desired to document such progress in this SI TR (i.e., TR 38.922).

Proposal 1: RAN4 to agree on TDD as the current duplexing candidate for 7125 – 8400 MHz frequency range and capture more text on SBFD in TR 38.922.

Proposal 2: RAN4 to reply in the LS to WP5D that 200MHz is a typical value and that other channel bandwidths are not precluded for the 7125 – 8400 MHz frequency range.

Proposal 3: As signal bandwidth depends on CHBW and SCS, no need to specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs.

Proposal 4: RAN4 to reuse the existing BS RF parameters, captured in TR 38.921 for the power dynamic range, ACLR, ACS, spectral mask (i.e., OBUE), spurious emission, noise figure, and blocking response.

Proposal 5: RAN4 to study the feasibility of employing larger number of antenna elements in the BS compared to Rel-17 parameters captured in TR 38.921.

Observation 2: From adjacent channel coexistence point of view, increasing UE maximum output power from 23 dBm (TR 38.921 assumptions) to 26 dBm (proposed for 74125-8400 MHz) does not impact derived ACLR/ACS values from TR 38.921.

Proposal 6: RAN4 to agree on 26 dBm (i.e., PC3) as UE maximum output power.

Proposal 7: RAN4 to agree on 59 dBm as UE power dynamic range.

Proposal 8: For UE ACLR, RAN4 to agree on 31 dB value for PC2.

Proposal 9: RAN4 to reuse the existing UE RF parameters for n104 for the spectral mask (i.e., OBUE), ACS, spurious emission, noise figure, and blocking response for the 7125 - 8400 MHz frequency range.

# Reference

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2] R4-2406615, “WF on IMT parameters for other frequency ranges”, CATT, RAN4#110-bis, Changsha, China

[3]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, 3GPP V17.1.0.

[4] RP-240789, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon,   3GPP TSG RAN#103.