3GPP TSG-RAN WG4 Meeting #111	R4-2408616

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	10.11.4

Source:	Intel Corporation

Title:	Views on AI/ML testability for CSI compression

Document for:	Discussion

# Introduction

In Rel-18 RAN WG4 conducted detailed studies on the testability and interoperability aspects of AI/ML enabled techniques for NR air interface as a part of FS_NR_AIML_Air SI [1] and the conclusions are summarized in TR 38.843 [2]. A new WI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting [3] with the following RAN4 objectives (the study stage objectives are marked in yellow):

### Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: - … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - … - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. ### Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

The work item stage discussions took place in RAN4 #110 and #110bis with agreements summarized in [4-5]. In this contribution we provide views and proposals on the AI/ML testability and interoperability aspects for two-sided AI/ML models.

# Testability and interoperability for two-sided AI/ML models

In previous meetings RAN4 discussed feasibility of testing 2-sided AI/ML models for CSI compression use case and several candidate options on how to test encoder/decoder implementation at the test equipment side were identified [2].

Following the above principles, the considered options of test decoder are listed below - Option 1: DUT provides the decoder - Option 2: Infra vendor provides the decoder - Option 3: Full decoder specification in standard - Option 4: TE vendor provides the decoder

During the RAN4 #110 meeting it was decided to narrow the testing options for two-sided AI/ML models with the focus on defining a common reference decoder for 2-sided AI/ML models [4].

Issue 4-2: Testing options for 2-sided model - RAN4 to further discuss only options 3 and 4

The WF [4] does not describe the eventual procedure for Options 3/4 feasibility analysis, and before proceeding with definition of detailed model we recommend to outline: 1) the steps required to conduct analysis of the feasibility, 2) principles to confirm feasibility, 3) principles to select the decoder for specification (if multiple options are identified).

In RAN4#110bis a discussion on the potential next steps took place and a high-level procedure was presented in [6]. In our view the respective procedure can be considered as a starting point.

Proposal #1:	Further discuss and agree on the procedure to conduct analysis of Option 3/4 and on the principles to select the decoder for specification.

## Inter-vendor training collaboration framework

The concurrent discussion with respect to inter-vendor training collaboration of AI/ML-based CSI compression took place in RAN1 #116 (Feb 2024) and the following agreements were reached:

Agreement To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc. Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

In RAN1 #116bis additional agreements on details of respective options were made. In addition, the following intermediate conclusions/observations were made:

Conclusion: - Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors). - It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4. Observation - Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed. - Option 1 and 2 may require high specification effort from RAN1 perspective. Conclusion - Deprioritize Option 2 for inter-vendor training collaboration. - Note: This deprioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

As acknowledged in RAN1 discussion the RAN1 Option 1 closely resembles RAN4 Options 3/4, with the primary distinction being the potential inclusion of a reference encoder in RAN1 approach. If RAN1 Option 1 is adopted, then the respective models can be reused for RAN4 testing. Meanwhile, RAN1 options 3-5 are reflecting a broader range of considerations and if any of the options are adopted, then additional RAN4 discussions may be required. RAN1 discussions on reference models have significant implications for RAN4 work on defining reference decoder options and decisions made by RAN1 will influence the direction of RAN4 standardization efforts. Given the interdependency of RAN1/4 discussions, it is recommended for RAN4 to continue analyzing both Options 3 and 4 concurrently and wait for RAN1 conclusions on how to facilitate inter-vendor collaboration before deriving final conclusions. The respective proposal is in-line with the common understanding in RAN4 #110bis [5] (“RAN4 continues to discuss Option 3 and 4 and can review/discuss RAN1 agreements on interoperability if there will be any”).

Proposal #2:	Further align the AI/ML test methodology with RAN1's conclusions on inter-vendor training collaboration for AI/ML-based CSI compression.

## Reference and Test encoder/decoder definitions

The definition of Reference and Test encoder/decoder was discussed in RAN4 #110bis with the following agreements reached in [5]:

Issue 4-2: Reference and test encoder/decoder Test decoder definition: Test decoder(for UE side test): the decoder to be used in RAN4 tests and implemented in TE. it will be captured in the specifications if necessary (for example, for Option 3 it would be explicitly captured in the specifications) test decoder definition covers both Option 3 and Option 4 Companies are invited to bring proposals to clarify the meaning/definition of the reference encoder/decoder used in the RAN4 discussions.

From RAN4 requirements perspective it is important to establish a common baseline for performance and complexity (for both 1-sided and 2-side models) based on reference AI/ML encoder/decoder models. Similar to legacy requirements reference receiver algorithms (e.g., LMMSE-IRC or R-ML) such models will be utilized during the performance requirements definition stage to achieve a certain minimum level of performance under an assumption of a specific complexity level. The reference models are not necessarily meant to be explicitly standardized in specifications, but rather should serve as a general guideline to define the minimum requirements. For the case of CSI compression use case the Reference decoder is expected to be identical to the Test decoder implemented by TE.

Proposal #3:	Adopt the following definition of Reference decoder/encoder

Reference decoder/encoder: The decoder/encoder model used to define the minimum performance requirements. The Reference decoder is identical to the Test decoder.

## Models complexity

Due to a diverse nature of AI/ML models implementations, there may be a significant variation in their complexity and performance, which is illustrated by Rel-18/19 RAN1 studies. The summary AI/ML models used for RAN1 evaluations for 2-sided AI/ML models for CSI compressions is provided in TR 38.843 [2] with the complexity in the range from 10M to 1100M FLOPs and with 1M to 17M parameters for the decoder (CSI reconstruction) part:

The complexity metric in terms of FLOPs and number of parameters of AI/ML models adopted in the evaluations of CSI compression are summarized in Figure 6.2.2.1-1, where the complexity for the CSI generation part and the complexity for the CSI reconstruction part are illustrated separately. - A majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M. - A majority of 21 sources adopt the CSI generation model subject to the number of parameters from 1M to 13M, and 22 sources adopt the CSI reconstruction model subject to the number of parameters from 1M to 17M. - Results refer to Table 1 of clause 7.3, R1-2310450. Figure 6.2.2.1-1: Complexity of AI/ML models from evaluation results in terms of FLOPs and number of parameters for CSI compression

In the figure below we illustrate the dependency of SGCS performance depending on the AI/ML model complexity based on results in [6]. It can be observed that AI/ML model complexity has a direct impact on the overall performance and that the complexity of the methods may substantially exceed the complexity of legacy PMI-based CSI feedback schemes.

Figure 1: SGCS and complexity for eType II PMI codebook and different transformer-based AI/ML models.

The RAN4 reference models for testing are expected to set the basis for both UE and NW implementations and serve as reference decoders for performance requirements definition. Therefore, RAN4 needs to further discuss on associated complexity for Options 3/4 considering RAN1 analysis on the performance as well as practical considerations on the algorithm implementation complexity including computational complexity and memory consumption. Additional RAN4 analysis on complexity should be performed to ensure that reference models represent practical implementations in terms of complexity and power consumption. The respective bounds on the maximum complexity shall be considered for both Option 3 and Option 4 methods.

Proposal #4	Further discuss and define the upper bound complexity including the number of computations and number of parameters for test/reference encoders/decoders.

## Option 3: Fully standardized reference/test decoder

In RAN4 #110 the initial set of parameters for the definition of Option 3 reference decoder were decided [4], while no agreements were made in RAN4 #110bis. Below we provide views on selected aspects of Option 3:

- Model architecture parameters:

  - Model type/depth, layers type/size: The full model architecture needs to be aligned/decided. Several model types can be considered (e.g., transformer, CNN, MLP). The upper bound model complexity (number of computations and number of parameters) shall be decided taking into account RAN1 performance evaluations and implementation aspects.

  - Encoder-decoder interface: Shall be a part of the agreed model. The number of bits of latent message can be based on RAN1 conclusions. For early analysis 60 and 100 bits can be considered as a starting point.

  - Fixed point representation: Shall be a part of the agreed model. The eventual model(s) shall give flexibility to support different representations including INT and FP.

  - Format of input to encoder/output of decoder: Needs to be aligned as well. SVD vectors of channel covariance matrix per subband. Separate inference per each layer can be assumed.

- Training datasets

  - For feasibility analysis synthetic channel models can be considered and aligned with agreed simulation assumptions.

The RAN4 work is aiming at confirming the feasibility of the test method. Same time if the RAN4 is to define a certain Test decoder model, further check with RAN1 may required to provide inputs on the associated performance to make sure that the selected model provides good enough performance.

## Option 4: Partially standardized reference/test decoder

The option 4 is defined as follows in [2]:

For option 4, the following aspects should be considered: - TE vendor should be able to develop the decoder based on the specifications - Test repeatability should be ensured (variation among TE vendor implementations should be bound) - Other vendors should also be able to develop such a decoder and which can deliver similar performance - Interoperability should be ensured based on the parameters that need to be specified - Parameters that need to be specified are FFS - Candidate parameters/conditions that may be considered for defining test decoder include - Training data set for TE decoder training - Model structure (Activation function is included in the model structure) - Performance parameters for the TE decoder (e.g. cosine similarity, loss function, etc) - Maximum FLOPs allowed for the test decoder - Maximum number/size of model parameters - Compression ratio of decoder (output size/input size) - Quantization level - Other parameters are not precluded and to be further discussed. - Note: Feasibility of definition of parameters needs further investigated.

The eventual goal of AI/ML test feasibility analysis is to make ensure test methodology provides a framework to provide repeatable results at the TE side. From this point of view, there may be no fundamental difference between Option 3 and Option 4 approaches in terms of feasibility as long as sufficient details of Option 4 model are provided such that TE vendors can come up with models providing aligned results. Basically, the discussion and analysis of Option 4 feasibility can be performed jointly with discussion on Option 3 method and follow the same principles. In particular, the definition of Option 4 framework will require definition of:

- Model architecture parameters:

  - The subset of parameters identified for Option 3 can be used as the basis for Option 4. In particular, the discussion on the AI/ML models for Options 3 and 4 can be performed jointly without the need for parallel tracks.

- Training data set for model training:

  - The respective dataset shall be clearly standardized and included in specifications so that TE vendors can use it for training. The same dataset can be for both Option 3 and 4 analysis.

- Procedure for UE model training

  - The procedure shall be based on the eventual RAN1/2 design/specification for inter-vendor training collaboration and Type 3 training framework (separate training at NW side and UE side) with NW-first training can be used as the basis.

Proposal #5:	Conduct Option 4 analysis in parallel with Option 3 reusing same assumptions on Model architecture parameters and Training data set for model training.

## Simulation assumptions

In RAN4 #110bis an initial discussion on the simulation assumptions for test decoder feasibility analysis took place and companies were encouraged to bring parameters/values proposals. For initial link-level analysis we suggest focusing on a simplified analysis/setup focusing on the AI/ML decoder/encoder implementations and consider intermediate performance metrics.

| Parameter | Value |
| --- | --- |
| Frequency range | FR1 |
| Duplex | FDD |
| Bandwidth | 10 MHz |
| Subcarrier spacing | 15 kHz |
| Slot/non-slot | 14 OFDM symbol slot |
| Channel models | 1) CDL-C 300ns DS, 100Hz (TR 38.901) 2) TDLC300-100 (TS 38.101-4) |
| Antenna setup and port layouts at gNB | CDL channel models - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ TDL channel models - Low correlation |
| Antenna setup and port layouts at UE | CDL channel models - 4RX UE: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) TDL channel models - 4RX UE, Low correlation |
| MIMO configurations | Fixed rank 1 as the starting point 4 PRB subband size |
| UE RX assumptions | Realistic channel estimation algorithms MMSE-IRC as the baseline receiver |
| Evaluation metrics | SGCS metric (as described in TR 38.843 6.2.1) or NMSE |

# Conclusion

In this paper we provide our views on general aspects of AI/ML testability and interoperability aspects, and, in summary, make the following proposals:

Proposal #1:	Further discuss and agree on the procedure to conduct analysis of Option 3/4 and on the principles to select the decoder for specification.

Proposal #2:	Further align the AI/ML test methodology with RAN1's conclusions on inter-vendor training collaboration for AI/ML-based CSI compression.

Proposal #3:	Adopt the following definition of Reference decoder/encoder

Reference decoder/encoder: The decoder/encoder model used to define the minimum performance requirements. The Reference decoder is identical to the Test decoder.

Proposal #4	Further discuss and define the upper bound complexity including the number of computations and number of parameters for test/reference encoders/decoders.

Proposal #5:	Conduct Option 4 analysis in parallel with Option 3 reusing same assumptions on Model architecture parameters and Training data set for model training.

# References

- RP-221348, “SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #96, June 2022

- 3GPP TR 38.843 “Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface”

- RP-234039, “WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #102, December 2023

- R4-2403712, “WF on AI/ML”, Qualcomm, RAN4 #110, February 2024

- R4-2406617 WF on AI/ML, Qualcomm, RAN4 #110bis, April 2024

- R1-2402147, “AI/ML for CSI compression”, Intel Corporation, RAN1 #116bis, April 2024

- R4-2405653, “Further study on testability and interoperability issues for AI-CSI” , Samsung, RAN1 #116bis, April 2024