3GPP TSG-RAN WG4 Meeting #111							                 				    R4-2408665

# Fukuoka, Japan, May 20 – May 24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.133 on Cell reselection to FR1 intra-frequency NR for UE operating on a cell with less than 5MHz BW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2024-13-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 introdcued new 12 PRB SSB for UE operating in less than 5 MHz and the requirements needs to be tested |  |  |  |  |  |  |  |  |
| Summary of change: |  | Intrdocuing test case Cell reselection to FR1 intra-frequency NR for UE operating on a cell with less than 5MHz BW |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature is not testable and performance is not guranteed in field |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

#### A.6.1.1.x	Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5MHz BW

##### A.6.1.1.x.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements specified in clause 4.2.2.3. for UE capable of operating on a cell with less than 5MHz BW.

Supported test configurations are specified in Table A.6.1.1.x.2-1. General test parameters as specified in Table A.6.1.1.1.2-2 with config 1 apply except those specified in Table A.6.1.1.x.2-2. Cell specific test parameters as specified in Table A.6.1.1.1.2-3 apply. The test parameters specified in A.6.1.1.1.2 applies to this test. The test procedure specified in A.6.1.1 applies to this test.

Table A.6.1.1.x.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 3 MHz bandwidth |

Table A.6.1.1.x.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| BWchannel | MHz | Config 1 | 3: NRB,c = 12 |  |
| SSB Configuration |  | Config 1 | TBD |  |
| PRACH Configuration index |  | Config 1 | TBD |  |

##### A.6.1.1.x.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34 s + 2 measurement samples for UE operating on a cell with less than 5MHz BW.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than 8 s + 2 measurement samples for UE operating on a cell with less than 5MHz BW..

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra 	+ 40ms See Table 4.2.2.3-1 in clause 4.2.2.3

Tevaluate, NR_ intra See Table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 33.32 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.72 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

<End of change 1>