3GPP TSG-RAN WG4 Meeting #111	R4-2408780

Fukuoka, Japan, 20 May – 24 May 2024

Agenda Item:	7.19.3.1

Source:	Ericsson

Title:	PMI reporting requirements for MIMO evolution

Document for:	Discussion

# 1	Introduction

RAN4#110bis agreed with the way forward on MIMO evolution for Downlink and Uplink [1]. We discuss the open issues on the PMI reporting requirements for Rel-18 MIMO evolution.

# 2	PMI reporting requirements with typeII-Doppler

## 2.1	Simulation results

Issue 1-1-1: Propagation channel for both FDD and TDD cases For Rel18 Doppler codebook requirements, decide the propagation channel from below options based on the updated simulation results on the May meeting For FR1 FDD: - Option 1: TDLA30-30 - Option 2: TDLA30-20 For FR1 TDD: - Option 1: TDLA30-20 - Option 2: TDLA30-10 Issue 1-1-5: Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 FDD 15kHz SCS case, test setup is as below descriptions: - Non-CSI slots: scheduled in mod(i, 10) = {1,3,5,7,8,9}, i={0,1,2,…,19} - CSI-RS is scheduled in mod(i, 10) = {0,2,4,6}, i={0,1,2,…,19} - SSB slots: i=0 - TRS slots: i=10 and 11. - CSI report is transmitted in slot#10n+10 to estimate precoder in slot#10n+11 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 17, 18, 19, 21, 23} Issue 1-1-6: Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 TDD 30kHz SCS case, test setup is as below descriptions for the simulation of following Rel18 Doppler codebook vs random Type I single panel codebook: - Scheduling pattern for TDD SCS=30kHz with the periodicity of 20ms (40 slots): - Non-CSI slots: scheduled in mod(i, 10) = {5,6}, i={0,1,2,…,39} - CSI slots: scheduled in mod(i, 10) = {0,1,2,3}, i={0,1,2,…,39} - SSB slots: i=0 - TRS slots: i=20 and 21. - CSI report is transmitted in slot#10n+9 to estimate precoder in slot#10n+10 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 16}

RAN4#110bis agreed with the scheduling of CSI-RS, PMI reporting and PDSCH for PMI reporting requirements with typeII-Doppler-r18. RAN4 also endorsed the draft CR on PMI reporting test with typeII-Doppler-r18 [2].

### 2.1.1	FDD

Figure 1 and Figure 2 show the FDD SCS=15kHz simulation results with 2Rx and 4Rx UEs, respectively, according to the scheduling agreed in [1][2]. Figure 1 (a) and Figure 2 (a) compares the relative throughput curves with follow PMI with typeII-Doppler-r18 and random PMI with typeI-r15 with different channel model options. Figure 1 (b) and Figure 2 (b) show the throughput ratio of follow PMI with typeII-Doppler-r18 and random PMI with typeI-r15.

| (a) PDSCH throughput with follow PMI | (b) Throughput ratio of follow PMI with typeII-Doppler-r18 and random PMI with typeI-r15. |
| --- | --- |

Figure 1	FDD SCS=15kHz simulation results with 2Rx.

| (a) PDSCH throughput with follow PMI | (b) Throughput ratio of follow PMI with typeII-Doppler-r18 and random PMI with typeI-r15. |
| --- | --- |

Figure 2	FDD SCS=15kHz simulation results with 4Rx.

Table 1 summarizes the FDD SCS=15kHz simulation results for PMI reporting test with typeII-Doppler-r18, that is, the SNR to achieve 90% of the maximum throughput with follow PMI with typeII-Doppler-r18, and the throughput ratio of follow PMI with typeII-Doppler-r18 and random PMI with typeI-r15. From the simulation results, it is observed that the TDLA30-20 shows larger throughput gain compared with TDLA30-30 for both 2Rx and 4Rx.

Observation 1: TDLA30-20 shows larger throughput gain compared with TDLA30-30 for both 2Rx and 4Rx.

Table 1	Summary of FDD SCS=15kHz simulation results of the PMI reporting tests with typeII-Doppler-r18.

| Channel model | Receive antennas | SNR (dB) to achieve 90% of the maximum throughput with follow PMI | Throughput ratio of follow PMI and random PMI |
| --- | --- | --- | --- |
| TDLA30-20 | 2Rx | 5.6 (dB) | 2.4 |
|  | 4Rx | 1.2 (dB) | 2.6 |
| TDLA30-30 | 2Rx | 8.9 (dB) | 1.7 |
|  | 4Rx | 2.7 (dB) | 2.1 |

Although the typeII-Doppler-r18 targets relatively higher Doppler frequency, considering the throughput gain from the simulation results, we slight prefer to set TDLA30-20 for FDD SCS=15kHz. However we are also fine with configuring TDLA30-30 because of the reasonable throughput gain.

Proposal 1: For the FDD SCS=15kHz PMI reporting requirements with typeII-Doppler-r18, configure TDLA30-20.

Proposal 2: For the FDD SCS=15kHz PMI reporting requirements with typeII-Doppler-r18, set γ=2.0 for both 2Rx and 4Rx.

### 2.1.2	TDD

Figure 3 and Figure 4 shows the TDD SCS=30kHz simulation results with 2Rx and 4Rx, respectively, according to the scheduling agreed in [1][2].

| (a) PDSCH throughput with follow PMI | (b) Throughput ratio of follow PMI with typeII-Doppler-r18 and random PMI with typeI-r15. |
| --- | --- |

Figure 3		TDD SCS=30kHz simulation results with 2Rx.

| (a) PDSCH throughput with follow PMI | (b) Throughput ratio of follow PMI with typeII-Doppler-r18 and random PMI with typeI-r15. |
| --- | --- |

Figure 4		TDD SCS=30kHz simulation results with 4Rx.

Table 2	Summary of TDD SCS=30kHz simulation results of the PMI reporting tests with typeII-Doppler-r18.

| Channel model | Receive antennas | SNR (dB) to achieve 90% of the maximum throughput with follow PMI | Throughput ratio of follow PMI and random PMI |
| --- | --- | --- | --- |
| TDLA30-10 | 2Rx | 2.2 (dB) | 5.0 |
|  | 4Rx | -0.7 (dB) | 4.2 |
| TDLA30-20 | 2Rx | 3.4 (dB) | 3.5 |
|  | 4Rx | 0.1 (dB) | 3.3 |

Table 2 summarizes the TDD SCS=30kHz simulation results for PMI reporting test with typeII-Doppler-r18, that is, the SNR to achieve 90% of the maximum throughput with follow PMI with typeII-Doppler-r18, and the throughput ratio of follow PMI with typeII-Doppler-r18 and random PMI with typeI-r15. From the simulation results, it is observed that the TDLA30-10 results in larger throughput gain compared with TDLA30-20 for both 2Rx and 4Rx.

Observation 2: TDLA30-10 shows larger throughput gain compared with TDLA30-20 for both 2Rx and 4Rx.

Considering the larger throughput gain, we slight prefer to set TDLA30-10 for TDD SCS=30kHz. However we are also fine with configuring TDLA30-20 because the throughput gain is also high enough.

Proposal 3: For the TDD SCS=30kHz PMI reporting requirements with typeII-Doppler-r18, configure TDLA30-10.

Proposal 4: For the TDD SCS=30kHz PMI reporting requirements with typeII-Doppler-r18, set γ=4.0 for 2Rx and γ=3.5 for 4Rx.

## 2.2	PMI delay

Although RAN4 endorsed the draft CR on PMI reporting requirements [2], RAN4 has not well discussed the CQI/RI/PMI delay for predicted PMI scenario. Since the existing CSI reporting requirements are based on single NZP-CSI-RS resource, the CQI/RI/PMI delay is set from the slot CSI-RS is transmitted to the time the (first) PDSCH is transmitted where the CSI is applied. On the other hand, the predicted PMI is configured with K=4 NP-CSI-RS resources, it is not clear the definition of CQI/RI/PMI delay.

Figure 5	Scheduling of CSI-RS resources, CSI report, and PDSCH for FDD SCS=15kHz.

Figure 6	Scheduling of CSI-RS resources, CSI report, and PDSCH for TDD SCS=30kHz.

Figure 5 and Figure 6 illustrates the scheduling of CSI-RS resources, CSI report, and PDSCH for FDD SCS=15kHz and TDD SCS=30kHz, respectively. If we define CQI/RI/PMI delay from the time the first NZP-CSI-RS resource transmission to the time the first PDSCH transmission where PMI is applied, the PMI delay is 15ms and 7.5ms for SCS=15kHz and SCS=30kHz, respectively, and which is same values as the endorsed CRs in []. To avoid the misunderstanding in the future, we propose to describe the definition of the CQI/RI/PMI delay parameters for the predicted PMI reporting requirements, as shown in Table 3.

Table 3	Example of the note for CQI/RI/PMI delay for predicted PMI reporting requirements.

| Parameter | Unit | Value |
| --- | --- | --- |
| … |  | … |
| CQI/RI/PMI delay (Note 4) | ms | [15] |
| … |  | … |
| … Note 4: Delay is defined as the time from the first NZP CSI-RS resources for CSI acquisition to the time to the first PDSCH transmission where the reported PMI is applied. |  |  |

Proposal 5: Add the following note to CQI/RI/PMI delay in the Predicted PMI reporting test parameters:

- CQI/RI/PMI delay: Delay is defined as the time from the first NZP CSI-RS resources for CSI acquisition to the time to the first PDSCH transmission where the reported PMI is applied.

Proposal 6: Confirm the CQI/RI/PMI delay is 15ms for FDD SCS=15kHz and 7.5ms for TDD SCS=30kHz.

# 3	PMI reporting requirements with typeII-CJT

RAN4#110bis endorsed the draft CR on PMI reporting requirements with typeII-CJT-r18 [2], but the final requirements are TBD.

Issue 1-2-7: Test metric of TypeII-CJT-r18 codebook Decide the gamma value based on the updated simulation results on the May meeting

Figure 7 (a) show our FDD SCS=15kHz simulation results of follow PMI with typeII-CJT-r18 and random PMI with typeI-r15 for 2Rx and 4Rx. Figure 7 (b) also show the throughput ratio between follow typeII-CJT-r18 and random typeI-r15. We also show the simulation results with TDD SCS=30kHz in Figure 8.

| (a) PDSCH throughput with follow PMI and R15 random PMI typeI. | (b) Throughput ratio. |
| --- | --- |

Figure 7		Simulation results with PMI reporting test with typeII-CJT (FDD SCS=15kHz).

| (a) PDSCH throughput with follow PMI and R15 random PMI typeI. | (b) Throughput ratio. |
| --- | --- |

Figure 8		Simulation results with PMI reporting test with typeII-CJT (TDD SCS=30kHz).

Table 3 summarizes the simulation results of the PMI reporting tests with typeII-CJT-r18. Based on the simulation results we propose to set γ=1.8 for FDD/TDD with 2Rx/4Rx.

Table 4	Summary of simulation results of the PMI reporting tests with typeII-CJT-r18.

| FDD/TDD and SCS | Receive antennas | SNR (dB) to achieve 90% of the maximum throughput with follow PMI | Throughput ratio of follow PMI and random PMI |
| --- | --- | --- | --- |
| FDD SCS=15kHz | 2Rx | 8.9 (dB) | 2.3 |
|  | 4Rx | 5.3 (dB) | 2.3 |
| TDD SCS=30kHz | 2Rx | 8.7 (dB) | 2.4 |
|  | 4Rx | 5.5 (dB) | 2.3 |

Proposal 7: For the PMI reporting requirements with typeII-CJT-r18, set γ=1.8 for FDD/TDD with 2Rx/4Rx.

# 4	Summary

Observation 1: TDLA30-20 shows larger throughput gain compared with TDLA30-30 for both 2Rx and 4Rx.

Proposal 1: For the FDD SCS=15kHz PMI reporting requirements with typeII-Doppler-r18, configure TDLA30-20.

Proposal 2: For the FDD SCS=15kHz PMI reporting requirements with typeII-Doppler-r18, set γ=2.0 for both 2Rx and 4Rx.

Observation 2: TDLA30-10 shows larger throughput gain compared with TDLA30-20 for both 2Rx and 4Rx.

Proposal 3: For the TDD SCS=30kHz PMI reporting requirements with typeII-Doppler-r18, configure TDLA30-10.

Proposal 4: For the TDD SCS=30kHz PMI reporting requirements with typeII-Doppler-r18, set γ=4.0 for 2Rx and γ=3.5 for 4Rx.

Proposal 5: Add the following note to CQI/RI/PMI delay in the Predicted PMI reporting test parameters:

- CQI/RI/PMI delay: Delay is defined as the time from the first NZP CSI-RS resources for CSI acquisition to the time to the first PDSCH transmission where the reported PMI is applied.

Proposal 6: Confirm the CQI/RI/PMI delay is 15ms for FDD SCS=15kHz and 7.5ms for TDD SCS=30kHz.

Proposal 7: For the PMI reporting requirements with typeII-CJT-r18, set γ=1.8 for FDD/TDD with 2Rx/4Rx.

# References

- R4-2406111, “WF on [110bis][328] NR_MIMO_evo_DL_UL_demod”, Samsung.

- R4-2404756, “Draft Big CR for UE demodulation and CSI requirements for Rel-18 MIMO in 38.101-4”, Samsung.