3GPP TSG-RAN WG4 Meeting #111				                              R4-2408830

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	7.20.1.1

Source: 	OPPO

Title: 	further simulation results for NS values for SL-U

Document for:	Approval

- Introduction

In RAN4#106bis meeting, the NS values for further SL-U development has been discussed and A-MPR for NS_28 has been additionally agreed besides the already agreed 5 NS values in Rel-18 timeline. Still there are 11 NS values left for accomplishment. Due to the limit time, in this paper, we give the simulation results for NS_29 and NS_54 for further consideration.

- Discussion

For NS_29, the emission requirement and PSD requirement differ with different channel bandwidth and hence simulation are down with different channel bandwidth. The simulation result is shown below:

Table 1 Simulation results for 20 and 40MHz

|  | 20MHz channel bandwidth |  |  |  | 40MHz channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 15 kHz contiguous | 30 kHz contiguous | 15 kHz Interlaced | 30 kHz Interlaced | 15 kHz contiguous | 30 kHz contiguous | 15 kHz Interlaced | 30 kHz Interlaced |
| QPSK | 4.03 | 4.11 | 4.19 | 4.28 | 3.22 | 3.22 | 3.14 | 3.38 |
| 16QAM | 4.03 | 4.03 | 4.20 | 4.28 | 3.70 | 3.70 | 3.54 | 3.38 |
| 64QAM | 4.97 | 5.23 | 4.37 | 4.28 | 5.05 | 4.97 | 5.05 | 4.11 |
| 256QAM | 8.80 | 8.61 | 7.86 | 7.49 | 8.61 | 8.80 | 7.40 | 6.48 |

Table 2 Simulation results for 600 and 800MHz

|  |  | contiguous |  | Interlaced |  | Contiguous Bitmap |  |  |  |  |  |  |  | Interlaced Bitmap |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 60MHz | 80 MHz | 60 | 80 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 |
| Contiguous | QPSK | 3.2 | 3.2 | 3.2 | 3.3 | 3.5 | 2.8 | 3.6 | 2.8 | 3.5 | 2.8 | 3.5 | 2.8 | 5.9 | 3.0 | 6.0 | 2.9 | 5.9 | 2.6 | 5.9 | 2.9 |
|  | 16QAM | 3.7 | 3.7 | 3.2 | 3.3 | 3.5 | 3.6 | 3.6 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 | 6.0 | 3.0 |
|  | 64QAM | 5.2 | 5.1 | 4.1 | 3.9 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 6.0 | 4.2 | 6.0 | 4.2 | 5.9 | 4.1 | 5.9 | 4.2 |
|  | 256QAM | 8.6 | 8.5 | 6.6 | 6.5 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 |

For NS_54, the band edge spectrum emission limit -27 dBm/MHz is the dominant factor and hence the band edge will suffer lager A-MPR while for the channels in the middle of the band can have smaller A-MPR. The requirement is shown as below.

Table 3 Spectrum emission limit for NS_54

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

For the A-MPR simulation result, they are captured below for edge channel and non-edge channel. For QPSK and 16QAM single CC with smaller bandwidth, the edge effect is more obvious. For higher order modulation as 64QAM and 256QAM, the dominant factor is more like EVM and hence no big difference found for edge and non-edge channel.

Table 4 A-MPR for single CC NS_54 edge channel

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 4.4 | 4.4 | 3.5 | 3.5 | 3.2 | 3.2 | 3.2 | 4.5 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 16QAM | 4.4 | 4.4 | 3.7 | 3.7 | 3.7 | 3.7 | 3.6 | 4.4 | 4.6 | 3.5 | 3.5 | 3.2 | 3.3 | 3.2 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 4.5 | 4.6 | 4.1 | 4.4 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

Table 5 A-MPR for single CC NS_54 non-edge channel

| case | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 |

The wide-band operation simulation result is further provided below:

Table 6 A-MPR for Wideband operation NS_54 edge channel

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 3.5 | 3.0 | 3.0 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 3.7 | 3.1 | 3.1 | 2.8 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.7 | 2.7 | 2.7 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 3.7 | 3.1 | 3.1 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 3.0 | 2.8 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Table 7  A-MPR for Wideband operation NS_54 non-edge channel

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 |

Proposal: It is proposed to capture the simulation results into TR 38.786.

3	Conclusions

In this contribution, we give further simulation results for PSSCH and PSCCH for SL-U A-MPR with NS_29 and NS_54.

Proposal: It is proposed to capture the simulation results into TR 38.786.

4 References