3GPP TSG-RAN WG4 Meeting #111				                              R4-2408833

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	10.9.2.2

Source: 	OPPO

Title: 	on Intra-band non-contiguous CA TX requirements

Document for:	Approval

- Introduction

In RAN4#110bis meeting, the intra-band non-contiguous CA has been discussed. As it is the first meeting, mostly the scenarios and A-MPR simulation assumptions are discussed while for the RF requirements, they are left open. In this contribution we would like to finalize the A-MPR simulation assumption and also give some initial discussion on the RF requirements.

- Discussion

2.1 MPR A-MPR simulation

In the WF [1] agreed in the last RAN4#110bis meeting, the A-JMPR architecture

WF:

  - For PC3 and PC2 intra-band non-contiguous CA, it is proposed to use Option1 and Option 2 as a starting point assumption for MPR/A-MPR simulation.

  - RAN4 can consider all candidate RF architectures to derive PC3/PC2 MPR/A-MPR requirements for intra-band non-contiguous CA.

  - Further discuss these options in the next meeting.

The different architecture assumption has been also listed in the WF,

Firstly, for PC2 intra-band non-contiguous CA:

- Option 1: 1PA + 1LO

Figure 2-a PC2/PC3 intra-band non-contiguous CA assumption of 1PA+1LO

- Option 2: 2PA + 2LO,

Figure 2-b PC2/PC3 intra-band non-contiguous CA assumption of 2PA+2LO

- Option 3: 2PA + 1LO

Figure 2-c PC2/PC3 intra-band non-contiguous CA assumption of 2PA+1LO

For the NR intra-band non-contiguous CA requirement, it has been captured from TS 38.101-1 as:

6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA

6.2A.2.2.0	General

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

For UE indicating dualPA-Architecture supported

If OR (LCRB1 = 0, LCRB2 = 0)

MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively

Else If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively.

Else

MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively.

For UE without indicating dualPA-Architecture supported

If OR( LCRB1 = 0, LCRB2 = 0 )

For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used;

For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used;

For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used.

Else If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2A.2.2.2.3 and Clause 6.2A.2.2.2.4 for PC3 and PC2 UE respectively.

Else

MPR defined in Clause 6.2A.2.2.1.3 and Clause 6.2A.2.2.1.4 for PC3 and PC2 UE respectively.

Similar discussion on intra-band contiguous CA MPR requirement has been carried out in our companion paper and hence we don’t repeat them again but to capture the observations below which is the same for intra-band non-contiguous CA.

Observation 1: There are two table for PC2 sidelink MPR with 1TX and 2TX.

Observation 2: 2TX in PC2 sidelink MPR is defined as two PAs.

Observation 3: For NR intra-band non-contiguous CA, the dualPA-Architecture IE is used to differentiate the MPR requirements.

Observation 4: The dualPA-Architecture IE assumes dual PA and dual LO.

In such case, to derive the MPR for PC2 intra-band non-contiguous sidelink CA, as agreed in the previous meeting, both 1TX and 2TX needs to be considered. Here 2 TX at least should refers to 2 PAs which is the same for both PC2 NR intra-band CA and PC2 Sidelink.

Proposal 1: To consider both 1 PA and 2 PA architecture for intra-band non-contiguous sidelink CA MPR simulation for both PC3 and PC2.

Although the frequency range is only 70MHz for n47, still to consider one LO to cover both TX chain seems not a normal implementation and hence in the last meeting it is agreed option 1 and option 2 are considered as baseline assumption. In our view, for intra-band non-contiguous CA, the option 3 can be excluded for simulation scenarios.

Proposal 2: The 2PA+1LO architecture can be precluded for intra-band non-contiguous CA A-MPR simulation for both PC2 and PC3..

2.2 UE TX requirements

As currently the intra-band PC3 CA has already been finished in Rel-18, we try to figure out the requirements that will be impacted by PC2 and PC3 intra-band non-contiguous CA.

| Requirement | Update |
| --- | --- |
| UE power class | New sub-clause needed for intra-band non-contiguous CA |
| MPR/A-MPR | New sub-clause needed for intra-band non-contiguous CA |
| Configured output power | New sub-clause needed for intra-band non-contiguous CA |
| Output power dynamics | New sub-clause needed for intra-band non-contiguous CA |
| Transmit signal quality | New sub-clause needed for intra-band non-contiguous CA |
| Output RF spectrum emissions | New sub-clause needed for intra-band non-contiguous CA |

Proposal 3: To add new sub-clause to introduce UE TX requirement of intra-band non-contiguous CA.

3	Conclusions

In this contribution, we give further analysis on the intra-band non-contiguous sidelink CA TX requirements. The observations and proposals are as below.

Proposal 1: To consider both 1 PA and 2 PA architecture for intra-band non-contiguous sidelink CA MPR simulation for both PC3 and PC2.

Proposal 2: The 2PA+1LO architecture can be precluded for intra-band non-contiguous CA A-MPR simulation for both PC2 and PC3..

Proposal 3: To add new sub-clause to introduce UE TX requirement of intra-band non-contiguous CA.

4 References

[1] R4-2406611 	WF on NR_SL_ intraB_CA_ITS, OPPO, Meta