3GPP TSG-RAN WG4 Meeting #111                                                                  R4-2408886

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source:	Samsung

Title:                        Discussion on defining the missing testing parameter for PC1/5/6

Agenda item:			11.1

Document for:	Discussion

1. Introduction

In RAN4 #109 meeting, RAN5 sent a LS [1] on defining the missing relative angular offsets and UE gain-related parameters for different power classes to RAN4. In the LS, they indicate there are some testing parameters are missing, and PC6 would be better to define the parameters shown in the following Table, otherwise the WI cannot be completed in RAN5.

In RAN4 #110 meeting, RAN4 had some initial discussions on the missing parameters, and some concrete values were agreed with brackets in [2] for the missing parameters. There were extensive discussions on the missing values in last meeting, and the related open/remaining issues can be found in the agreed WF [3]. In this contribution, we continue to discuss the values of missing parameters.

2 Discussion

2.1 What is the assumption for Gain difference Y and Z between fine beam and rough beam for PC1/5/6?

From our understanding, what would mostly differentiate the requirements of different types of UE should be the number of antenna element in an antenna array. From element notion design perspective, the antenna elements to be considered for PC1/PC5/PC6 are: 32, 16, 16 [4-5]. However, we acknowledge that if we use such values to define Y/Z in RRM, the gain differences seem to be too tighten, since a good UE may has better performance than the minimum requirement of EIS peak. From this, we agree to define a loosen Y/Z to ensure the compatibility. The following [18]/[15.5] dB were agreed for Y and Z for PC1/PC5/PC6 with brackets in RAN4 #110,

Y Value “Y” in dB, for each UE power class. 1; 2; 3; 4; 5; 6; 7. [18]; 9.0; 7.0; FFS; [15.5]; [15.5]; FFS. Z Value “Z” in dB, for each UE power class. 1; 2; 3; 4; 5; 6; 7. [18]; 9.0; 7.0; FFS; [15.5]; [15.5]; FFS.

The derivation behind the values are

| Power Class | # of antenna elements | Y/Z |
| --- | --- | --- |
| PC1 | 64 | 10*log(64)=18 |
| PC5 | 36 | 10*log(36)=15.5 |
| PC6 | 36 | 10*log(36)= 15.5 |

where 64/36/36 are typical implementations, mentioned in [6]. It is reasonable for us to adopt such values to compute Y/Z in Annex B, and the square brackets on those numbers can be removed accordingly.

Proposal 1: For the gain difference Y and Z between fine beam and rough beam for PC1/5/6, the square brackets can be removed.

Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction

| Value "Y" in dB, for each UE power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 18 | 9.0 | 7.0 | FFS | 15.5 | 15.5 | FFS |

Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions

| Value "Z" in dB, for each UE power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 18 | 9.0 | 7.0 | FFS | 15.5 | 15.5 | FFS |

2.2 What is the UE gain G for PC 1/5/6?

Actually, the minimum and maximum allowable value of G can be derived as below:

- Minimum allowable value of G:

We take PC6 as example, from our understanding, the value can be derived from EIS requirements and Refsens based on the following HST SLS assumption, some assumed parameters are updated on top of our contribution submitted in last meeting [9].

| 28GHz，n257 |  |
| --- | --- |
| Max. RX BW (Max. Received Bandwidth) | 50 MHz |
| IL (Implementation Losses) | [7.6 dB] |
| UE Noise Figure (F_UE) | 10 dB (from 38.854) |
| Nktb (Thermal noise level) | -174 dBm/Hz |
| Diversity Gain | 0 |
| SNR/SINR | -1 dB (SNR target of -1 dB) |
| Reference sensitivity for power class 6 | -92.6 dBm |
| Reference sensitivity for power class 1 | -97.5 dBm |

Then based on the definition of Sensitivity equation in mmWave, that is

    - Sensitivity = -174dBm(Nktb) + 10*log(Max. RX BW) + NF – Total Ant. gain - diversity gain + SNR + ILs    (1)

    - Then, we can obtain:

  - Total Ant. gain=12.19≈12 dBi           (2)

And the 12 dBi also align to observation in [6]. Following the previous logic to obtain minimum allowable gain for PC3, that is considering there is a 2*Y dB difference between fine and rough beam [8], then the minimum rough antenna gain in the peak direction is 12-2*Y. If we can align Y=15.5 for PC6, we then obtain -19 dBi. Furthermore, a 3 dB margin is allowed for electrical losses, then finally we have -22 dBi for the minimum allowable value of G.

The same logic, for PC5, Total Ant. gain≈12 dBi, for PC1, Total Ant. gain≈17 dBi, then for PC5: Gmin= -22 dBi, for PC1: Gmin=17-2*18=-22 dBi

- Maximum allowable value of G:

Although we can understand the intention of defining 57 dBi, the value is too large. A large value will make the testing becomes meaningless, and actually we wonder that whether such a high gain could practically be used in a NR UE.  Of course, if we to define the maximum allowable value of G based on (5+10log(16)+3)+3, the range is too stringent. To guarantee the compatibility, we prefer to use the following values: 64/36/36 to define the Gmax

PC1: Gmax=5+10log⁡(64)+3=26 dBi

PC5: Gmax=5+10log⁡(36)+3=23.5≈24 dBi

PC6: Gmax=5+10log⁡(36)+3=23.5≈24 dBi

Proposal 2: The UE gain G for PC 1/5/6 are:

Table B.2.1.5.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Minimum, dBi | -22 | FFS | -10 | FFS | -22 | -22 | FFS |
| Maximum, dBi | +26 | FFS | +20 | FFS | +24 | +24 | FFS |

2.3 What is the UE rough beam gain reduction D for PC 1/5/6?

For FR2 inter-frequency relative RSRP accuracy, the accuracy relaxation Ginter, D and E were introduced. To be specific, the D and E are taken as the additional margins to compensate the impact of mis-alignment between fine beam and rough beam. The D is used to limit the lower bound of SS-RSRP relative accuracy test requirement (Table A.7.7.1.2.3-2), and the corresponding explanation can be found in TR 38.903, below:

TR 38.903 Section A.2.2 UE Fine beams and Rough beams Figure A.2.2.1-3: Fine and rough beams alignment, Rx Beam peak direction

For PC6, actually, the test A.7.7 will not to be discussed in HST in Rel-18, and the discussions was not occurred in Rel-17. And checked with RAN5, they don’t need the parameter as well. From this, we think there is no need to define D for PC6.

Proposal 3: There is no need to define the value of rough beam gain reduction “D” in B.2.1.5 for PC6

Recalling the derivation of D for PC3, we can find that the 5.5 dB is from 3.5 dB difference plus additional margin (insufficient sample) 2 dB, while the worst case 3.5 dB (SSB_RP3, peak - SSB_RP2, 50% sphere coverage)) highly depends on the UE implementation and simulation results [7]. From this, we think it is a little bit difficult to obtain a precise solution in terms of the D for PC1/PC5. Let’s try to give a rough analysis by taking FWA as example:

- More active antenna element considered for PC1 than PC3 (32 vs 4)

- Beamformed EIS theoretically would improve by 3 dB when doubling the number of active antenna element in an array approximately

- Spherical coverage for FWA UE type: 8.0 dB down from peak at 85%-tile point of CDF, and the spherical coverage can be relatively narrow

From this, we think the D for FWA will become looser, since the narrower the beams, the severer the impact of mis-alignment between fine beam and rough beam.

Observation 1: The D for FWA will become looser than PC3.

To derive the D roughly,

AoA2 Rx rough beam peak-AoA1 Rx rough X% Spherical coverage=AoA2 Rx fine beam peak-Y-AoA1 Rx fine X% Spherical coverage-Z=AoA2 Rx fine beam peak-Y-AoA1 Rx fine beam peak-X-Z=AoA2 Rx fine beam peak-Y-AoA1 Rx fine beam peak+X+Z=AoA2 Rx fine beam peak-AoA1 Rx fine beam peak+X

For PC1, the X=-8, for PC5, the X=-8. Then we have

Proposal 4: The value of rough beam gain reduction “D” in B.2.1.5 for PC1/PC5 are as follows:

Table B.2.1.5.3-1: Rough Beam gain reduction “D” in Rx Beam Peak direction

|  | UE Power class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| Maximum gain reduction, dB | 10 | FFS | 5.5 | FFS | 10 |

2.4 What is Ginter for PC 1/5/6?

Ginter is the margin due to different antenna gain on different bands, which comes from RF transceiver gain difference. Separation in frequency between two carriers no matter in inter-/intra-band is the main thing. Since in HST, the applicable frequency band is 28 GHz, the candidate frequency bands including band n261, n257 and n258, which in line with PC3 assumption. That means the Ginter is still applicable to PC6 and so as the PC1. But we are skeptical about the applicability of the Ginter for PC5, since n261 is not considered for PC5.

Proposal 5: For UE gain difference between inter-frequencies Ginter, 3dB for PC1 and PC6

2.5 Whether UE gain to PRS-RSRP measurement point for FR2 needs to be defined for PC 1, PC 5, and PC 6?

Regarding Clause 2.1.6 Gain to PRS-RSRP measurement point for FR2, since the section was introduced for PRS based measurement, the PRS-RSRP accuracy requirement, which is for FR2 positioning topic instead of HST, and positioning discussion was not included in the HST scope, we don’t think there is a need to define the parameter for PC6

Proposal 6: There is no need to define G gain in Clause 2.1.6 Gain to PRS-RSRP measurement point for FR2 for PC6

Regarding the other PCs, have checked the intention and discussion of introducing G to PRS-RSRP measurement point for FR2 in Annex B [10], the same value as G in Table B.2.1.5.1-1 was used for PC3. From this, if we can achieve alignment on G defined to SS-RSRP and CSI-RSRP measurement point for FR2 in Section 2.2, we prefer that

Proposal 7: For the UE gain to PRS-RSRP measurement point for FR2 for PC1/PC5

Table B.2.1.6.1-1: UE gain G, Rx beam peak direction

|  |  | UE Power class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 7 |
| Minimum, dBi | -22 | FFS | -10 | FFS | -22 | FFS |
| Maximum, dBi | +26 | FFS | +20 | FFS | +24 | FFS |

3 Conclusions

In this contribution, we provided our viewpoints to RAN5 LS on defining the missing relative angular offsets and UE gain-related parameters for different power classes to RAN4. The following observations and proposals are obtained:

Observation 1: The D for FWA will become looser than PC3.

Proposal 1: For the gain difference Y and Z between fine beam and rough beam for PC1/5/6, the square brackets can be removed.

Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction

| Value "Y" in dB, for each UE power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 18 | 9.0 | 7.0 | FFS | 15.5 | 15.5 | FFS |

Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions

| Value "Z" in dB, for each UE power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 18 | 9.0 | 7.0 | FFS | 15.5 | 15.5 | FFS |

Proposal 2: The UE gain G for PC 1/5/6 are:

Table B.2.1.5.1-1: UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Minimum, dBi | -22 | FFS | -10 | FFS | -22 | -22 | FFS |
| Maximum, dBi | +26 | FFS | +20 | FFS | +24 | +24 | FFS |

Proposal 3: There is no need to define the value of rough beam gain reduction “D” in B.2.1.5 for PC6

Proposal 4: The value of rough beam gain reduction “D” in B.2.1.5 for PC1/PC5 are as follows:

Table B.2.1.5.3-1: Rough Beam gain reduction “D” in Rx Beam Peak direction

|  | UE Power class |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| Maximum gain reduction, dB | 10 | FFS | 5.5 | FFS | 10 |

Proposal 5: For UE gain difference between inter-frequencies Ginter, 3dB for PC1 and PC6

Proposal 6: There is no need to define G gain in Clause 2.1.6 Gain to PRS-RSRP measurement point for FR2 for PC6

Proposal 7: For the UE gain to PRS-RSRP measurement point for FR2 for PC1/PC5

Table B.2.1.6.1-1: UE gain G, Rx beam peak direction

|  |  | UE Power class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 7 |
| Minimum, dBi | -22 | FFS | -10 | FFS | -22 | FFS |
| Maximum, dBi | +26 | FFS | +20 | FFS | +24 | FFS |

4 References

[1] R5-237837, “LS on defining the missing relative angular offsets and UE gain-related parameters for different power classes”, Nokia

[2] R4-2403446, “Ad-hoc minutes on Reply_LS”, Samsung

[3] R4-2406391, “WF on defining the missing testing parameter for PC1/5/6”, Samsung

[4] R4-1903394, “Reconsideration for PC4 REFSENS requirements”, MTK

[5] R4-2106100, “WF on FR2 HST Deployment Scenario Analysis”, Samsung

[6] R4-2401607, “Missing Parameters for FR2 RRM Testing of Different Power Classes”, Qualcomm Incorporated

[7] R4-2118098, “Discussion on FR2 inter-frequency relative RSRP accuracy”, MTK

[8] R4-1909564, “Ideal RSRP in NR RRM tests”, Ericsson

[9] R4-2405440, “Discussion on defining the missing testing parameter for PC1/5/6 and reply LS”, Samsung

[10] R4-2109939, “Remaining issues on PRS-RSRP accuracy requirements”, vivo