3GPP TSG-RAN WG4 Meeting # 111	R4-2408997

# Fukuoka, Japan, May 19th – 20th, 2024

Title: 	Discussion on extension FR2 UE power class 7 to non-RedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	5.3

Document for:	Discussion

Background

UE Power class 1~7 are introduced to specify the corresponding performance requirements for FR2 based on the assumption of different UE architecture as stated in TS 38.101-2 section 6.2.1.0. The assumed UE types during the discussion are also listed for reference in the specification. But there are some restrictions on the applicability of PC7 that are different from other UE power classes 1~6. In this contribution, we share our views on the applicability of PC7.

- Discussion

During the discussion on the introduction of each UE Power class 1~7 for FR2, certain specific UE device architecture is assumed, such as the assumed UE device architecture is single panel, dual polarization, 2x1 array [1] for PC7. This is clearly reflected in the related performance requirements, such as the min peak EIRP 16.4 dBm which is 6dB below the min Peak EIRP of the PC3 [2], and also captured in the specification TS 38.101-2 as below:

### 6.2.1 UE maximum output power #### 6.2.1.0 General NOTE: Power classes are specified based on the assumption of certain UE types with specific device architectures. The UE types can be found in Table 6.2.1.0-1. Table 6.2.1.0-1: Assumption of UE Types UE Power class; UE type. 1; Fixed wireless access (FWA) UE. 2; Vehicular UE. 3; Handheld UE. 4; High power non-handheld UE. 5; Fixed wireless access (FWA) UE. 6; High Speed Train Roof-Mounted UE. 7; RedCap UE. Note: RedCap variants of non-RedCap UEs are not precluded.

So we can understand that the listed UE types in Table 6.2.1.0-1 of TS 38.101-2 are only example UE types for the assumed UE specific device architecture instead of the only UE Type to which one UE power class can be applied, i.e. no one-to-one mapping relationship between UE power class and the UE type listed in Table 6.2.1.0-1 of TS 38.101-2. For example, the Note in Table 6.2.1.0-1 “Note: RedCap variants of non-RedCap UEs are not precluded” specifies that UE power class 1 to power class 6 are also applicable for RedCap UE besides those listed example UE types. Also the UE power class capability clarification in RAN2 specification TS 38.306, it clearly specifies that UE can support different power class by reporting the supported UE power class as yellow highlighted below:

ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 For FR1, if the UE supports the different UE power class than the default UE power class as defined in clause 6.2 of TS 38.101-1 [2], or in clause 6.2 of TS 38.101-5 [34], the UE shall report the supported UE power class in this field. For FR2, UE shall report the supported UE power class as defined in clause 6 and 7 of TS 38.101-2 [3] in this field. UE indicating support for pc6 supports the enhanced intra-NR RRM and demodulation processing requirements for FR2 to support high speed up to 350 km/h as specified in TS 38.133 [5]. This capability is not applicable to IAB-MT. The power class pc7 is only applicable for RedCap UEs operation in FR2. This capability is not applicable for UEs indicating support of maxOutputPowerATG-r18.

Observation 1: The UE power class definition for FR2 is based on certain assumed UE device architecture, the applicable UE types are not limited to those listed UE types in Table 6.2.1.0-1 of TS 38.101-2 which are only example UE types.

Observation 2: PC1 ~ PC6 are also applicable for RedCap UE.

So we think that it is intuitive to make PC7 to be also applicable to non-RedCap UE with the similar architecture as RedCap UE, such as some wearable devices for FR2 also have single panel and dual polarization architecture due to the limited form factor, but they can support larger bandwidth than 100MHz to support higher resolution video call, video tracking and so on, such as 200MHz per CC that is the basic requirement in China. This kind of device has higher throughput than RedCap UE and can be used to meet diverse needs of people that require higher throughput with increasing richer and varied life, but it only requires reasonable device cost compared to power class 7 with less antenna panel and smaller antenna array. UE power class 7 is strictly limited to be only applicable to RedCap UE, such as only max 100MHz channel bandwidth for minimum output power defined, no CA and UL-MIMO related requirements and etc., Also the UE power class 7 capability clarification in RAN2 TS 38.306 “The power class pc7 is only applicable for RedCap UEs operation in FR2.” as shown above in blue highlighted specifies that PC7 is only applicable for RedCap.

Observation 3: The existing transmitter and receiver requirements defined for PC7 are only applicable for RedCap UE.

Actually RedCap UE can be identified by the network by using the reported UE capability supportOfRedCap-r17 that includes all related RedCap functional components as defined in section 4.2.21.2 of TS 38.306. With the extension PC7 to non-RedCap UE, RedCap UE can still report PC7 with additional UE capabilities of supportedBandwidthUL and supportedBandwidthUL-v1710, supportedBandwidthDL and supportedBandwidthDL-v1710 for the supported maximum channel bandwitdth 20 MHz for FR1 and 100 MHz for FR2, not reporting CA and UL-MIMO related UE capabilities to decide the corresponding requirements to be met.

Observation 4: Extension PC7 to non-RedCap UE has no impact to RedCap UE. RedCap UE can decide to pass which transmitter and receiver requirements of PC7 by reporting UE capabilities of supportOfRedCap-r17, the supported UL and DL bandwidth and not report CA and UL-MIMO capabilities.

To extend the requirements defined for PC7 to be applicable for non-RedCap UE, the main work is to define requirements for 200MHz channel bandwidth if necessary, define CA and UL-MIMO related performance requirements. In general, the necessary specification work on TS 38.101-2 can be summarized as below:

Table 1: The summary of extension of PC7 to non-RedCap UE

| Requirements | Section | Requirements for PC7 | Condition for PC7 | Extend PC7 to non-RedCap UE |
| --- | --- | --- | --- | --- |
| Maximum output power | 6.2.1.7 | UE maximum output power for power class 7 | Non-CA configuration | Remove the non-CA configuration limitation |
| Transmit power for CA | 6.2A.2 | UE maximum output power reduction for CA | n/a | Define MPR for CA for PC7 under new section 6.2A.2.8 |
|  | 6.2A.3.3 | A-MPR for CA_NS_202 | n/a | Define A-MPR for CA_NS_202 for PC7 under new section 6.2A.3.3.7 |
|  | 6.2A.3.4 | A-MPR for CA_NS_203 | n/a | Define A-MPR for CA_NS_203 for PC7 under new section 6.2A.3.4.7 |
| Transmit power for UL MIMO | 6.2D.1 | UE maximum output power for UL MIMO | n/a | Define UE max power for UL MIMO for PC7 under new section 6.2D.1.7 |
|  | 6.2D.2 | UE maximum output power reduction for modulation / channel bandwidth for UL MIMO | n/a | Define UE MPR for UL MIMO for PC7 under new section 6.2D.2.7 |
|  | 6.2D.3 | UE maximum output power reduction with additional requirements for UL MIMO | n/a | Define A-MPR for UL MIMO for PC7 under new section 6.2D.3.7 |
| Min output power | 6.3.1.4 | Minimum output power for power class 7 | Only for Max channel bandwidth 100MHz | Define min out power for 200MHz CBW |
| Min output power for CA | 6.3A.1 | Minimum output power for CA | n/a | Define min out power for PC7 CA under new section 6.3A.1.4 |
| Min output power for UL MIMO | 6.3D.1 | Minimum output power for UL MIMO | n/a | Define min output power for UL MIMO for PC7 under new section 6.3D.1.4 |
| Carrier leakage for CA | 6.4A.2.2 | Carrier leakage | n/a | Define carrier leakage for CA for PC7 under new section 6.4A.2.2.8 |
| In-band emissions for CA | 6.4A.2.3 | Inband emissions | n/a | Define in-band emissions for CA for PC7 under new section 6.4A.2.3.8 |
| Reference sensitivity | 7.3.2.7 | Reference sensitivity power level for power class 7 | Only for Max channel bandwidth 100MHz | Define REFSENS for 200MHz CBW |
| EIS spherical coverage | 7.3.4.7 | EIS spherical coverage for power class 7 | Only for Max channel bandwidth 100MHz | Define EIS for 200MHz CBW |

For the detailed updates to the specification to extend PC7 to be applicable for non-RedCap UE, the corresponding CR is submitted for agreement.

Proposal 1: Extend PC7 to be applicable for non-RedCap UE.

Proposal 2: Agree the updates to requirements for PC7 in CR R4-2408998.

Proposal 3: Send LS to RAN2 to remove the limitation of PC5 only applicable to RedCap UE in FR2.

- Conclusion

As per our analysis above, our observations and proposals are summarized as following:

Observation 1: The UE power class definition for FR2 is based on certain assumed UE device architecture, the applicable UE types are not limited to those listed UE types in Table 6.2.1.0-1 of TS 38.101-2 which are only example UE types.

Observation 2: PC1 ~ PC6 are also applicable for RedCap UE.

Observation 3: The existing transmitter and receiver requirements defined for PC7 are only applicable for RedCap UE.

Observation 4: Extension PC7 to non-RedCap UE has no impact to RedCap UE. RedCap UE can decide to pass which transmitter and receiver requirements of PC7 by reporting UE capabilities of supportOfRedCap-r17, the supported UL and DL bandwidth and not report CA and UL-MIMO capabilities.

Proposal 1: Extend PC7 to be applicable for non-RedCap UE.

Proposal 2: Agree the updates to requirements for PC7 in CR R4-2408998.

Proposal 3: Send LS to RAN2 to remove the limitation of PC5 only applicable to RedCap UE in FR2.

- Reference

[1] R4-2206544 WF on the RedCap RF, RAN4#102-e, Ericsson

[2] R4-2206545 LS on FR2 RedCap UE, RAN4#102-e, Ericsson