3GPP TSG-RAN WG4 Meeting # 111	R4-2409017

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title: 	Discussions on UE performance requirements for FR1 spectrum less than 5MHz

Source: 	Huawei, HiSilicon

Agenda item:	7.8.7.1

Document for:	Discussion

Background

In last meeting, a WF [1] on FR1 less than 5MHz bandwidth demod was agreed. This contribution provides our views on UE performance requirements.

Discussions

Based on the agreement, CCE#4,#5,#6 and #7 are occupied and second half of CCE#7 is punctured at transmitting side. There is large span between companies’ results, which may come from different implementation on CCE#7.

We provide the simulation results for both algorithms in Table 2-1:

- Algorithm 1: UE decodes PDCCH as usual with no additional processing for CCE#7.

- Algorithm 2: For CCE#7, frequency channel interpolation is performed only for non-punctured REs and LLR is set to zero for punctured REs.

Table 2-1: Simulation results for punctured PDCCH

| Algorithm | Number of Tx | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| Algorithm 1 | 1 | 3 | 15 | 3 | 4 | TDLA30-10 | 1x2 Low | 1 | 3.7 |
| Algorithm 2 | 1 | 3 | 15 | 3 | 4 | TDLA30-10 | 1x2 Low | 1 | 3.4 |
| Algorithm 1 | 2 | 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | 1 | -4.5 |
| Algorithm 2 | 2 | 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | 1 | -4.7 |

We can observe that the performance gap is quite limited, the reason is that the punctured REs is quite few compared to total number of PDCCH REs. Hence, we suggest to leave the algorithms to UE implementation.

Proposal 1: Not to specify the algorithms for punctured CCE and leave it to UE implementation.

Conclusion

- In this contribution we provide our simulation results for FR1 spectrum less than 5MHz. The proposal is:

Proposal 1: Not to specify the algorithms for punctured CCE and leave it to UE implementation.

Reference

[1]    R4-2406044  WF on NR_FR1_lessthan_5MHz_BW_demod. Nokia.  Nokia Shanghai Bell