3GPP TSG-RAN WG4 Meeting # 109	R4-2409018

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Title: 	Discussions on BS performance requirements for FR1 spectrum less than 5MHz

Source: 	Huawei, HiSilicon

Agenda item:	7.8.7.2

Document for:	Discussion

Background

In last meeting, a WF [1] on FR1 less than 5MHz bandwidth demod was agreed. This contribution provides our simulation results for PF2 with 12PRB allocation.

Simulation results

In last meeting, RAN4 agreed to define PF2 requirements with 12PRB allocation with 2Rx, 4Rx and 8Rx.

Issue 2-1-1: Antenna configuration for PUCCH requirements Agreement: - Introduce conducted requirements with 2Rx, 4Rx, and 8Rx. Way forward: - Companies are encouraged to provide simulations results for 3MHz CBW with 4Rx and 8Rx. Issue 2-1-2: Definition and applicability rules for PUCCH requirement Agreement: - Define a single set of requirements based on 12 PRBs FH BW. - Frist PRB after frequency hopping is 3 for 3MHz CBW and “The largest PRB index – (Number of PRBs – 1)” otherwise. - For a BS declaring to support less than 5MHz (see D.14 in table 4.6-1) a UCI BLER performance test with Format 2 PUCCH shall be tested in 3 MHz. The UCI BLER performance tests on 5MHz PUCCH Format 2 to be skipped if applicable.

Table 2-1 summarizes the simulation results.

Table 2-1: Simulation results for PF2 with intra-slot frequency hopping enabled

|  | Ideal results (dB) | Impairment results(dB) |
| --- | --- | --- |
| 1T2R | -1.16 | 0.34 |
| 1T4R | -5.85 | -4.35 |
| 1T8R | -9.53 | -8.03 |

Conclusion

- In this contribution we provide our simulation results for PF2 with 12PRB allocation

Reference

[1]    R4-2406044  WF on NR_FR1_lessthan_5MHz_BW_demod. Nokia.  Nokia Shanghai Bell