# 3GPP TSG-RAN WG4 Meeting # 111                                   R4-2409090

# Fukuoka, Japan, 20 May – 24 May, 2024

# Title: 	Simulation results for BS demodulation performance for less than 5MHz

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.8.7.2

# Document for:	Discussion

# 1 Introduction

This contribution contains initial simulation results for the <5MHz WID [1]. The results are provided to aid in determining where new requirements can/should be introduced.

# 2 Discussion

## 2.1 PUCCH requirements

Based on the previous meeting’s WF [2], we evaluated the PUCCH format 2 demodulation performance under the following parameters.

Table 1. PUCCH format 2 simulation assumptions

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 2. PUCCH format 2 simulation results for 12PRB

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex G) | SNR [dB] | Impairment SNR [dB] |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 12PRB | 12PRB |
| 1 | 2 | Normal | TDLC-300-100 Low | -0.4 | 2.1 |
| 1 | 4 | Normal | TDLC-300-100 Low | -5.1 | -2.6 |
| 1 | 8 | Normal | TDLC-300-100 Low | -8.7 | -6.2 |

Figure 1.  PUCCH format 2 1T2R TDLC300-100 Low

# 3 Conclusion

In this contribution, we give some simulation results on demodulation performance requirements for less than 5MHz BS demodulation requirements.

# 4 References

- RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, RAN#95e.

- R4-2406044, WF on [110bis][323] NR_FR1_lessthan_5MHz_BW_demod. Nokia, Nokia Shanghai Bell.