3GPP TSG-RAN WG4 Meeting #111	R4-2409257

Fukuoka, Japan, 20 – 24 May, 2024

Title: 	Discussion on remaining issues for BWP without restriction

Source: 	Huawei, HiSilicon

Agenda item:	7.6.1

Document for:	Discussion

Introduction

Maintenance of RRM core requirements for BWP without restriction is discussed in RAN4#110-bis, and the outcomes are captured in [1]. Based on [1], further discussions are needed for the following issues.

- Configuration of SSB outside BWP for L1 measurement

In this paper, we will provide our views on remaining issues in RRM core requirements for BWP without restriction.

Discussion

Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1 - FFS following options - Option 1: - LS RAN2 and RAN1 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1. - Option 2a: - RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1. - Option 2b: - Configuration of SSB resources outside active BWP is allowed within CSI-ResourceConfig settings for UEs supporting option B-1-1 - No need to send an LS to RAN1/RAN2. - Option 2c: - RAN4 should further clarify what is the ambiguity part of the other WG spec before sending an LS requesting for further details. - Option 3: - Update FG53-1 related clarification in L1 RSRP measurement requirements.

SSB for L1 measurement is configured via CSI-ResourceConfig, and the configuration includes an IE bwp-Id. Assuming this bwp-Id points to BWP-1, the question is whether SSB outside BWP-1 can be used for L1 measurement when BWP-1 is the active BWP.

In our view, the answer should be “yes” for UE supporting option B-1-1. Otherwise, we do not see how NW could configure UE to measure SSB outside active BWP for L1, and how option B-1-1 would work. In fact, in the field description for IE bwp-Id, only CSI-RS is limited to the pointed BWP.

If this is common understanding in RAN4, we believe there is no need to send LS to RAN1/2 as so far there is no restriction that the SSB is limited to the pointed BWP in RAN1 or RAN2 spec. Otherwise, we can send LS to confirm with RAN1/2.

Proposal 1: RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1.

Conclusions

In this paper we provided our views on remaining issues in RRM core requirements for BWP without restriction.

Proposal 1: RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1.

Reference

- R4-2406364, WF on core maintenance and performance part for BWP without restriction, vivo, Vodafone