# 3GPP TSG-RAN WG4 Meeting # 111	R4-2409320

# Fukuoka, Japan, May 20 – May 24, 2024

Source: 	Huawei, HiSilicon

Title: 	Clarifications on general definition for ATG UE

Agenda Item:	5.2.6.1

Document for:	Approval

Introduction

In the RAN4#110bis meeting, one draft CR [1] was provided to clarify the applicability of the requirements. Since some of Tx requirements are not applicable by defining them as the sum of measurement of all antenna connectors, the following changes are proposed.

## 6.1J General Unless otherwise stated, the transmitter characteristics are specified at the antenna connector(s) of the ATG UE with one or multiple omni-directional antenna(s) or at the transceiver array boundary (TAB) connectors of the ATG UE with the antenna array. The definition about transceiver array boundary (TAB) is specified in clause 4.3.2 of TS 38.104 [16].

However, it’s unclear how to measure the Tx requirements if WG just simply remove these clarifications. In my understanding, RAN4 need to identify which requirements shall be measured as the sum of all antenna connectors and which requirements shall be measured per antenna connector. In this paper, we’d like to share our views on this issue.

Discussion

Before starting to discuss this issue, we just went through the method of Tx requirements definition for UE supporting UL MIMO in the table 1.

Table 1 Tx requirements definition for UE supporting UL MIMO

| Tx requirements for UL MIMO | The definition for Tx requirements (Defined as the sum of all antenna connectors or at each antenna connector) |
| --- | --- |
| 6.2D Transmitter power for UL MIMO |  |
| 6.2D.1 UE maximum output power for UL MIMO | Defined as the sum of all antenna connectors |
| 6.2D.2 UE maximum output power reduction for UL MIMO | Defined as the sum of all antenna connectors |
| 6.2D.4 Configured transmitted power for UL MIMO | For UE supporting UL MIMO, the transmitted power is configured per each UE. |
| 6.3D Output power dynamics for UL MIMO |  |
| 6.3D.1 Minimum output power for UL MIMO | Defined as the sum of all antenna connectors |
| 6.3D.2 Transmit OFF power for UL MIMO | Defined at each antenna connector |
| 6.3D.3 Transmit ON/OFF time mask for UL MIMO | Defined at each antenna connector |
| 6.3D.4 Power control for UL MIMO | Defined as the sum of all antenna connectors |
| 6.4D Transmit signal quality for UL MIMO |  |
| 6.4D.0 General | defined per layer or as the sum of emissions from both antennas to account for the UL MIMO scheme |
| 6.4D.1 Frequency error for UL MIMO | per layer |
| 6.4D.2 Transmit modulation quality for UL MIMO |  |
| 6.4D.2.0 General | For UE supporting UL MIMO, the transmit modulation quality requirements are specified based on measurements made at each transmit antenna connector. |
| 6.4D.2.1 Error Vector Magnitude | per layer |
| 6.4D.2.2 Carrier leakage | per layer |
| 6.4D.2.3 In-band emissions | apply at each transmit antenna connector |
| 6.4D.2.4 EVM equalizer spectrum flatness for UL MIMO | per layer |
| 6.5D Output RF spectrum emissions for UL MIMO |  |
| 6.5D.1 Occupied bandwidth for UL MIMO | Defined as the sum of all antenna connectors |
| 6.5D.2 Out of band emission for UL MIMO | Defined as the sum of all antenna connectors |
| 6.5D.3 Spurious emission for UL MIMO | Defined as the sum of all antenna connectors |
| 6.5D.4 Transmit intermodulation for UL MIMO | Defined as the sum of all antenna connectors |

Similarly, the methods for UE supporting UL MIMO could be considered for ATG UE. Thus, we propose the following methods to define Tx requirements for ATG UE.

Table 2 Tx requirements definition for ATG UE

| Tx requirements for ATG UE | The definition for Tx requirements (Defined as the sum of all antenna connectors or at each antenna connector) |
| --- | --- |
| 6.2J Transmitter power for ATG |  |
| 6.2J.1 UE maximum output power for ATG | Defined as the sum of all antenna connectors |
| 6.2J.2 Configured transmitted power for ATG | Defined as the sum of all antenna connectors |
| 6.3J Output power dynamics for ATG |  |
| 6.3J.1 Minimum output power for ATG | Defined as the sum of all antenna connectors |
| 6.3J.2 Transmit OFF power for ATG | Defined at each antenna connector |
| 6.3J.3 Transmit ON/OFF time mask for ATG | Defined at each antenna connector |
| 6.3J.4 Power control for ATG | Defined as the sum of all antenna connectors |
| 6.4J Transmit signal quality for ATG |  |
| 6.4J.1 Frequency error for ATG | Defined at each antenna connector |
| 6.4J.2 Transmit modulation quality for ATG | TBD |
| 6.5J Output RF spectrum emissions for ATG |  |
| 6.5J.1 Occupied bandwidth for ATG | Defined as the sum of all antenna connectors |
| 6.5J.2 Out of band emission for ATG | Defined as the sum of all antenna connectors |
| 6.5J.3 Spurious emissions for ATG | Defined as the sum of all antenna connectors |

Proposal 1: RAN4 can discuss whether the following definitions of Tx requirements for ATG in table 2 are applicable or not.

Summary

Proposal 1: RAN4 can discuss whether the following definitions of Tx requirements for ATG in table 2 are applicable or not.

| Tx requirements for ATG UE | The definition for Tx requirements (Defined as the sum of all antenna connectors or at each antenna connector) |
| --- | --- |
| 6.2J Transmitter power for ATG |  |
| 6.2J.1 UE maximum output power for ATG | Defined as the sum of all antenna connectors |
| 6.2J.2 Configured transmitted power for ATG | Defined as the sum of all antenna connectors |
| 6.3J Output power dynamics for ATG |  |
| 6.3J.1 Minimum output power for ATG | Defined as the sum of all antenna connectors |
| 6.3J.2 Transmit OFF power for ATG | Defined at each antenna connector |
| 6.3J.3 Transmit ON/OFF time mask for ATG | Defined at each antenna connector |
| 6.3J.4 Power control for ATG | Defined as the sum of all antenna connectors |
| 6.4J Transmit signal quality for ATG |  |
| 6.4J.1 Frequency error for ATG | Defined at each antenna connector |
| 6.4J.2 Transmit modulation quality for ATG | TBD |
| 6.5J Output RF spectrum emissions for ATG |  |
| 6.5J.1 Occupied bandwidth for ATG | Defined as the sum of all antenna connectors |
| 6.5J.2 Out of band emission for ATG | Defined as the sum of all antenna connectors |
| 6.5J.3 Spurious emissions for ATG | Defined as the sum of all antenna connectors |

- References

[1] R4-2406592	(NR_ATG-Core) CR for 38101-1 to update ATG related requirement, Apple