3GPP TSG-RAN WG4 Meeting #111             	R4-2409478

Fukuoka, Japan, 20th-24th April, 2024

Agenda item:	7.8.7.2

Source:	Samsung

Title:	Discussion and simulation results for BS demodulation requirements for less than 5MHz

Document for:     Discussion

- Introduction

In the last RAN4 meeting, the test setup for performance part of NR support for dedicated spectrum less than 5MHz for FR1 was under discussion. The related agreement was captured in the WF [1] as following

- Antenna configuration for PUCCH requirements - Introduce conducted requirements with 2Rx, 4Rx, and 8Rx. - Definition and applicability rules for PUCCH requirements - Define a single set of requirements based on 12 PRBs FH BW. - Frist PRB after frequency hopping is 3 for 3MHz CBW and “The largest PRB index – (Number of PRBs – 1)” otherwise. - For a BS declaring to support less than 5MHz (see D.14 in table 4.6-1) a UCI BLER performance test with Format 2 PUCCH shall be tested in 3 MHz. The UCI BLER performance tests on 5MHz PUCCH Format 2 to be skipped if applicable.

In this contribution, the remaining issue of PUCCH formats is provided, and the ideal and impairment results for PUCCH format 2 are provided for requirement derivation.

- Discussion

AWGN power level setting

Regarding the AWGN power level setting, since new channel bandwidth was introduced, the related AWGN power level should be set to facilitate the BS conformance test including conducted and OTA testing as following

Table 8.2.1.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
|  | 5 | -86.5 dBm / 4.5MHz |
| 15 kHz | 10 | -83.3 dBm / 9.36MHz |
|  | 20 | -80.2 dBm / 19.08MHz |
|  | 10 | -83.6 dBm / 8.64MHz |
| 30 kHz | 20 | -80.4 dBm / 18.36MHz |
|  | 40 | -77.2 dBm / 38.16MHz |
|  | 100 | -73.1 dBm / 98.28MHz |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

Table 8.2.1.4.2-2: AWGN power level at the BS input

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O (Note 4) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -80.2 - ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -80.4 - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -73.1 - ΔOTAREFSENS dBm / 98.28 MHz |

Regarding the 3MHz, based on the WID, both 12 PRB and 15 PRB can be supported for different bands. As agreed, only one set requirement is defined based on fixed frequency hopping pattern regardless the maximum number of PRB.  According to the RF core spec, the maximum number of RB is 15 for 3MHz, then, effective bandwidth should be 2.7MHz.  For the maximum of PPB is 12 for 3MHz, the effective bandwidth will be 2.16MHz. Then, the AWGN power level will be different between 12 PRB and 15 PRB. Considering the AWGN level can be reduced from the default by any value in the range of 0dB to 16dB for test purposed as indicated in the note, then we prefer to apply the maximum number of PRB for the AWGN power level setting for BS conformance test as following

Proposal 1: Appling the following AWGN power level setting for less than 3MHz BS conducted testing

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
|  | 3 | -88.7dBm / 2.7MHz |
| 15 kHz | 5 | -86.5 dBm / 4.5MHz |
|  | 10 | -83.3 dBm / 9.36MHz |
|  | 20 | -80.2 dBm / 19.08MHz |
|  | 10 | -83.6 dBm / 8.64MHz |
| 30 kHz | 20 | -80.4 dBm / 18.36MHz |
|  | 40 | -77.2 dBm / 38.16MHz |
|  | 100 | -73.1 dBm / 98.28MHz |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

And the following AWGN power level setting for less than 3MHz BS OTA testing

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O (Note 4) | 15 | 3 | -88.7 - ΔOTAREFSENS dBm / 2.7 MHz |
|  |  | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -80.2 - ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -80.4 - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -73.1 - ΔOTAREFSENS dBm / 98.28 MHz |

- Simulation results

For this part, the ideal and impairment simulation results for PUCCH format 2 based on the agreed simulation assumption in the last meeting as following table

Table 1:  Simulation assumption for PUCCH format 2

| Parameters | Value |
| --- | --- |
| Modulation order | QPSK |
| First RB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| Frist symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 2: ideal and impairment simulation results for PUCCH format 2

| Test case | Format | SCS&BW | Maximum RB | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of symbols | BLER@1% (ideal) | BLER@1% (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | Format 2 | 15KHz, 3MHz | 12 | Enabled | 22 | TDLC300-100 | 9 | 1T2R | 2 | 0.23 | 2.23 |
| Case 2 | Format 2 | 15KHz, 3MHz | 12 | Enabled | 22 | TDLC300-100 | 9 | 1T4R | 2 | -4.38 | -2.38 |
| Case 3 | Format 2 | 15KHz, 3MHz | 12 | Enabled | 22 | TDLC300-100 | 9 | 1T8R | 2 | -7.97 | -5.97 |

- Conclusion

In this paper, the view on remaining issue of PUCCH with 3MHz is provided, meanwhile, the ideal and impairment simulation results are provided for requirement derivation

Proposal 1: Appling the following AWGN power level setting for less than 3MHz BS conducted testing

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
|  | 3 | -88.7dBm / 2.7MHz |
| 15 kHz | 5 | -86.5 dBm / 4.5MHz |
|  | 10 | -83.3 dBm / 9.36MHz |
|  | 20 | -80.2 dBm / 19.08MHz |
|  | 10 | -83.6 dBm / 8.64MHz |
| 30 kHz | 20 | -80.4 dBm / 18.36MHz |
|  | 40 | -77.2 dBm / 38.16MHz |
|  | 100 | -73.1 dBm / 98.28MHz |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

and the following AWGN power level setting for less than 3MHz BS OTA testing

| BS type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O (Note 4) | 15 | 3 | -88.7 - ΔOTAREFSENS dBm / 2.7 MHz |
|  |  | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  |  | 10 | -83.3 - ΔOTAREFSENS dBm / 9.36 MHz |
|  |  | 20 | -80.2 - ΔOTAREFSENS dBm / 19.08 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
|  |  | 20 | -80.4 - ΔOTAREFSENS dBm / 18.36 MHz |
|  |  | 40 | -77.2 - ΔOTAREFSENS dBm / 38.16 MHz |
|  |  | 100 | -73.1 - ΔOTAREFSENS dBm / 98.28 MHz |

# References

R4-2406044, WF on NR_FR1_lessthan_5MHz_BW_demod