3GPP TSG-RAN WG4 Meeting#111                              R4-2409595

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	10.13.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Co-existence evaluations

Document for:	Approval

Introduction

In the RAN#102 meeting, Study on solutions for Ambient IoT (Internet of Things) in NR has been approved and it’s expected to start the normative work from this April meeting. In the last RAN4#110bis meeting, we have reached some initial consensus on the coexistence evaluations, however there are still some remaining issues. In this contribution, we would like to share some further views on these remaining issues.

- Topic 2-5: Evaluation cases Issue 2-5-1: device type Agreement: - Prioritize device 1 and 2a without a frequency shifter for coexistence evaluation. Issue 2-5-2: Evaluation cases for D1T1 for device 1 and 2a between NR and AIOT Agreement: - FFS on evaluation cases for D1T1 for device 1 and 2a between NR and AIOT. Corresponding evaluation cases are listed for information. Note that some duplicated cases are omitted in the table. Deployment scenario and topology; spectrum; aggressor; victim. c · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum; R2D: DL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; device and/or reader. reader; NR DL. NR DL; device. R2D: DL CW2D and D2R: DL; CW and/or device; NR DL. NR DL; device and/or reader. R2D: UL CW2D and D2R: UL; Reader; NR UL. NR UL; reader. Self interference cancelation is needed for reader · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum; R2D: DL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; device and/or reader. reader; NR DL. NR DL; device. R2D: DL CW2D and D2R: DL; CW and/or device; NR DL. NR DL; device and/or reader. R2D: UL CW2D and D2R: UL; reader; NR UL. NR UL; reader. Self interference cancellation is needed for reader · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum; R2D: DL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; device and/or reader. reader; NR DL. NR DL; device. R2D: UL CW2D and D2R: UL; reader; NR UL. NR UL; reader. Issue 2-5-3: Evaluation cases for D2T2 for device 1 and 2a between NR and AIOT Agreement: - FFS on evaluation cases for D2T2 for device 1 and 2a between NR and AIOT. Corresponding evaluation cases are listed for information. Note that some duplicated cases are omitted in the table. Deployment scenario and topology; spectrum; aggressor; victim. Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum; R2D: UL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; Device and/or reader. reader; NR UL. NR UL; device. \ Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum Self interference cancelation is needed for reader; R2D: UL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; Device and/or reader. reader; NR UL. NR UL; device. Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Self interference cancelation is needed for reader; R2D: UL CW2D and D2R: UL; CW and/or device; NR UL. NR UL; Device and/or reader. reader; NR UL. NR UL; device. R2D: UL CW2D and D2R: DL; CW and/or device; NR DL. NR DL; device and/or reader. Issue 2-5-4: Evaluation cases for device 2b between AIOT and NR Agreement: - FFS for evaluation cases for device 2b Issue 2-5-5: Interference between AIOT systems Agreement: - RAN4 to first evaluate interference between AIOT and NR. - FFS on interference between AIOT and AIOT - Topic 2-6: Evaluation parameters There are no agreements on detailed parameters in RAN4#110bis meeting. Deatiled parameters for information can be found in summary document R4-2405289 and will be further discussed in next meeting. Issue 2-6-2 and 2-6-3: Layout for D1T1 and D2T2 Agreement: Use RAN1 agreements in RAN1#116bis meeting as baseline (copied as below). - FFS on whether any updates are needed for RAN4 co-existence evaluation. - FFS on other parameters. RAN1 agreements: For D1T1, - InF-DH NLOS model defined in TR38.901 is used for D2R and R2D links as pathloss model in coverage evaluation. For D2T2, - InF-DL and InH-Office model defined in TR38.901is used as pathloss model in coverage evaluation, - NLOS for D2R and R2D links if InF-DL is used - LOS for D2R and R2D links if InH-Office is used The following layout is used for evaluation purpose, - FFS: CW distribution for D1T1-B and D2T2-B Parameter; Assumptions for D1T1; Assumptions for D2T2. Scenario; InF-DH; InH-office; InF-DL. Hall size; 120x60 m; 120 x50 m; 300x150 m. Room height; 10 m; 3m; 10 m. Sectorization; None. BS deployment / Intermediate UE dropping; 18 BSs on a square lattice with spacing D, located D/2 from the walls. - L=120m x W=60m; D=20m - BS height = 8 m; - L=120m x W=50m; - Intermediate UE height = 1.5 m FFS: Intermediate UE dropping; - L=300m x W=150m; - Intermediate UE height = 1.5 m FFS: Intermediate UE dropping. Device distribution; Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area; Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations; Device Height= 1.5m AIoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations. Device mobility (horizontal plane only); 3 kph; 3 kph; 3 kph.

Discussion

- 2.1. Discussion on coexistence cases and simulation assumptions

In the following section, we provide some initial views on the coexistence cases and simulation assumptions for A-IoT coexistence study.

From our understanding, at least the following coexistence cases should be taken into account and further discuss downselection if some tightening cases among all could be identified.

- Table 1. Evaluation cases for A-IoT coexistence study for topology 1

| Topology | Case | Aggressor | Victim | Mode | Inventory command | CW signal/Backscattering signal | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1-2 | 1 | Legacy TN DL | A-IoT DL | In-band/guard band | DL | - UL | To check ACS requirement of A-IoT device for OOK signal decoding |
| Case 1-2 | 2 | A-IoT DL | Legacy TN DL | In-band/guard band | DL | - UL | To check the ACLR requirement of A-IoT BS DL |
| Case 1-2 | 3 | Legacy TN UL | A-IoT UL | In-band/guard band | DL | - UL | To check the ACS requirement of A-IoT BS DL |
| Case 1-2 | 4 | A-IoT UL | Legacy TN UL | In-band/guard band | DL | - UL | To check the impacts of A-IoT CW signal and backscattering signal on legacy TN UL |
| Case 1-2 | 5 | Legacy TN UL | A-IoT Device | In-band/guard band | UL | - UL | To check ACS requirement of A-IoT device for OOK signal decoding |
| Case 1-2 | 6 | Legacy TN UL | A-IoT BS | In-band/guard band | UL | - UL | To check ACS requirement of A-IoT BS for backscattering signal detection |
| Case 1-2 | 7 | A-IoT UL | Legacy TN UL | In-band/guard band | UL | - UL | To check the impacts of inventory command, CW and backscattering signal on legacy BS receiver |
| Case 1-4 | 8 | Legacy TN DL | A-IoT DL | In-band/guard band | DL | - UL | To check ACS requirement of A-IoT device for OOK signal decoding |
| Case 1-4 | 9 | A-IoT DL | Legacy TN DL | In-band/guard band | DL | - UL | To check the ACLR requirement of A-IoT BS DL |
| Case 1-4 | 10 | Legacy TN UL | A-IoT UL | In-band/guard band | DL | - UL | To check the ACS requirement of A-IoT BS DL |
| Case 1-4 | 11 | A-IoT UL | Legacy TN UL | In-band/guard band | DL | - UL | To check the impacts of A-IoT CW signal and backscattering signal on legacy TN UL |
| Case 1-4 | 12 | Legacy TN UL | A-IoT Device | In-band/guard band | UL | - UL | To check ACS requirement of A-IoT device for OOK signal decoding |
| Case 1-4 | 13 | Legacy TN UL | A-IoT BS | In-band/guard band | UL | - UL | To check ACS requirement of A-IoT BS for backscattering signal detection |
| Case 1-4 | 14 | A-IoT UL | Legacy TN UL | In-band/guard band | UL | - UL | To check the impacts of inventory command, CW and backscattering signal on legacy BS receiver |
| Case 1-1 | 15 | Legacy TN DL | A-IoT Device | In-band/guard band | DL | - DL | To check ACS requirement of A-IoT device for OOK signal decoding |
| Case 1-1 | 16 | Legacy TN DL | A-IoT BS | In-band/guard band | DL | - DL | To check ACS requirement of A-IoT BS. |
| Case 1-1 | 17 | A-IoT DL | Legacy TN DL | In-band/guard band | DL | - DL | To check the impacts of A-IoT DL towards TN DL. E.g. A-IoT BS ACLR and A-IoT UE ACLR requirements |
| - NOTE 1: The threshold for CW signal reception is to be checked. - NOTE 2: The threshold of OOK decoding is to be checked. - NOTE 3: The standalone operation could be considered as well. |  |  |  |  |  |  |  |

- Table 2. Evaluation cases for A-IoT coexistence study for topology 2

| Topology | Case | Aggressor | Victim | Mode | Inventory command | CW signal/Backscattering signal | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2-2 | 1 | Legacy TN UL | A-IoT device | In-band/guard band | UL | - UL | To check ACS requirement of A-IoT device for OOK signal decoding |
| Case 2-2 | 2 | Legacy TN UL | Intermediate UE | In-band/guard band | UL | - UL | To check the ACS requirement of intermediate UE |
| Case 2-2 | 3 | A-IoT UL | Legacy TN UL | In-band/guard band | UL | - UL | To check the intermediate UE ACLR, A-IoT Device ACLR, CW impacts of legacy BS receiver. |
| Case 2-4 | 4 | Legacy TN UL | A-IoT device | In-band/guard band | UL | - UL | To check ACS requirement of A-IoT device for OOK signal decoding |
| Case 2-4 | 5 | Legacy TN UL | Intermediate UE | In-band/guard band | UL | - UL | To check the ACS requirement of intermediate UE |
| Case 2-4 | 6 | A-IoT UL | Legacy TN UL | In-band/guard band | UL | - UL | To check the intermediate UE ACLR, A-IoT Device ACLR, CW impacts of legacy BS receiver. |
| Case 2-3 | 7 | Legacy TN UL | A-IoT device | In-band/guard band | UL | - DL | To check ACS requirement of A-IoT device for OOK signal decoding |
| Case 2-3 | 8 | Legacy TN DL | Intermediate UE | In-band/guard band | UL | - DL | To check ACS requirement of intermediate UE |
| Case 2-3 | 9 | A-IoT UL | Legacy TN UL | In-band/guard band | UL | - DL | To check ACLR requirement of f intermediate UE |
| Case 2-3 | 10 | A-IoT DL | Legacy TN DL | In-band/guard band | UL | - DL | To check ACLR requirement of A-IoT Device and the impacts of CW signal on the legacy UE. |
| - NOTE 1: The threshold for CW signal reception is to be checked. - NOTE 2: The threshold of OOK decoding is to be checked. - NOTE 3: The standalone operation could be considered as well. |  |  |  |  |  |  |  |

For the simulation assumptions for A-IoT system and NR/NB system could be found in the Table 3 and 4.

Table 3. the simulation assumption for A-IoT system

|  | Ambient-IoT system | NOTE |
| --- | --- | --- |
| Carrier frequency in GHz | 900MHz |  |
| Size of each nominal channel BW in MHz | 200kHz as starting point |  |
| Transmission bandwidth in MHz | 180kHz as starting point | DL OFDM with OOK1 or OOK4 [DFT-s-OFDM] |
| Environment | Topology 1 or 2 |  |
| Network layout | 19-sites [57 sectors] with wrap-around |  |
| Inter-site distance in meter | For Topology 1, ISD coverage 10-50m For Topology 2, ISD as 500m | Refer to 38.901 InF-SH: 300x150 m InF-DH: 120x60 m Only foucs on reader and tag is placed indoor |
| System loading and activity | Full buffer 100% |  |
| Network location | TBD |  |
| DL subcarrier spacing | 15kHz | Data or signalling |
| UL subcarrier spacing | TBD | Single tone (CW) or multiple tone. Without any subcarrier spacing concept. Under the discussion in RAN1. |
| DL power control | No |  |
| UL power control | No | This might be possible, not high priority Devie 2B might have the power control |
| Frequency reuse | [1] as worst case | FFS for other frequency reuse factor Note that RFID has considered sub-channel different from FRF=1 |
| Number of scheduled UE per cell (DL) | 1 |  |
| Number of scheduled UE per cell (UL) | TBD | Depend on the multi-access mechanism This is also under the discussion in RAN1 and also dependent on the backscattering waveform/mutiplexing factor in the uplink direction. |
| Device antenna height in meter | 1.5 |  |
| Device TX power in dBm | TBD as fixed value proposed by RAN1 | This also depends on the power efficiency of backscattering. |
| Device antenna gain in dBi | 9dBi |  |
| Building penetration loss | TR 45.820[9] Annex D.1 |  |
| Cell selection margin in dB | NA |  |
| BS-MS min distance in meters | 35 |  |
| BS noise figure in dB | TBD | 10/13dB as starting point |
| A-IoT Device noise figure in dB | [-45dBm] |  |
| BS-UE path-loss model | TR 38.901 |  |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 |  |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 |  |
| Link-level performance model | To be further checked since modulation order has changed | BLER vs SNR for OOK/OOK4 signal could be further checked. |
| UE distribution | Random distribution |  |
| Evaluation metrics | SINR vs ACS (as victim) |  |

Table 4. the simulation assumption for Tag or device

|  | NB-IoT standalone | NR [in-band/guard band] Adjacent carrier coexistence |
| --- | --- | --- |
| Carrier frequency in GHz | 0.9 | 0.9 |
| Size of each nominal channel BW in MHz | 0.2 | 20 |
| Transmission bandwidth in MHz | 0.18 | N/A |
| Environment | Urban macro Rural | Deployment scenario related, see Table 6.2.2.4-2. |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA TBD For Rural | Deployment scenario related, see Table 6.2.2.4-2 |
| System loading and activity | Full buffer 100% | See Table 6.2.1.1-1 |

Table 5. the simulation assumption for NR and standalone NB

|  | NB-IoT standalone | NR [in-band/guard band] Adjacent carrier coexistence |
| --- | --- | --- |
| Carrier frequency in GHz | 0.9 | 0.9 |
| Size of each nominal channel BW in MHz | 0.2 | 20 |
| Transmission bandwidth in MHz | 0.18 | N/A |
| Environment | Urban macro Rural | Deployment scenario related, see Table 6.2.2.4-2. |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA TBD For Rural | Deployment scenario related, see Table 6.2.2.4-2 |
| System loading and activity | Full buffer 100% | See Table 6.2.1.1-1 |
| Network location | TBD | See Table 6.2.1.1-1 |
| DL subcarrier spacing | 15kHz | 15kHz |
| UL | See RP-152284[7] | OFDMA |
| DL power control | No | No |
| UL power control | TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | TR 36.942[8] |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | 3 |
| UE antenna height in meter | 1.5 | 1.5m |
| UE TX power in dBm | -40 to 23 | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | TR 45.820[9] Annex D.1 | In pathloss model, TR 38.901[10] |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min distance in meters | 35 | 35 |
| BS noise figure in dB | 5 | 5 |
| UE noise figure in dB | 9 | 9 |
| BS-UE path-loss model | TR 36.942[8] macro urban | TR 38.901[10] |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | Deployment scenario related, referring to TR 38.901[10] |
| Shadowing correlation | Inter-cell 0.5 Intra-cell 1 | Inter-cell 0.5 Intra-cell 1 |
| Link-level performance model |  | See Section 6.2.8 Throughtput-SINR mapping |
| UE distribution |  | Uniform |
| Evaluation metrics | SINR vs ACS (as victim) | See Section 6.2.8 Throughtput or SNR loss criteria |

Conclusions

In this contribution, we shared some further views on coexistence evaluations and proposals are made as following:

- Proposal 1: to consider the coexistence evaluation assumptions in section 2.1.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.