3GPP TSG-RAN WG4 Meeting#111                              R4-2409601

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	10.8.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	RF requirement for Intra-band contiguous CA of ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we have some initial discussions for ATG UE supporting intra-band CA. In this contribution, we would like to share some further views on these remaining issues.

Common requirements Issue 3-1-1: whether maximum output power is declared per single carrier or per band(aggregated across all CCs) for inter-band CA and intra-band contiguous CA Agreement: - Reuse the existing Rel-18 capability for MOP for ATG UE in Rel-19 Issue 3-1-2: Tx RF requirements for intra-band contigous CA with one UL carrier and inter-band CA with one UL carrier. Agreement: - Reuse Tx RF requirements for ATG UE single CC operation intra-band contiguous CA Issue 3-2-1: more clarification for contiguous CA Agreement: - Only consider contiguous DL CA case for ATG UE intra-band CA. - The example band combination for intra-band contiguous CA is CA_n79C Issue 3-2-2: BCS for DL CA_n79C - Proposals - Option 1: BCS4 (Huawei) NR CA configuration / Bandwidth combination set. NR CA configuration; Uplink CA configurations or single uplink carrier5; Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Maximum aggregated bandwidth (MHz); Bandwidth combination set. CA_n79C; -; See n79 channel bandwidths in Table 5.3.5-1 for each carrier2; 200; 4. NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 2: The aggregated bandwidth must be greater than or equal to the minimum for the bandwidth class defined in Table 5.3A.5-1, and smaller than or equal to the maximum aggregated bandwidth. NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 4: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 5: Only single uplink carriers with power class other than PC3 are listed.. - Option 2: (APPLE) Table 2.2-1 BCS for CA_n79C NR CA configuration / Bandwidth combination set. NR CA config; Uplink CA config; Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Maximum aggregated bandwidth (MHz); BCS. CA_n79C; -; 50; 60, 80, 100; 200; 0. 60; 60, 80, 100. 80; 80, 100. 100; 100. Agreement: - FFS on the supported BCS for DL CA_n79C

Discussion

- 2.1. BCS for CA_n79C

Usually for BCS configuration of certain band combination, it’s up to operator’s decision or request. From the practical market perspective, it seems that minimum channel bandwidth to be supported is 40MHz and 10/20/30MHz might be not necessary. In addition, as we know that 70MHz channel bandwidth is somehow optional. if there are no practical market request for 10/20/30MHz and 70MHz channel bandwidth, BCS0 could be used for band combination n79C.

Proposal 1: use BCS0 for band combination n79C if there are no practical market demand for 10/20/30/70MHz channel bandwidth.

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA config | Uplink CA config | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | BCS |
| CA_n79C | - | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |

- 2.2. RF requirements for Intra-band CA

In this section, we provide some initial analysis on the overall requirement for ATG UE supporting intra-band DL CA and also provided the summary of proposals for it.

Table 2. Analysis and summary of proposals for ATG UE supporting intra-band DL CA

| The existing RF requirement for intra-band contiguous CA | ATG UE supporting intra-band contiguous CA |
| --- | --- |
| 6.2A.1.1 UE maximum output power for Intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.2A.2.1 UE maximum output power reduction for Intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.2A.3.1.1 UE additional maximum output power reduction for Intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.2A.4.1.1 Configured transmitted power for Intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.3A.1.1 Minimum output power for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.3A.2.1 Transmit OFF power for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.3A.3.1 Transmit ON/OFF time mask for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.3A.4.1 Power control for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.4A.1.1 Frequency error for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.4A.2.1 Transmit modulation quality for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.5A.1.1a Occupied bandwidth for Intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.5A.2.2.1 Spectrum emission mask for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.5A.2.3.1 Additional spectrum emission mask for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.5A.3.2.1 Spurious emissions for UE co-existence for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.5A.3.3.1 Additional spurious emissions for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 6.5A.4.2.1 Transmit intermodulation for intra-band contiguous CA | No impacts, to reuse Rel-18 ATG UE single carrier requirements |
| 7.3A.2.1 Reference sensitivity power level for Intra-band contiguous CA | Reference sensitivity power level for Intra-band contiguous CA For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3.2-3. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. |
| 7.4A.1 Maximum input level for Intra-band contiguous CA | Maximum input level for Intra-band contiguous CA For intra-band contiguous carrier aggregation maximum input level is defined as the maximum mean power received at the UE antenna port, over the Transmission bandwidth configuration of each CC. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4A.1-1 for each component carrier. Table 7.4A.1-1: Maximum input level for Intra-band contiguous CA Rx Parameter; Units; NR CA Bandwidth Class. B; C; D. Power in largest transmission bandwidth configuration CC, Plargest BW; dBm; [-42]2 for omini antenna [-30] for array antenna; [-42]2 for omini antenna [-30] for array antenna; N/A. [-44]3 for omini antenna [-32] for array antenna; [-44]3 for omini antenna [-32] for array antenna; N/A. Power in each other CC; dBm; Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}. NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM.. |
| 7.5A.1 Adjacent channel selectivity for Intra-band contiguous CA | To following the existing requirement defined in clause 7.5A.1 with the following power to be updated for ATG UE. - Pw in Transmission Bandwidth Configuration, per CC - PInterferer |
| 7.6A.2.1 In-band blocking for Intra-band contiguous CA | To following the existing requirements defined in Clause 7.6A.2.1. |
| 7.6A.3.1 Out-of-band blocking for Intra-band contiguous CA | To following the existing requirements defined in Clause 7.6A.3.1. |
| 7.6A.4.1 Narrow band blocking for Intra-band contiguous CA | N/A |
| 7.7A.1 Spurious response for Intra-band contiguous CA | To following the existing requirements defined in Clause 7.7A.1. |
| 7.8A.2.1 Wide band intermodulation for Intra-band contiguous CA | To following the existing requirements defined in Clause 7.8A.2.1. |

Conclusions

In this contribution, we shared some further views on the RF requirements for ATG UE supporting intra-band DL CA and proposals are made as following:

Proposal 1: use BCS0 for band combination n79C if there are no practical market demand for 10/20/30/70MHz channel bandwidth.

Proposal 2: for RF requirements for inter-band DL CA, refer to the proposals in table 2.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.