# 3GPP TSG-RAN WG4 Meeting #111	R4-2409645

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source: 	Huawei, HiSilicon

Title:	Discussion on increasing UE transmission power limit

Agenda Item:	10.1.1.2.3

Document for:	Approval

# 1	Introduction

A WF [1] was agreed in RAN4#110bis. In the following, we will share our views on the two open issues listed in the WF, i.e. R19 scenarios and MSD impact.

## 4.1 Scenarios to be considered in Rel-19

Way forward: Consider the following options and further discuss it in future meetings

- Option 1: For 2Tx, if based on existing specified combos, there seems no new scenarios to be considered so far. For 3Tx, the following scenarios may could be considered. (Samsung)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band |
| PC1.5 | PC3 | PC1.5 | 30.0 | One CC per band; For FWA only |

- Option 2: the following power class configuration could be considered in Rel 19 for UE increasing high power limit. (Xiaomi)

    - PC3 (TDD/FDD) +PC1.5

    - PC2 (TDD with TxD) +PC3 (TDD/FDD)

    - PC2 (TDD) +PC5

- Option 3: To consider the following eligible inter-band CA/ENDC band combination to enable increase higher power limit feature in Rel-19 (ZTE)

    - For PC2 2Tx inter-band NR CA and ENDC:

      - Inter-band with intra-band UL CA in one of the bands

    - For HPUE 3Tx inter-band NR CA and ENDC:

      - PC2 band combination of PC3+PC2 with single carrier in each band.

      - PC1.5 band combination of PC3+PC1.5 with single carrier in each band.

      - PC1.5 band combination of PC2+PC1.5 with single carrier in each band.

Note: Only PC3 is considered for LTE FDD in EN-DC

- Option 4: For both FWA and handheld UE, at least, the increasing UE power high limit feature should be introduced for 3Tx UL CA/EN-DC configured with PC3 band and PC2 band. Other power class configurations can also be discussed if they are included in the scope of 3Tx in Rel-19 WI. (DOCOMO)

## On MSD impact

Way forward: Consider the following proposals and further discuss it in future meetings

- Proposal 1: Consider impact to the MSD requirements, e.g., whether to define MSD requirements for various combinations of per-band power classes such as PC1.5+PC2/3/5, PC2+PC5, etc. (Huawei)

- Proposal 2: RAN4 shall be mindful on defining MSD requirements for the new UL configurations supporting “Increasing UE power high limit for CA and DC” feature. (Apple)

- Proposal 3: For Rel-19, RAN4 to identify the additional conditions that must also be met before a new MSD test case is justified in context of a new power class aggregation for an already specified ULCA inter-band combination. For example, if an MSD test case exists for a PC1.5 CA power class UE comprising a PC3 UL band and a PC1.5 UL band, does a new case need to be defined when the same band combination is enabled for PC2 + PC1.5 UL? (Qualcomm)

# 2	Discussion

For convenience, the WID objective is duplicated below:

Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified

Observation 1: The WI scope is to study UL bands with different existing power classes. Except that, the WID imposes no restrictions on power class configurations, 2Tx/3Tx, FDD/TDD.

In Rel-18, the higherPowerLimit-r17 feature was excluded from the 3Tx inter-band CA/DC WI. It can be studied now in the Rel-19 WI.

Observation 2: Increasing UE transmission power limit for 3Tx band combinations can be studied in the Rel-19 WI.

Proposal 1: Consider the following list of scenarios for increasing UE transmission power limit as shown in Table 1 below.

Table 1: A list of applicable scenarios for increasing UE transmission power limit

| CA power class | Power class configuration Band A + Band B |  | 2Tx | 3Tx |
| --- | --- | --- | --- | --- |
| PC3 | PC3 | PC5 | Completed in R18 | N/A |
| PC2 | PC2 | PC3 | Completed in R17 | R19 |
|  |  | PC5 | R19 | R19 |
| PC1.5 | PC1.5 | PC2 | N/A | R19 |
|  |  | PC3 | N/A | R19 |
|  |  | PC5 | N/A | R19 |

As pointed out before, 26dBm+29dBm = 30.8dBm, 23dBm+29dBm = 30.0dBm, which are close to PC1 of 31dBm. Generally, PC1 is not targeted for handheld UEs.

Proposal 2: Increasing the total Tx power limit beyond PC1.5 is only for FWA UEs, not for handheld UEs.

Regarding the MSD impact, it seems that no additional MSD requirements for a given band combination are needed as long as the MSD requirements for the CA power class (such as PC2 or PC1.5) have been specified.

Proposal 3 in Section 4.3 of the WF [1] points out an interesting scenario.

- Proposal 3: For Rel-19, RAN4 to identify the additional conditions that must also be met before a new MSD test case is justified in context of a new power class aggregation for an already specified ULCA inter-band combination. For example, if an MSD test case exists for a PC1.5 CA power class UE comprising a PC3 UL band and a PC1.5 UL band, does a new case need to be defined when the same band combination is enabled for PC2 + PC1.5 UL? (Qualcomm)

Strictly speaking, the MSD for IMD caused by PC2+PC1.5 UL should be specified, even if MSD for PC3+PC1.5 has been defined. This is because that the power of the dual-UL may be changed from 27.8+23 to 26+26 for the 2Tx+1Tx configuration. The difference in the MSD requirements could be small if the IMD order is low. Anyway, this is an issue that should be addressed in the agenda item for HPUE for NR inter-band UL CA and EN-DC.

Proposal 3: For increasing UE transmission power limit, no additional MSD requirements for a given band combination are needed as long as the MSD requirements for the CA/DC power class (such as PC2 or PC1.5) have been specified.

# 3	Conclusion

In this paper, we have shared our views on the open issues listed in the WF for increasing UE transmission power limit. The following observations and proposals are made:

Observation 1: The WI scope is to study UL bands with different existing power classes. Except that, the WID imposes no restrictions on power class configurations, 2Tx/3Tx, FDD/TDD.

Observation 2: Increasing UE transmission power limit for 3Tx band combinations can be studied in the Rel-19 WI.

Proposal 1: Consider the following list of scenarios for increasing UE transmission power limit as shown in Table 1 below.

Table 1: A list of applicable scenarios for increasing UE transmission power limit

| CA power class | Power class configuration Band A + Band B |  | 2Tx | 3Tx |
| --- | --- | --- | --- | --- |
| PC3 | PC3 | PC5 | Completed in R18 | N/A |
| PC2 | PC2 | PC3 | Completed in R17 | R19 |
|  |  | PC5 | R19 | R19 |
| PC1.5 | PC1.5 | PC2 | N/A | R19 |
|  |  | PC3 | N/A | R19 |
|  |  | PC5 | N/A | R19 |

Proposal 2: Increasing the total Tx power limit beyond PC1.5 is only for FWA UEs, not for handheld UEs.

Proposal 3: For increasing UE transmission power limit, no additional MSD requirements for a given band combination are needed as long as the MSD requirements for the CA/DC power class (such as PC2 or PC1.5) have been specified.

# References

- R4-2406583 WF on HPUE UE for UL CA and EN-DC, Samsung, RAN4#110bis

- R4-2404885 Initial consideration on R19 UE RF enhancements for HPUE in TN, Huawei, HiSilicon, RAN4#110bis