3GPP TSG-RAN WG4 Meeting #111									                          R4- 2409663

Fukuoka, Japan, May 20 – 24, 2024

Agenda item:			10.8.2.3

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE UL MIMO

Document for:	Discussion

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements were made addressing the general concerns [2]. Part of the agreements are shown below:

  - Common requirements

Issue 3-1-1: whether maximum output power is declared per single carrier or per band(aggregated across all CCs) for inter-band CA and intra-band contiguous CA

Agreement:

- Reuse the existing Rel-18 capability for MOP for ATG UE in Rel-19

Issue 3-1-2: Tx RF requirements for intra-band contigous CA with one UL carrier and inter-band CA with one UL carrier.

Agreement:

- Reuse Tx RF requirements for ATG UE single CC operation

  - UL MIMO

Issue 3-4-1: clarification of UL-MIMO

Agreement:

- Introduce UL MIMO requirements with 2Tx

Issue 3-4-2: max layer number for UL-MIMO

Agreement:

- 2 layers

Issue 3-4-4: the operation band for UL MIMO

Agreement:

- UL MIMO apply to all existing ATG operation bands

Issue 3-4-7: MPR/AMPR

Agreement:

- Do not define MPR/A-MPR requirements

There are still many open issues that need to be addressed and the following session will discuss the RF requirement for ATG UE UL MIMO.

## RF requirements for ATG UE UL MIMO

### ULFPTx mode for UL-MIMO

In the case of one antenna port being configured (single layer UL MIMO operation) but with two Tx antenna connectors. From Rel-16 onwards, with the support of full power mode, one can transmit at full power either by combining two connectors using a coherent precoder or by supporting a full power PA in full power mode. It is to ensure that UE can transmit at full power, i.e., if the UE is configured with only a single antenna port, it should always transmit at full power.

- ULFPTx mode should be supported for ATG UL-MIMO

### Maximum output power

We don’t see the need for a new maximum output power for ATG UE.

- For UE maximum output power, reuse the rated maximum output power declared by the ATG UE capability maxOutputPowerATG-r18.

### Configured transmitted power

- For configured transmitted power, reuse the existing ATG UE requirement with PCMAX,c tolerance of below Table

Table 2.1: ATG PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 40 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -19 ≤ PCMAX,c < 11 | 7.0 |  |

### Output power dynamic, Transmit signal quality, and output RF spectrum emissions

The UE is possible to be scheduled for a single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission with precoding matrix W=1. Therefore, from the completeness perspective, the single antenna requirement still needs to be specified for ATG UE UL MIMO.

It is suggested to re-use the existing NR UL-MIMO requirement for the following spec.

- Output power dynamic.

- Transmit signal quality.

- Output RF spectrum emissions.

- Reuse the existing NR UL-MIMO requirement for output power dynamic, transmit signal quality, and output RF spectrum emissions, and do not exclude the single antenna port requirement.

### Rx requirements for supporting UL-MIMO

Adopting a comparable approach in defining UL MIMO requirements for the TN UE, it is proposed that the following requirements could reuse the clauses defined for ATG UE.

- Reference sensitivity for UL MIMO

- Maximum input level for UL MIMO

- Adjacent channel selectivity for UL MIMO

- Blocking characteristics for UL MIMO

- Spurious response for UL MIMO

- Intermodulation characteristics for UL MIMO.

- Reuse the requirements defined in ATG UE for the following requirements: Reference sensitivity for UL MIMO, Maximum input level for UL MIMO, Adjacent channel selectivity for UL MIMO, Blocking characteristics for UL MIMO, Spurious response for UL MIMO, Intermodulation characteristics for UL MIMO.

# Conclusion

In the previous sections, we made the following observations:

No table of figures entries found.Based on the discussion in the previous sections we propose the following:

Proposal 1	ULFPTx mode should be supported for ATG UL-MIMO

Proposal 2	For UE maximum output power, reuse the rated maximum output power declared by the ATG UE capability maxOutputPowerATG-r18.

Proposal 3	For configured transmitted power, reuse the existing ATG UE requirement with PCMAX,c tolerance of below Table

Proposal 4	Reuse the existing NR UL-MIMO requirement for output power dynamic, transmit signal quality, and output RF spectrum emissions, and do not exclude the single antenna port requirement.

Proposal 5	Reuse the requirements defined in ATG UE for the following requirements: Reference sensitivity for UL MIMO, Maximum input level for UL MIMO, Adjacent channel selectivity for UL MIMO, Blocking characteristics for UL MIMO, Spurious response for UL MIMO, Intermodulation characteristics for UL MIMO.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2406594, WF on Rel-19 ATG UE requirements