3GPP TSG-RAN WG4 Meeting #111 	R4-2409750

Fukuoka, Japan, May 20th – 24th, 2024

Agenda Item: 7.6.1

Source: MediaTek inc.

Title: Discussion on core maintenance

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#100 and defined a new WI [1]. In this contribution paper, we provide analysis and discussion on remaining issues from the WF .

# Discussion on CSI-ResourceConfig

During the last meeting, RAN4 discussed the ambiguity of whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP or not if UE supports option B-1-1 and the issue is captured as below:

#### Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1 - FFS following options - Option 1: LS RAN2 and RAN1 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1. - Option 2a:RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1. - Option 2b: Configuration of SSB resources outside active BWP is allowed within CSI-ResourceConfig settings for UEs supporting option B-1-1 - No need to send an LS to RAN1/RAN2. - Option 2c: RAN4 should further clarify what is the ambiguity part of the other WG spec before sending an LS requesting for further details. - Option 3: Update FG53-1 related clarification in L1 RSRP measurement requirements.

Our view regarding the issue:

To further understand the issue, RAN4 should further check 38.331 and 38.214 spec prior sending an LS. From RAN2 spec (i.e. 38.331):

#### – CSI-ResourceConfig The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet. CSI-ResourceConfig information element -- ASN1START -- TAG-CSI-RESOURCECONFIG-START CSI-ResourceConfig ::= SEQUENCE { csi-ResourceConfigId CSI-ResourceConfigId, csi-RS-ResourceSetList CHOICE { nzp-CSI-RS-SSB SEQUENCE { nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need R csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId OPTIONAL -- Need R }, csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId }, bwp-Id BWP-Id, resourceType ENUMERATED { aperiodic, semiPersistent, periodic }, ..., [[ csi-SSB-ResourceSetListExt-r17 CSI-SSB-ResourceSetId OPTIONAL -- Need R ]] } -- TAG-CSI-RESOURCECONFIG-STOP -- ASN1STOP CSI-ResourceConfig field descriptions. bwp-Id The DL BWP which the CSI-RS associated with this CSI-ResourceConfig are located in (see TS 38.214 [19], clause 5.2.1.2.. csi-IM-ResourceSetList List of references to CSI-IM resources used for CSI measurement and reporting in a CSI-RS resource set. Contains up to maxNrofCSI-IM-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise (see TS 38.214 [19], clause 5.2.1.2).. csi-ResourceConfigId Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig.. csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt List of references to SSB resources used for CSI measurement and reporting in a CSI-RS resource set (see TS 38.214 [19], clause 5.2.1.2). The csi-SSB-ResourceSetListExt provides additional references and can only be configured if csi-SSB-ResourceSetList is configured and groupBasedBeamReporting-v1710 is configured in the CSI-ReportConfig that indicates this CSI-ResourceConfig as resourcesForChannelMeasurement. If groupBasedBeamReporting-v1710 is configured in the IE CSI-ReportConfig that indicates this CSI-ResourceConfig as resourceForChannelMeasurement, the network configures 2 resource sets, which may be two CSI SSB resource sets (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1..

While RAN1 spec (i.e. 38.214) provide further details as below:

#### 5.2.1.2 Resource settings Each CSI Resource Setting CSI-ResourceConfig contains a configuration of a list of S≥1 CSI Resource Sets (given by higher layer parameter csi-RS-ResourceSetList), where the list is comprised of references to either or both of NZP CSI-RS resource set(s) and SS/PBCH block set(s) or the list is comprised of references to CSI-IM resource set(s). Each CSI Resource Setting is located in the DL BWP identified by the higher layer parameter BWP-id, and all CSI Resource Settings linked to a CSI Report Setting have the same DL BWP.

Our understanding based on the spec sections captured above that the UE is configured to measure SSB within DL BWP, where the latter is identified by higher layer parameter BWP-id. Which in turn, the BWP-id is not necessarily to be the active BWP. Therefore, our view is that the SSB resources can be configured outside the active BWP when the UE support option B-1-1. Besides, the issues discussed above are related to RAN1/RAN2, hence, it is better to trigger the discussion in RAN1 directly.

Observation: The UE can be configured for csi-SSB-ResourceSetList linked to SSB outside the active BWP and identified by BWP-id.

- RAN4 should further clarify what is the ambiguity part of the other WG spec before sending an LS requesting for further details.

- The discussion should be left to RAN1 group.

# Summary

In this contribution, we have the following proposal:

Proposal 1: RAN4 should further clarify what is the ambiguity part of the other WG spec before sending an LS requesting for further details.

Proposal 2: The discussion should be left to RAN1 group.

# References

- RP-232677, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Bangalore, India, September 2023.

- R4-2406364, “WF on core maintenance and performance part for BWP without restriction”, vivo, Vodafone, Fukuoka, Japan, April 2024.