3GPP TSG-RAN WG4 Meeting #112                                                                                                      R4-2411119

Maastricht, Netherlands, 19th –23th August, 2024

Source: 	CATT

Title: 	Discussion on spatial channel modelling methodology for demodulation performance requirements

Agenda item:	8.14.2

Document for:	Approval

- Introduction

In RANP#104, a study item was approved on spatial channel modelling for demodulation performance requirements [1]. In this contribution, we provide our initial views.

- Discussion

Limitations of the current channel models

As per the SID [1], the issue concerning limitations of the current channel models is shown as below:

- Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance

In sub-clause G.2 of TS 38.104, the current (i.e. up to and including Release 18) channel model for demodulation performance requirements consists of multi-path fading propagation conditions and MIMO Channel Correlation Matrices. The delay profiles for MIMO Channel Correlation Matrices is TDL-x channel model, and for MIMO Correlation Matrices, g Uniform Linear Array (ULA) and Multi-Antenna channel models using cross polarized antennas were defined in sub-clause G.2.3.1 and G.2.3.2 respectively. The limitation lies in that MIMO Correlation Matrices just include the correlation between MIMO antennas which indicates same combined gain of same signal from MIMO antennas in any spatial angle, and don’t include antenna array response varied with spatial angle (AoD,ZoD, AoA and ZoA), hence the combined gain of same signal from MIMO antennas varied with spatial angle  is not modelled.

Proposal 1: To study the limitation of the current (i.e. up to and including Release 18) channel models, for example, MIMO channel correlation matrices doesn’t indicate combined gain of same signal from MIMO antennas varied with spatial angle.

Spatial channel modelling methodology

As per the SID [1], the issue concerning spatial channel modelling methodology is shown as below:

- Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes.

From above analysis for limitations of the current channel models, MIMO correlation matrices is used in channel model for demodulation performance requirement, but the antenna array response is not modelled, so it cannot show the combined gain of same signal from MIMO antennas is varied with spatial angle (AoD, ZoD for BS side, and AoA, ZoA for UE side).

The antenna array model includes element maximum gain, element response pattern, antenna polarization model, array response pattern, etc.

Referring to Table 7.3-1 of TR 38.901 as copied below, the element maximum gain is configured fixed value, for example, in following table, the element maximum gain(GE,max) is 8dBi.  From perspective of link demodulation performance evaluation, the element maximum gain is not need to be modelling for spatial channel model for demodulation performance requirement to ensure the signal power is equal 1.

From Table 7.3-1, the element response pattern is normalized 3D radiation power pattern, which is calculated from vertical cut of the radiation power pattern and horizontal cut of the radiation power pattern.

Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) |  |
| Horizontal cut of the radiation power pattern (dB) |  |
| 3D radiation power pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

The antenna polarization models Model-1 and Model-2 are defined in sub-clause 7.3.2 of TR 38.901, the antenna model is dependent on element response pattern.

The array response pattern is modelled in equation (7.5-28) and (7.5-29) of TR 38.901, the  and indicate RX array reponse for angle and  respectively. The  and indicate TX array response for angle and  respectively.  is the spherical unit vector with azimuth arrival angle ϕn,m,AOA and elevation arrival angle θn,m,ZOA, given by

,

where n denotes a cluster and m denotes a ray within cluster n.  is the spherical unit vector with azimuth departure angle ϕn,m,AOD and elevation departure angle θn,m,ZOD, given by

,

is the location vector of receive antenna element u and is the location vector of transmit antenna element s,   and is dependent on TX array parameter and RX array parameter respectively.

Based on above analysis, to solve the limitations of the current channel models, the antenna array model including element response pattern, antenna polarization model,  and array response pattern is need to be defined for spatial channel model for demodulation performance requirement. In TS 38.104, one to eight antennas for gNB, one to four for UE are defined, for spatial channel model, it is need to study how to define antenna array model including element response pattern, antenna polarization model, and array response pattern for spatial channel model for demodulation performance requirement

So, we propose,

Proposal 2: Study how to define antenna array model including element response pattern, antenna polarization model, and array response pattern for spatial channel model for demodulation performance requirement.

In TR 38.901, for link-level evaluations, the spatial angle ,, and are generated using CDL model. TDL model can not be used to geneate these angle for antenna array model. So, it is reasonable to use CDL model to generate angle ,, and .

In TR 38.901, both TDL model and CDL model can generate multi-path fading propagation conditions (path delay, and path power), but CDL model can generate path angle (, ) for every path, but TDL model can’t include any angle information. So we think that it is reasonable to use the CDL model to define multi-path fading propagation conditions. It is need to study what CDL model parameters are used for spatial channel model for demodulation performance requirement

Proposal 3: Study what CDL model parameters are used for spatial channel model for demodulation performance requirement.

- Conclusion

This contribution provides analysis on spatial channel modelling methodology for demodulation performance requirements, and the following proposals and observations are concluded as follows:

Proposal 1: To study the limitation of the current (i.e. up to and including Release 18) channel models, for example, MIMO channel correlation matrices doesn’t indicate combined gain of same signal from MIMO antennas varied with spatial angle.

Proposal 2: Study how to define antenna array model including element response pattern, antenna polarization model, and array response pattern for spatial channel model for demodulation performance requirement.

Proposal 3: Study what CDL model parameters are used for spatial channel model for demodulation performance requirement.

- References

- RP-241610, Study on spatial channel model for demodulation performance requirements, Nokia, BT Plc, etc., RAN#104