3GPP TSG-RAN WG4 Meeting #112		R4-2412413

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	8.4.2

Source:	Intel Corporation

Title:	Discussion on the remaining issues for Rel-19 less than 5MHz work item for TN phase 2

Document for:	Discussion

# Introduction

The Rel-19 NR channel BW less than 5MHz for FR1 Phase 2 work item was updated in June 2024 in [1]. The work item Core part objectives are provided below.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

The idea of the objectives is to first define generic RF requirements for inter-band NR-CA and NR-DC n100-n101 band combinations considering less than 5MHz bandwidth in FDD band n100 and 5 MHz or 10 MHz in TDD band n101, and specify general RF requirements/applicability for inter-band CA/DC. Further RRM requirements are also planned to be specified and the RRM work is scheduled to start from August WG meeting 2024 according to the RAN4 TU allocation.

In the last WG meeting, a WF in [2] was approved capturing the agreements on the RF scope of the item. In this paper we provide our further views on the open issues for RF requirements related to the scope of this item.

# Discussion

## Inter-band CA/DC with 5MHz channel bandwidth in n100

Example band combinations CA_n100-n101 and DC_n100-n101 with 5MHz support in n100

In the last WG meeting, support of 5MHz CBW in band n100 for CA and DC n100-n101 was agreed [2] as shown below.

Issue 1-1: Support of 5MHz CBW in band n100 for CA_n100-n101 and and NR DC_n100-n101 Agreement (online session) - In RAN4 common understanding, the support of 5MHz CBW on band n100 and 5MHz/10MHz on n101 in the band combination with band n100 and n101 can be included in Rel-19 WID.

We note that it is desirable to add 5MHz at an early stage of the item (before introducing the specification) to avoid defining a separate BCS to enable 5MHz channel bandwidth operation at a later stage. All DC combination requirements resemble the ones specified for the corresponding CA combination. Therefore, the support of 5MHz channel bandwidth shall be defined for DC_n100-n101 as well.

In the big draftCR for the RF requirements in this meeting, we add 5MHz support in the proposed TP.

Proposal 1: Specify support of 5MHz channel bandwidth together with 3MHz for band n100 in CA_n100-n101 and DC_n100-n101 in the UE RF specifications.

## Signalling aspects for UE support of NR CA/DC combos with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth

In our view, there is no need to discuss additional UE capabilities other than the supported bandwidth for CA and DC combos in the RF session for this work item. The existing framework for UE indications in CA and DC applies well to the 3MHz and 5MHz support for band n100 in CA_n100-n101 and DC_n100-n101. The UE indicating the support for 3MHz or 5MHz in the mentioned combinations is expected to meet all the RF requirements specified in this work item.

Regarding potential enhancements on the network signalling for better UE mobility performance in SCell addition or other related operations, which we raised in RAN4#110bis, we think that it would be more appropriate to discuss the related questions as a part of RRM requirement definition.

Proposal 2: No new signalling indication is required in the RF scope of this item.

Observation: potential enhancements on the network signalling for better UE mobility performance in SCell addition or other related operations can be discussed in RRM room.

However, in the existing Rel-18 UE features for less than 5MHz bandwidth support, descriptions about non-applicability against MR-DC deployment need to be removed or new set of UE features dedicated to CA and DC deployments need to be introduced.

In this sense we propose to remove the descriptions against non-single-carrier deployments in the existing IE descriptions and reuse the Rel-18 elements. This is actually dependent on RAN2 expertise and we propose to send an LS to RAN2 in this meeting about the consensus from RAN4 about removing and reusing.

Proposal 3: Send an LS to RAN2 about removing the descriptions against non-single-carrier deployments in the Rel-18 UE capability elements in the Rel-19 specs.

## Basket work item

In order to accommodate other CA/DC band combination requests than the example shown in the current WID, we need to adopt the basket work item approach. In June, a new WID for Rel-19 NR CA, DC and SUL band combinations is approved in [3].

We will use the approved item to accommodate new requests from interested parties and specify the combo-specific requirements in the place holder.

Proposal 4: Use the approved basket item [3] to accommodate new requests from interested parties and specify the combo-specific requirements.

Specifically, the RF requirements in this item should be specified following Objective #2 in the WID [3].

The preconditions to propose NR inter-band CA/DC configurations with 2 different bands DL with up to 2 different bands UL in Rel-19 are as follows. - Constituent NR band and NR Intra band CA shall be completed and specified in advance. - All the fallbacks of the proposed inter-band CA/DC shall be completed and specified in advance. Example 1: If the following configuration is proposed, CA configuration; Uplink CA configuration. CA_n1A-n2A; -. - NR Band n1 and n2 requirements shall be completed and specified in advanced.

## RF work plan

The work item targets completion of the Core part before December 2024 with the following TU allocation for RF/RRM Core parts:

|  | RAN4 #110bis Apr 2024 | RAN4 #111 May 2024 | RAN4 #112 Aug 2024 | RAN4 #112bis Oct 2024 | RAN4 #113 Nov 2024 |
| --- | --- | --- | --- | --- | --- |
| RF Core | 0.25 | 0.25 | 0.25 | 0.25 | 0.25 |
| RRM Core |  |  | 0.25 | 0.25 | 0.25 |

In the last WG meeting, the group agreed that

Rapporteur (Intel) will prepare a single draft CR for RAN4 #112 covering the agreements in RAN4 #110bis and #111 and share before the RAN4 #112 with interested companies.

Thus in this meeting we submit a big draftCR in [4] to collectively propose all the agreed RF requirements and we expect this tdoc to be endorsed in this meeting so that we can continue to work on the big draftCR until completion of the work item.

# Conclusion

In this paper we provide our views on the open issues for RF requirements related to the scope of this item.

Proposal 1: Specify support of 5MHz channel bandwidth together with 3MHz for band n100 in CA_n100-n101 and DC_n100-n101 in the UE RF specifications.

Proposal 2: No new signalling indication is required in the RF scope of this item.

Observation: potential enhancements on the network signalling for better UE mobility performance in SCell addition or other related operations can be discussed in RRM room.

Proposal 3: Send an LS to RAN2 about removing the descriptions against non-single-carrier deployments in the Rel-18 UE capability elements in the Rel-19 specs.

Proposal 4: Use the approved basket item [3] to accommodate new requests from interested parties and specify the combo-specific requirements.

In this meeting we submit a big draftCR in [4] to collectively propose all the agreed RF requirements and we expect this tdoc to be endorsed in this meeting so that we can continue to work on the big draftCR until completion of the work item.

# References

- RP-241622, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation

- R4-2410602, WF on UE RF requirements for NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation

- RP-241674, New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Moderator (RAN4 Chair, Huawei)

- R4-2412414, Big draftCR for less than 5MHz UE RF requirements in Rel-19, Intel Corporation