3GPP TSG RAN WG4 Meeting #112	R4-2413017

Maastricht, NL, 19 Aug. 2024 – 23 Aug. 2024

Agenda Item:	8.18.3

Source: 	Ericsson

Title:	Discussion on Rel-19 MIMO RRM requirements

Document for:	Decision

# Introduction

In RP-234007, WID for MIMO phase 5 is agreed. Same is copied here for reference.

RAN1:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

Acronyms:

- sTRP: single TRP (transmit-receive point)

- mTRP: multiple TRP (transmit-receive point)

- CJT: coherent joint transmission

- DU: distributed unit

- CRI: CSI-RS resource indicator

RAN2:

Specify higher layer support of the enhancements listed above.

RAN4:

Specify necessary core requirements for the enhancements listed above.
…

In this contribution we analyze the potential RAN4 RRM impacts for NR MIMO Phase 5.

# Discussion

From the WID we can observe that RAN4 scope depends on the RAN1 enhancements introduced for each objective. In this contribution, we look at each objective and the corresponding RAN4 impact based on the RAN1 agreements in the below sections.

## Objective 1: UE-initiated beam management

In Rel-19, RAN1 introduced event triggered or UE initiated beam reporting to reduce the signalling overhead arising from the periodic CSI in the UL. Further RAN1 agreed to support at least L1-RSRP as the measurement quantity for the event triggered beam reporting. Based on the RAN1 agreements (please check annex), we summarize the framework for UE initiated beam report in figure 1.

Figure 1: Framework for UE initiated beam reporting

As shown in figure 1, UE initiated beam management framework can be divided into configuration aspects and UE actions. From the RAN1 agreements we can observe that RAN1 is still discussing the resource configuration, event configuration, events defintion, and the report configuration. As the configuration aspects are specified in RAN1/2, RAN4 do not need to discuss the configuration aspects and RAN1 progress may not have impact on RAN4.

UE actions: From the figure 1, we think following UE actions may have impact on the RAN4 requirements and needs to be discussed in RAN4.

- L1 measurements for event triggered beam report

- Event evaluation

- Measurement Report transmission

  - Transmission on first UL channel

  - Obtain resource for second UL channel

  - Transmit on second UL channel

L1 measurements for event triggered beam reporting

RAN1 agreed to support at least L1-RSRP for the event triggered beam report. L1-SINR was discussed but it was not agreed yet. Further RAN1 agreed to support SSB based event triggered L1-RSRP reporting and periodic CSI-RS based event triggered L1-RSRP reporting. Hence, RAN4 should specify the requirements for SSB based event triggered L1-RSRP reporting and periodic CSI-RS based event triggered L1-RSRP reporting.

- RAN4 to specify the requirements for SSB based event triggered L1-RSRP reporting and periodic CSI-RS based event triggered L1-RSRP reporting.

Event evaluation:

L1-RSRP is used for beam management and periodic L1-RSRP can be configured with periodicity of the SSB or CSI-RS. Since the report is used for beam management, UE should be able to send the report as soon as event condition is met. Event triggering conditions can be changed as fast as the measurement occasions. UE is expected to evaluate the event after every measurement occasion to check if any of the configured event is met. Once the event is met as shown in figure 1, UE needs to send the first UL signal to the NW to indicate to the NW that event is met. If the time between measurement occasion and first UL channel is short UE may not be ready to transmit on the first UL channel. In this case UE may need to wait for next transmission opportunity of the first UL channel. To avoid the scheduling of the first UL channel so close to the measurement occasions, RAN4 should define the event evaluation time.

- RAN4 to disucss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on first UL channel from the measurement occasion)

Measurement Report transmission

Either Mode-A or Mode-B discussed in RAN1 can contain following three steps.

- Transmission on first UL channel

- Obtain resource for second UL channel

- Transmit on second UL channel

The above three steps depend on the configuration and NW scheduling. We may not need to specify RAN4 requirements for these steps.

- RAN4 may not need to specify requirements for Transmission on first UL channel, obtain resource for second UL channel and transmit on second UL channel.

## Objective 2: CSI enhancement for large arrays

Objective is copied below for the reference.

Specify CSI support for up to 128 CSI-RS ports, targeting FR1

- Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

- Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

- Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

As per our understanding objective 2 may have some Demod impact but we do not foresee any RRM requirements impact.

- Specification of CSI support for up to 128 CSI-RS ports may not have RRM requirements impact.

## Objective 3: UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

In previous releases, RAN1 specified CJT under ideal backhaul and ideal synchronization conditions. Since the ideal backhaul and ideal synchronization may not exist in real NW deployments, in Rel-19 RANs WG were tasked to specify CJT under non-ideal backhaul and non-ideal synchronization. To enable this, RAN1 introduced following aperiodic standalone CJT calibration reporting.

- Frequency offset (FO) reporting (ReportQuantity is cjtc-F)

- Delay offset (DO) reporting (ReportQuantity is cjtc-Dd)

- Joint delay offset and frequency offset (DO-FO) report

- DL/UL phase offset (PO) reporting (ReportQuantity is cjtc-P)

Through these reports CJT could be possible under real NW deployment conditions by NW compensating for the synchronization error or other impairments. Having said that, the benefit of the CJT depends on the newly introduced measurement reports and the accuracy of these reports from the UE. To get clearer picture on the importance of the accuracy of the measurement, we present different CJT applications and their expected accuracy level for different applications.

Different applications, like only coherent combining and nulling, depending on region of operation w.r.t SNR, needs different level of accuracy of the phase estimation from the UE. Different cases will also require different total budgets. As examples we will use a total budget of 50 degrees for only coherent combining and then 30 and 10 degrees for nulling, depending on SNR:

- only coherent combining total budget: 50 degrees

- nulling total budget, low and high SNR: 30 and 10 degrees.

To enable different applications at the NW side, NW needs to know what the UE estimated accuracy of the measurement so that NW may or may not schedule the UE with different feature based on the accuracy of the measurement.

- Different CJT applications need different levels of accuracy requirement.

- RAN4 to define the requirements for the Frequency offset (FO), Delay offset (DO), Joint delay offset and frequency offset (DO-FO), DL/UL phase offset (PO) reporting.

RAN4 measurement requirements traditionally consist of measurement delay and measurement accuracy requirements. In the subsequent part of the paper we discuss about measurement delay and accuracy.

Measurement delay requirements:

The CJT calibration reports introduced by RAN1 are aperiodic. The calibration measurements are based on the periodic TRS and is aperiodically requested by the NW. When an aperiodic report is requested by the NW, UE needs to report the measurement in the corresponding reporting resources (called as the reference resource) as defined in the RAN1 specification.  Since UE do not know when the NW will request the report, we expect UE to measure the CJT calibration measurement, during every measurement occasion and store it in the UE buffer.  While defining legacy measurement requirements, RAN4 did not define aperiodic reporting requirements. However, the UE behaviour for measurement was not same among different companies based on offline discussion among different companies during Rel-18 LTM WI.  To avoid ambiguity on the UE behaviour, we suggest defining the measurement behaviour or measurement delay for the aperiodic measurement reporting.

- RAN4 to define the measurement delay or measurement behaviour for the aperiodic standalone CJT reporting

For some of the measurements, we observed that RAN1 agreed reporting resolution does not work well to accommodate the accuracy level that was needed for different applications as mentioned above. In the subsequent sections we provide analysis on the reporting granularity for different measurement reports and provide our recommendation on the reporting granularity to be feedback to RAN1.

### Delay offset report

To compute the delay offset, multiple TRP and multiple resource sets (TRS) are configured for the UE. Up to 𝑁_𝑇𝑅𝑃∈{1,2,3,4} periodic TRS (CSI-RS for tracking) resource sets can be configured for the delay offset report. Where one resource set corresponding to one TRP. All the resources across the N_TRP TRS resource sets are configured with the same bandwidth. Among the resource sets configured for the reporting, UE assumes one resource set as the reference resource and measures the delay of the other resource sets w.r.t the reference resource. The measured delay offset is quantised and feedback to the NW in the delay offset report.

Measurement mechanism:

In a wireless environment, due to multipath, channel can have multiple taps and each tap corresponds to different paths. It is important that all the UEs measure the delay offset corresponding to the same tap or all the TRPs delay is measured using the same tap. Since RAN1 did not specify this aspect, it is important that RAN4 has common understanding on the which path to consider for the delay offset measurement. Generally, for the timing measurements, in RAN4 it is common practice to assume the first path. For example, similar measurement report such as RSTD developed as part of the positing WI considers the first path.

- RAN4 to agree that delay offset is measured on the first detected path in time of the periodic TRS resource set of reference TRP and other TRP.

RAN4 in previous releases defined RSTD measurement for positioning applications. We think both of these reports are similar (except the RS is different and the nodes transmitting the RS is different) and RAN4 can reuse the requirements framework developed in positioning WI here.

- RAN4 can use framework developed in positioning WI as a starting point for the delay offset

#### Accuracy requirements

Since this is the first meeting for MIMO phase 5 in RAN4, before discussing the accuracy requirements, we look at whether the RAN1 agreed reporting granularity is good enough to accommodate the potential RAN4 accuracy requirements.

To avoid wrap around ambiguity due to delay (delay uncertainty), we require that Phase Change within a Sub band < 180 degrees. If BRBis the number of resource blocks, ∆RTD is the receive time difference at the UE RX and SCS is the subcarrier spacing, then we get:

BRB∙360∙∆RTD<180

Where:

BRB=#RB∙12∙SCS

and ∆RTD=TAE+ ∆RF-propagation.

There will be errors in measurement and reporting resolution (εtot= εmes+ εres) that could lead to wrap around ambiguity. This gives us:

BRB∙360∙εtot<180 

εtot<0.5/BRB 

We get a max error due to Delay offset report resolution: εres=0.5⋅AD/MD. If we assume εmes= εres then εtot=2εres and we have:

εres<0.25/BRB

0.5⋅ADMD<0.25/BRB

MD>2ADBRB

If we use SCS = 15 kHz and SCS = 30 kHz together with AD=CP and  AD=CP/2 we get:

|  | BRB SCS (Hz) |  | SCS = 15 kHz |  | SCS = 30 kHz |  |
| --- | --- | --- | --- | --- | --- | --- |
| RB | 15000 | 30000 | MD (AD = CP) | MD (AD = CP/2) | MD (AD = CP) | MD (AD = CP/2) |
| 2 | 3,60E+05 | 7,20E+05 | 3,4 | 1,7 | 3,4 | 1,7 |
| 4 | 7,20E+05 | 1,44E+06 | 6,8 | 3,4 | 6,8 | 3,4 |
| 8 | 1,44E+06 | 2,88E+06 | 13,5 | 6,8 | 13,5 | 6,8 |
| 16 | 2,88E+06 | 5,76E+06 | 27,0 | 13,5 | 27,0 | 13,5 |
| 32 | 5,76E+06 | 1,15E+07 | 54,0 | 27,0 | 54,0 | 27,0 |

Table : Required number of levels for (DO) reporting

If we compare this with RAN1 agreements of MD = {32,64}, that is 5 to 6 bits. We observe that number of levels for DO reporting fits fairly well with what is required.

… Agreement (RAN1#116bis) For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for delay offset reporting Dn,offset, i.e. (AD, MD), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AD ={0.5CP, 0.75CP, CP, 1.5CP, 2CP, 14∆f, 112∆f, 124∆f} where CP and ∆f denote the length of the cyclic prefix according to the current specifications (for normal CP) within a slot and the SCS, respectively - FFS: Further down-selection of the above candidate values for AD, including the use of a same unit for all supported values - MD ={32, 64} - FFS: If TDD TX/RX timing misalignment report is supported, whether different set of candidate MD values is needed In addition, the inside/outside range for the 1-bit indicator dn is equal to [0, CP]. FFS: Further implicit/explicit restriction(s) on candidate value(s) depending on the CSI-RS configuration …

- We observe that the required number of levels for DO reporting fits well with RAN1 agreements of MD = {32,64}, that is 5 to 6 bits.

### Frequency offset report

To compute the frequency offset for the CJT scenario, multiple TRP and multiple resource sets (TRS) are configured for the UE. Up to 𝑁_𝑇𝑅𝑃∈{1,2,3,4} periodic TRS (CSI-RS for tracking) resource sets can be configured for the report, where one resource set corresponding to one TRP. All the resources across the N_TRP TRS resource sets are configured with the same bandwidth. Among the resource sets configured for the reporting, UE assumes one resource set as the reference resource and measures the frequency offset of the other resource sets w.r.t the reference resource. The measured frequency offset is quantised and fedback to the NW in the frequency offset report.

Measurement mechanism:

In this section, we first look at potential method of how UE estimates the frequency error for one TRP. Then, we look at the method of frequency error between reference TRP and other TRP configured for CJTC measurement. As per our understanding UE determine the frequency error in the FFT domain on the demodulated signal which is mapped to constellations as shown in figure 2. In the figure 2a, for a given frequency, UE knows what the reference signal transmitted and what was expected constellation. Due to frequency error, delay, and phase, received and ideal constellation is shown in figure 2b and 2c at time t1 and t2.

Figure 2: Example frequency error estimation method

For a given TRS, since the TRS is separated by 4 OFDM symbols, we can assume that at t1 and t2, frequency error, phase is same and does not change within the TRS resource.

Figure 3: example TRS configuration

Figure 3: example TRS configuration

yt1=h1ej2πf1-f0t1+φ1

y(t2)=h1ej(2πf1-f0t2+φ1)

By assuming channel does not change much in 4 OFDM symbols, and f1-f0 to be same at t1 and t2, UE can estimate φ1 first and the from y(t1) and y(t2) and φ1, UE can estimate the frequency offset or frequency error for a given TRP. In this example, delay is assumed same for both TRS  reception. In a similar manner UE can determine the frequency error for the other TRS or other TRPs. Since the frequency offset also depends on which path UE is measuring for the other TRP, it is important to have a common assumption on which path to use for frequency offset computation. Similar to delay offset, we think frequency offset shall also assume first detected path in time for the frequency offset computation.

- RAN4 to agree that frequency offset is measured on the first detected path in time of the periodic TRS resource set of reference TRP and other TRP.

#### Accuracy requirements:

Since this is the first RAN4 meeting for MIMO Phase 5, before discussing the accuracy requirements, we look at whether the RAN1 agreed reporting granularity is good enough to accommodate the potential RAN4 accuracy requirements.

The accuracy requirements for the delay offset reporting depends on the total phase error budget we can tolerate and how we allocate this total budget onto the terms on the budget. The total budget, in turn, depends on the use case for CJT. Examples of total budgets:

- only coherent combining total budget: 50 degrees

- nulling total budget, shallow and deep: 30 and 10 degrees.

If we further develop a budget with at least the following terms:

- UE measurement error.

- UE reporting resolution/quantization error.

- BS algorithmic compensation error.

- Inter TRP phase drift.

- Potential inter TRP mast sway.

- Potential UE mobility

If we divide a total budget of 50-, 30- or 10-degrees phase error evenly across the 6 terms in the budget we get 8.3, 5 or 1.7 degrees per term. We can now calculate the required number of bits for the frequency offset (FO) reporting.

The frequency resolution error is ½ of a quantization step:

∆fmax⁡_resolution_error=0.5fcarrierAFOMFO

Where AFO is the maximum carrier frequency error in Hz and MFO is the number of quantization levels.

We also know that:

360Treport∆fmax⁡_resolution_error<Allocation

Hence:

AFOMFO<Allocation360Treport0.5fcarrier⇒ MFO> AFO360Treport0.5fcarrierAllocation

If we assume fcarrier= 7.125 GHz, that is the highest FR1 band and a Treport= 30 ms we then get:

| Allocation | Treport | MFO (AFO = 200 ppb) | #bits | MFO (AFO = 100 ppb) | #bits | MFO (AFO = 50 ppb) | #bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8,30 | 0,01 | 309 | 8,3 | 155 | 7,3 | 77 | 6,3 |
| 8,30 | 0,03 | 927 | 9,9 | 464 | 8,9 | 232 | 7,9 |
| 5,00 | 0,01 | 513 | 9,0 | 257 | 8,0 | 128 | 7,0 |
| 5,00 | 0,03 | 1539 | 10,6 | 770 | 9,6 | 385 | 8,6 |
| 1,70 | 0,01 | 1509 | 10,6 | 754 | 9,6 | 377 | 8,6 |
| 1,70 | 0,03 | 4526 | 12,1 | 2263 | 11,1 | 1132 | 10,1 |

Table : Required number of bits for frequency offset (FO) reporting

If we compare this with RAN1 agreements of MFO = {16,32}, that is 4 to 5 bits. We observe that we require more bits for the frequency offset (FO) reporting.

… Agreement (RAN1#116bis) For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for frequency offset reporting FOn, i.e. (AFO, MFO), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - AFO = {0.01ppm, 0.1ppm, 0.2ppm, f, f/2, f/4,f/8, 1/(4t), 1/(8t), 1/(16t), 1/(32t), 1/(512t)} where f and t denote the SCS and duration of one OFDM symbol, respectively - FFS: Further down-selection of the above candidate values for AFO, including the use of a same unit for all supported values - MFO = {16,32} FFS: Whether additional restriction(s) based on CSI-RS configuration is supported, including implicit configuration of quantization range …

- We require more bits for the frequency offset (FO) reporting than the 4 to 5 bits (MFO = {16,32}), agreed in RAN1.

In the analysis in this section, we considered equal error allocation to all the error terms. However, it may not be the case and RAN4 should investigate this.

- RAN4 should investigate the allocation of total budget onto separate error terms, including UE Resolution, for frequency offset reporting, to fit the required resolution to a total budget approach.

- The required UE Resolution for frequency offset reporting, from a RAN4 point of view, has to be feed back to RAN1.

### Joint Delay offset/Frequency offset (DO-FO)

Since this a joint report containing delay and frequency offset, we think we can revisit this after sufficient agreements in delay and frequency offset are made.

### DL/UL phase offset report

In reciprocity-based DL transmission, UE calibrates the phase for each TRP before transmitting on the UL. However, after the TRP calibration the absolute phase difference between Tx and Rx at each TRP is unknown and could be different for different TRP. Without knowing the absolute phase for each TRP, coherent joint transmission is not possible. Due to this RAN1 introduced DL/UL phase offset report.

From the CSI-RS reference resource for tracking, UE can estimate the phase of each TRP as explained in 2.3.3. Upon the DL phase estimation, UE use the same phase for UL calibration so that the signal is received with same phase as the gNB transmitted.  For this report, UE considers phase from one TRP as the reference and estimate the phase difference of the other TRPs w.r.t reference TRP and transmit the quantized value  to NW in the phase offset report.

#### Accuracy requirements

Since this is the first RAN4 meeting for MIMO Phase 5, before discussing the accuracy requirements, we look at whether the RAN1 agreed reporting granularity is good enough to accommodate the potential RAN4 accuracy requirements.

If we revisit the argument of a total budget for some examples of total budgets:

- only coherent combining total budget: 50 degrees

- nulling total budget, shallow and deep: 30 and 10 degrees.

Where the total budget has at least the following terms:

- UE measurement error.

- UE reporting resolution.

- BS algorithmic compensation error.

- Inter TRP phase drift.

- Potential inter TRP mast sway.

- Potential UE mobility.

Then, if we divide a total budget of 50-, 30- or 10-degrees phase error evenly across the 6 terms in the budget we get 8.3, 5 or 1.7 degrees per term.

RAN1 has agreed 16 or 32 levels for the DL/UL phase offset report:

… Agreement (RAN1#116bis) For the Rel-19 aperiodic standalone CJT calibration reporting, the resolution parameters for, n, i.e. M are NW-configured via higher-layer (RRC) signalling from the candidate values {16, 32}, where Φn,σ∈0,2πMΦ-1,4πMΦ-1,…,MΦ-22πMΦ-1,'invalid'. …

We then get a worst phase error when we calculate the quantization uncertainties of:

0.5⋅36016=11.25

0.5⋅36032=5.6

If we compare this with the 8.3, 5 or 1.7 degrees per term, we are fine when it comes to only coherent combining, but not when it comes to coherent combining and nulling.

- We observe that the required number of levels for DL/UL phase offset report reporting fits well with what with RAN1 agreements of MΦ = {32,64}, for coherent combining, but not when it comes to coherent combining and nulling.

In the analysis in this section, we considered equal error allocation to all the error terms. However, it may not be the case and RAN4 should investigate this.

- RAN4 should investigate the allocation of total budget onto separate error terms, including UE Resolution, for DL/UL phase offset report, to fit the required resolution to a total budget approach.

- The required UE Resolution for DL/UL phase offset report, from a RAN4 point of view, has to be feed back to RAN1.

## Objective 4: 3 Tx UL

Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

As per our understanding objective 4 may have some RF impact and Demod impact but we do not foresee any RRM requirements impact.

- specification of 3-TX UL may not have RRM requirements impact.

## Objective 5: UL-only node

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

Based on the above RAN1 agreements, we can summarize the enhancements introduced or discussed in RAN1 for supporting UL only TRP as following.

- Two closed-loop PC adjustment states for SRS, both separate from PUSCH

- PL offset configuration and update

  - PRACH enhancements using different PL offset

  - TCI state enhancements using different PL offset

- Two TA enhancements (FFS in RAN1)

Two closed-loop PC adjustment states for SRS, both separate from PUSCH

Typically, power control and power control offset are not discussed in RRM. Hence, RRM impact is not foreseen for the two CLPC for SRS.

- Two closed-loop PC adjustment states for SRS, both separate from PUSCH may not have RRM requirements impact.

PL offset configuration and update:

As per the RAN1 discussion we can see that PL offset is introduced per TCI state. Whether same offset can be used for PRACH, or different offset is needed is under discussion in RAN1.

TCI state enhancements using different PL offset

The offset within the TCI state is applicable for SRS, PUSCH/PUCCH. It is configured in the RRC message and can also be updated using the MAC CE at later stage. Corresponding RAN1 agreement can be found below.

For the association between PL offset and joint/UL TCI state, support the following

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

PL-RS configuration requirements:

When RRC configures the PL-RS and pathloss offset for the TCI state associated with UL only TRP, the PL-RS may be same as the PL-RS configured for the reference TRP or may be different. When a PL-RS is configured, the time required for PL estimation from PL-RS and the configured pathloss offset can depend on whether the PL-RS is maintained or not. RAN4 to study PL estimation requirements from the PL-RS and pathloss offset.

- RAN4 to study PL estimation requirements from PL-RS and pathloss offset.

As per the RAN1 agreement, MAC CE can update the PL offset. Since the MAC CE does not change the PL-RS, pathloss offset estimation requirements do not depend on whether PL-RS is maintained or not as the PL-RS was configured earlier through RRC and MAC CE just update the pathloss offset. Hence, MAC CE based pathloss offset estimate contains only the MAC CE processing time.

- MAC CE based pathloss offset update requirement delay is equal to MAC CE processing time.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to specify the requirements for SSB based event triggered L1-RSRP reporting and periodic CSI-RS based event triggered L1-RSRP reporting.

- RAN4 to define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on first UL channel from the measurement occasion).

- RAN4 to define the requirements for the Frequency offset (FO), Delay offset (DO), Joint delay offset and frequency offset (DO-FO), DL/UL phase offset (PO) reporting.

- RAN4 to define the measurement delay or measurement behaviour for the aperiodic standalone CJT reporting

- RAN4 to agree that delay offset is measured on the first detected path in time of the periodic TRS resource set of reference TRP and other TRP.

- RAN4 to agree that frequency offset is measured on the first detected path in time of the periodic TRS resource set of reference TRP and other TRP.

- RAN4 should investigate the allocation of total budget onto separate error terms, including UE Resolution, for frequency offset reporting, to fit the required resolution to a total budget approach.

- The required UE Resolution for frequency offset reporting, from a RAN4 point of view, has to be feed back to RAN1.

- RAN4 should investigate the allocation of total budget onto separate error terms, including UE Resolution, for DL/UL phase offset report, to fit the required resolution to a total budget approach.

- The required UE Resolution for DL/UL phase offset report, from a RAN4 point of view, has to be feed back to RAN1.

- RAN4 to study PL estimation requirements from PL-RS and pathloss offset.

- MAC CE based pathloss offset update requirement delay is equal to MAC CE processing time.

# References

[1] 	RP-234007, New WID: NR MIMO Phase 5

# Annex

UE initiated BM RAN1 agreements:

RAN1#117 (2024-05)

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

    - FFS: candidate value of ‘N’.

  - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams

- FFS: Option-1/1a/1b/2.

- Above applies at least for the single CC case

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows.

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

  - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state.

- Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Enabling one of either Scheme-1 or Scheme-2 is selected by NW.

  - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

- The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

Agreement

Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported

- Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured

- FFS: Option-3b/3c

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported.

- RRC can enable or disable whether current beam is always reported

    - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported

      - Note: The reported current beam is NOT counted in the N reported beams.

    - When disabled by RRC, N beams are reported.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

    - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

      - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

    - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

      - FFS: DCI format 1_3

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported)

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118:

- Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration;

  - FFS: The RS in the RS resource set can be updated by MAC-CE.

- Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

  - FFS: If a list size is small, MAC-CE activation is not needed

- Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state.

- Others are not precluded.

- FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state.

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement (i.e., in such case, Scheme-2 is used)

- Others are not precluded.

RAN1#116bis (2024-04)

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- At least L1-RSRP is supported as quality metrics used for Event-2

  - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient)

  - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event

- Regarding RS measurement for the new beam for Event-2, down-select one or more of the following:

  - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE

  - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s).

- Note-1: ‘New/current beam’ is for discussion purpose.

- Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded.

- Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state.

- Regarding RS measurement for the current beam for Event-2, Option-2a is supported:

  - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state.

    - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state.

  - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE.

    - Note: SSB or CSI-RS can be configured

Agreement

On UE-initiated/event-driven beam reporting, further study the following trigger events:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-3: Quality of a new beam is better than a certain threshold.

- Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

- Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold.

- Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting).

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

- Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam.

- Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

- Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

  - FFS: Details on how value of N is determined by the UE

- Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - Payload size does not vary as a function of N

  - FFS: Zero-padding can be provided if N is less than Nmax.

- Option-2: Only N=1 beam is reported in the report instance

  - The reported beam should satisfy the condition of Event-2

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

- Other options are not precluded.

- FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC.

- FFS: When current beam is reported, whether the current beam is counted in the N reported beams.

The selected option shall satisfy Event-2.

RAN1#116 (2024-02)

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold.

- Further study trigger events, including the following example as a starting point

  - Event-1: Quality of the current beam is worse than a certain threshold.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - Event-3: Quality of a new beam is better than a certain threshold.

  - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

  - Others are not precluded.

- Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

Agreement

On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP

- FFS: Study and decide whether additional contents can be supported.

- FFS: L1-RSRP format, e.g., absolute and/or differential value.

- Note: Above does not imply to preclude discussion on L1-RSRP filtering.

- The actual reported content depends on the triggering event

Support of one or multiple events will be discussed separately

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

Agreement

On UE-initiated/event-driven beam report, at least of following aspects should be included:

- Trigger-event detection for beam reporting by UE

  - UE monitors RS to assess if a beam-reporting trigger condition has been met

  - FFS: Trigger condition for declaring beam-reporting event

- Beam-report transmission by UE

  - Signaling contents in the beam report

  - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission

    - MAC-CE

    - UCI

    - Others are not precluded.

On UE-initiated/event-driven beam report, the following aspects may be included:

- UE requesting UL resource(s) for the beam report

- UE notifying transmission of beam report

- gNB preconfigured resources

Other procedure(s) as required

UL Only TRP RAN1 agreements

RAN1#117 (2024-05)

- Proposal 3.1:

- To fulfil the asymmetric DL sTRP/UL mTRP deployment scenarios, support two TAs for single DCI based multi-TRP/panel and single TRP.

- Reuse Rel-18 specification of two TA for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured.

Companies are encouraged to consider above for further discussion in RAN1#118

Agreement

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting:

- Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

- FFS: the details of DCI field design.

Agreement

In Rel-19, the value range of starting bit of block in DCI format 2-3 is extended from 1~31 to 1~X.

- FFS the value of X>31

FFS: Condition under which the above is applicable

Agreement

Introduce a new RRC parameter per BWP/CC to indicate that two separate SRS CLPC adjustment states are configured for SRS in a BWP/CC

Agreement

For the association between PL offset and joint/UL TCI state, support the following

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Conclusion

There is no consensus on the following proposal:

Proposal 1.6: Support to update a UL PL for a joint/UL TCI state as follows:

- When this joint/UL TCI state is activated and it is not in the current active TCI state list, a UL PL is calculated as: UL PL = PL estimated from DL PL RS – the value of PL offset.

- When this joint/UL TCI state is activated and it is in the current active TCI state list, the UE updates the UL PL as: new UL PL = current UL PL + the updated delta indicated by the NW.

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, study and decide the value range and candidate values of PL offset value

Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS

Conclusion

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

RAN1#116bis (2024-04)

Agreement

For a UE configured with two SRS CLPC adjustment states, support Alt2 for indicating one of the SRS CLPC adjustment states to SRS:

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 for SRS in the TCI state indicates one of the SRS CLPC adjustment states

  - The candidate value of i0 and i1 in closedLoopIndex-r17 for SRS refers to the first and the second CLPC adjustment state separate from PUSCH, respectively

Ericsson raised concerns on Alt2 due to potential issues with beam management.

Agreement

For indicating TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured with two SRS CLPC adjustment states, support Option3:

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

Agreement

For enhancing DCI format 2_3 for indicating TPC command for two SRS CLPC adjustment states, support Alt2:

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

Note: this 1-bit indicator is present for the CC where two SRS CLPC adjustment states are configured.

Agreement

For FR1, a joint TCI state can be associated with a PL offset.

- When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

Agreement

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification

- Above is subject to a separate UE capability signaling

Agreement

Consider and down-select one from the following alts for indicating a PL offset for PDCCH-order PRACH transmission at least for FR1.

- Alt1: RRC configures multiple PL offset values in PRACH-Config and PDCCH-order DCI indicates one of them through one DCI field.

- Alt2: PDCCH order DCI indicates one PL offset value

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

- Alt4: The PDCCH order DCI indicates one TCI state associated with a PL offset and the associated PL offset is applied on the PRACH transmission.

- Alt5: RRC configures one PL offset value for PRACH and the PDCCH order DCI indicates whether this PL offset value is applied on PRACH transmission or not.

Note: Other alternatives are not precluded

Agreement

For the association between PL offset and joint/UL TCI state, consider and down-select one from the following Alts:

- Alt1a: One PL offset value is configured in a joint or UL TCI state by RRC only

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

- Alt2a: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations.

- Alt2b: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations. A MAC CE can update the association between a joint or UL TCI state and PL offset configuration

- Alt3: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value.  A MAC CE can activate/indicate one PL offset configuration for each activated joint or UL TCI state. In each joint or UL TCI state, the initial PL offset value is 0dB.

- Alt4: A list of PL offset values is provided in a joint or UL TCI state by RRC. Each PL offset value is applied to a corresponding measured PL range.

Other alternatives are not precluded.

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, study whether and how to indicate TPC command for SRS CLPC adjustment states through DCI format 1_1 and/or 0_1 when the UE is configured two SRS CLPC adjustment states.

RAN1#116 (2024-02)

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset:

- When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

- Note: it does not intend to increase the number of maintained PLs per cell.

- FFS: whether to support associating joint TCI state (if supported) with a PL offset.

Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too.

- FFS: how to determine the Tx beam of PRACH towards UL TRP

- Note: this does not imply to support 2 TA for single-DCI based system.

Agreement

To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support configuring two closed-loop PC adjustment states for SRS in one CC, both of which are separate from that of the PUSCH.

Agreement

Study how to indicate TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured two SRS CLPC adjustment states, down-select from the following options:

- Option 1: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA;

- Option 2: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeB;

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

- Option 4: enhance DCI format 1_1 and/or 0_1 to indicate TPC for SRS CLPC adjustment states

- Option 5: enhance the legacy DCI format 2_3 by introducing a new Type for higher layer parameter srs-TPC-PDCCH-Group

- Option 6: new DCI format to indicate TPC for SRS CLPC adjustment states

- Other options are not precluded.

For the Options1, 2, 3 and 5, consider at least the following Alts as possible examples:

- Alt1: In DCI format 2_3, add one additional TPC command for each CC configured with two SRS CLPC adjustment states,

  - the first TPC command is associated with the first SRS CLPC adjustment state and the second TPC command is associated with the second SRS CLPC adjustment state.

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

- Alt3: use two different TPC-SRS-RNTIs for DCI format 2_3:

  - DCI format 2_3 with CRC scrambled with the first TPC-SRS-RNTI and the second TPC-SRS-RNTI indicates the TPC command for the first and second SRS CLPC adjustment state, respectively.

- Alt4: Implicit method:

Agreement

To support two SRS CLPC adjustment states, study and possibly down-select at least one from the following Alts:

- Alt1: SRS CLPC adjustment state is associated with SRS resource set

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 in the TCI state indicates one of the SRS CLPC adjustment states

- Alt3: Add one extra parameter in P0AlphaSet-r17 of TCI state to indicate one of those two SRS CLPC adjustment states

- Alt4: SRS CLPC adjustment state is associated with SRS resource usage type

Note: Other alternatives are not precluded

Agreement

Down-select one from the following alternatives:

- Alt1: Use only RRC to update the PL offset associated with the UL TCI state

- Alt2: In addition to RRC, MAC-CE can be used to update the PL offset associated with the UL TCI state

  - FFS: Details on MAC CE

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2.

- Joint TCI state mode can be configured at least for FR1