3GPP TSG-RAN WG4 Meeting #112										                    R4-2413973

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.5.4

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

Document for:	Approval

Sub-topic 1: RRM requirements impact

<Agreement> Issue 1-1: MRTD and MTTD for Type 4 capable UE in non-collocated scenario

Agreement:

  - R18 MRTD/MTTD requirements defined for Type 2 UE in non-collocated scenarios can be reused for Type 4 capable UE in non-collocated scenarios, that is

-	non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 4 UE,

•	MRTD=33us (Table 7.6.2.1-1)

•	MTTD=35.21us (Table 7.5.2.1-1)

-	 non-collocated FR1 intra-band non-contiguous NR-CA for Type 4 UE,

•	MRTD=33us (Table 7.6.4-2)

•	MTTD=34.6us (Table 7.5.4-1)

<Agreement> Issue 1-2: MRTD and MTTD for Type 4 capable UE in collocated scenario

Agreement:

  - Assuming the network signaling indicating the collocated and non-collcoated conditions will be defined depending on RF progress, R18 MRTD/MTTD requirements defined for Type 2 UE in collocated condition can be reused for Type 4 capable UE in collocated condition, that is

-	for Type 4 UE non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands in collocated condition,

•	MRTD=3us (Table 7.6.3-1)

•	MTTD=5.21us (Table 7.5.3-1)

-	 for Type 4 UE non-collocated FR1 intra-band non-contiguous NR-CA in collocated condition,

•	MRTD=3us (Table 7.6.4-1)

<Way Forward> Issue 1-3: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

- Proposals

  - Option 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

<Way Forward> Issue 1-4: Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE

- Proposals

  - Option 1(Apple, Nokia, Huawei, ZTE, Ericsson, Samsung, KDDI): The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE

- Interruption

- SCell activation delay

- Scheduling availability

- SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection)

<Agreement> Issue 1-5: Interruption requirements for Type 4 capable UE when UE operates with separate RF chain in non-collocated scenario

Agreement:

  - When Type 4 capable UE operates in non-collocated condition, interruption requirements (below listed) defined for type 2 UE in non-collocated condition in R18 can be applied

- Interruption at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions during measurements on deactivated SCC

<Way Forward> Issue 1-6: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated scenario

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied

- Interruption at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions during measurements on deactivated SCC

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

<Agreement> Issue 1-7: SCell activation delay requirements for Type 4 capable UE when UE operates with separate RF chain in non-collocated condition

Agreement:

  - When Type 4 capable UE operates in non-collocated condition, SCell activation delay defined for type 2 UE in non-collocated condition in R18 can be applied.

<Way Forward> Issue 1-8: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): When Type 4 capable UE operates with shared RF chain in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

<Agreement> Issue 1-9: Scheduling availability for Type 4 capable UE when UE operates with separate RF chain in non-collocated condition

Agreement:

  - When type 4 capable UE operates in non-collocated condition, scheduling availability defined for type 2 UE in non-collocated condition in R18 can be applied

<Way Forward> Issue 1-10: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): When type 4 capable UE operates with shared RF chain in collocated condition, scheduling availability defined for type 2 UE in collocated condition in R18 can be applied

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

<Way Forward> Issue 1-11: SCell BFD/CBD requirements for Type 4 capable UE

- Proposals

  - Option 1(Apple, Huawei, ZTE): Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band.