3GPP TSG-RAN WG4 Meeting #112bis 					R4-2415379
Hefei, Anhui, China, 14th – 18th October 2024

Agenda item		6.9.3

Source:			Intelsat, SKY Perfect JSAT

Title:			Analysis of NTN channel bandwidths for optimal

efficiency

Document for:	Discussion and decision

# Introduction

At RAN #104, the WID in [1] was approved to commence work to introduce the Ku band to the 3GPP specifications for NTN. The purpose of this contribution is to introduce 10MHz FR1 option to the list of available bandwidths to the satellite operators.  The current NTN proposed FR1 channel bandwidth are 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz, and 100 MHz

Satellite operators wishing to re-use their legacy transparent architecture for 5G services, must re-use the existing transponders. It is therefore essential to use the bandwidth of the existing transponders as efficiently as possible. For example, Intelsat operates the largest integrated space and terrestrial network in the world, including satellites from all major satellite manufactures.  Due to the large and diverse space assets, the satellite transponder bandwidth are not fixed but vary depending on the type of satellite, namely: legacy, EPIC, or software defined satellites (SDS).

The discussion presented in this contribution will illustrate how the addition of 10MHz bandwidth option of FR1 could efficiently re-use the bandwidths of the legacy transponders, with minimum bandwidth wastage. In addition, this document will compare the bandwidth wastage when using the channel bandwidths of FR1 and FR2, considering which channel width allows for more efficient channel utilization.

# Analysis and Discussions

In this analysis, the most practical implementation mode was considered. In this scenario, the 5G Bandwidth was fitted into a legacy transponder bandwidth with no overlap. In this analysis the guard bands of both legacy transponder and 5G was also considered. See following Figure 1.  For this example, the transponder bandwidth is 85 MHz, the 5G channel bandwidth that fits is 70 MHz, and the bandwidth wastage is 15 MHz.

Figure 1: 5G bandwidth fitted into legacy transponder bandwidth.

To proceed with the analysis, the resource blocks, payload bandwidths(actual utilized bandwidth by a number of resource blocks), and guard-band of FR1 and FR2 options are calculated and are illustrated in following Figure 2.

With reference to the Figure 2, FR1 or FR2 bandwidth are comprised of one actual utilised payload bandwidth and two guard-bands.

Figure 2: Resource blocks, payload bandwidths and guard-band of FR1 and FR2 Options.

For this analysis several legacy and newer bandwidths were selected, see following Figure 3 [2].

With reference to Figure 3, it can be observed that by utilising additional 10MHz bandwidth the bandwidth wastage of a number of legacy transponder bandwidths are reduced.  The 8 MHz transponder bandwidth is not supported because its lower than the minimum 5G channel bandwidth.

Figure 3: Legacy transponder bandwidths.

The bandwidth savings, by utilising 10MHz bandwidth is also illustrated in following Figure 4.

Figure 4: Legacy transponder bandwidth utilisation by utilising 10 MHz bandwidth.

Further analysis of the bandwidth utilisation reveals that by adding 10 MHz bandwidth option, the maximum unused bandwidth is reduced by a factor of two. See following Figure 5.  The 10 MHz 5G channel bandwidth also supports transponder bandwidth above 10 MHz instead of 20 MHz.

Figure 5: Unused bandwidth reduction across various transponder bandwidths with addition of 10 MHz bandwidth.

Observation 1:  The addition of 10 MHz channel bandwidth (10, 20, 25, 35, 50, 70, 100 MHz) result in lower unused bandwidth (and higher transponder bandwidth efficiency).

Observation 2:  The addition of 10 MHz channel bandwidth reduces the maximum unused bandwidth by factor of two.

As a way of comparison, the following Figure 6, illustrates legacy bandwidth channel utilisation, if FR2 Numerology was used.  The transponder bandwidths of 8 MHz, 27 MHz, and 36 MHz cannot be supported with FR2 channel bandwidths.

|  | FR2 channel definition (MHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| transponder bandwidth (MHz) | 50 | 100 | 200 | 400 | Wasted bandwidth (MHz) |  |
| 8 |  |  |  |  | N/A |  |
| 27 |  |  |  |  | N/A |  |
| 36 |  |  |  |  | N/A |  |
| 54 | 1 |  |  |  | 4 |  |
| 68 | 1 |  |  |  | 18 |  |
| 72 | 1 |  |  |  | 22 |  |
| 108 |  | 1 |  |  | 8 |  |
| 119 |  | 1 |  |  | 19 |  |
| 125 |  | 1 |  |  | 25 |  |
| 144 |  | 1 |  |  | 44 |  |
| 216 |  |  | 1 |  | 16 |  |
| 250 | 1 |  | 1 |  | 0 |  |
| 500 |  | 1 |  | 1 | 0 |  |

Figure 6 Bandwidth utilisation using FR2 Numerology

As it can be observed from Figure 6, the bandwidth wastage is significantly more in all cases for FR2.

Observation 3:  FR1 channel bandwidth is more efficient compared to FR2 channel bandwidth.

Proposal 1:  In order to reduce bandwidth wastage, FR1 channel bandwidth should be expanded to include 10 MHz channel bandwidth (10, 20, 25, 35, 50, 70, 100 MHz).

In the contribution from the previous meeting [2], JSAT explained the channel allocation of legacy satellites and concluded that the channel width of FR1, similar to the discussions in this document, allows for more efficient use of legacy satellites. Furthermore, based on the findings in this document, in evaluating efficient channel utilization to determine the feasibility of FR1 and FR2, RAN4 should consider that FR1 reduces the unavailable bandwidth compared to FR2, allowing for more efficient use.

Proposal 2: In evaluating efficient channel utilization to determine the feasibility of FR1 and FR2, RAN4 should consider that FR1 reduces the unavailable bandwidth compared to FR2, allowing for more efficient use.

# Conclusion

The conclusions that can be drawn from the presented analysis are as follows:

Observation 1:  The addition of 10 MHz channel bandwidth (10, 20, 25, 35, 50, 70, 100 MHz) result in lower unused bandwidth (and higher transponder bandwidth efficiency).

Observation 2:  The addition of 10 MHz channel bandwidth reduces the maximum unused bandwidth by factor of two.

Observation 3:  FR1 channel bandwidth is more efficient compared to FR2 channel bandwidth.

Proposal 1:  In order to reduce bandwidth wastage, FR1 channel bandwidth should be expanded to include 10 MHz channel bandwidth (10, 20, 25, 35, 50, 70, 100 MHz).

Proposal 2: In evaluating efficient channel utilization to determine the feasibility of FR1 and FR2, RAN4 should consider that FR1 reduces the unavailable bandwidth compared to FR2, allowing for more efficient use.

# References

[1]	RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104.

[2]   R4-2413458Ku Band Channel Allocation for Legacy Satellite System; SKY Perfect JSAT.