3GPP TSG-RAN WG4 Meeting #112bis	R4-2415559

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on testability for LP-WUR

Source:	Samsung

Agenda item:	6.23.2.4

Document for:	Discussion

1.	Introduction

Testability is one of the scope for this work item but no much progress yet, the way forward of last meeting is as following:

Sub-topic 4-1 Testability for UE RF requirements Issue 4-1-1: General framework on LP-WUS testing Agreements: - RAN4 further discuss the high-level approach on how to achieve LP-WUS performance verification.

Given the performance metric and MDR value is getting stable, it is necessary to further consider the testability aspect, especially test time is also one of the concerned issues when discussing the MDR value. In this contribution, we share our views on LP-WUR testability.

2. 	Discussion

1% MDR has been agreed as performance metric for evaluation and tentatively also for core requirements.

Issue 1-1-2: Performance metric for Rx RF requirements Agreement: - 1% MDR for evaluation [and core requirements] - FFS on the testability issue. Issue 1-1-3: Performance metric for Demodulation requirements Agreement: - [1% or 0.1%] FAR as assumption for MDR result calibration.

Due to test time consideration, it is still open if the same MDR value will be used in conformance test or not, e.g., 1% MDR as RF core requirement but 5% or 10% MDR used in conformance test corresponding to a scaled requirement

Issue 2-2-9: MDR value for REFSENS test case Agreements: - FFS the MDR for conformance testing should be aligned with the value for RF requirement.

The intention is good but the mapping between different MDR value and REFSENS value may not be robust enough to guarantee the equivalence. The mapping relationship may varies depending on UE implementations and hence there would not be a unified mapping between MDR and REFSENS.

Observation 1:	the mapping between different MDR value and REFSENS value may not be robust enough to guarantee the equivalence for all UEs.

In addition, the MR wakeup can be triggered not only by LP-WUS signal but also by other side conditions. In RF test cases it is expected that the MR is only waken up by LP-WUS signal. So adopting test mode for LP-WUR RF testing not only saves test time, but also disable the other possibilities of MR wakeup except by LP-WUS signal.

Observation 2:	adopting test mode for LP-WUR RF testing not only saves test time, but also disable the other possibilities of MR wakeup except by LP-WUS signal.

Moreover, the REFSENS test cases of LP-WUR is likely to be more than REFSENS of NR, e.g. REFSENS of NR is only conducted under RRC connected mode, but REFSENS of LP-WUR is probably conducted in both connected mode and idle mode. So test time consideration for LP-WUR testing is important.

Based on above discussions, we have the following proposals as the high-level approach on how to achieve LP-WUS performance verification

Proposal 1:	the MDR value for conformance testing should be aligned with the value for RF requirement.

Proposal 2:	RAN4 to adopt test mode for LP-WUR RF testing for the sake of test time saving and test robustness.

3. 	Conclusion

Observation 1:	the mapping between different MDR value and REFSENS value may not be robust enough to guarantee the equivalence for all UEs.

Observation 2:	adopting test mode for LP-WUR RF testing not only saves test time, but also disable the other possibilities of MR wakeup except by LP-WUS signal.

Proposal 1:	the MDR value for conformance testing should be aligned with the value for RF requirement.

Proposal 2:	RAN4 to adopt test mode for LP-WUR RF testing for the sake of test time saving and test robustness.

4. 	References

- R4-2414309    “WF on LP-WUS UE RF requirements”, vivo