3GPP TSG-RAN WG4 Meeting #112bis	R4-2415630

# Hefei, China, 14 - 18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0759 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction of RF requirements for FR2 power class 8 [FR2_PC8] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New power class 8 with similar UE RF architecture as RedCap UE but with support of larger channel bandwidth than 100MHz, CA and UL-MIMO is to be introduced, related RF requirements need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced related RF requirements for the new FR2 power class. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for the new power class will still be missing for the non-RedCap UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.0, 6.2.1.7, 6.2A.2.8(New), 6.2A.3.3.7(New), 6.2A.3.4.7(New), 6.2D.1.7(New), 6.2D.2.7(New), 6.2D.3.7(New), 6.3.1.4, 6.3D.1.4(New), 6.4.2.1, 6.4A.2.2.8(New), 6.4A.2.3.8(New), 7.3.2.7, 7.3.4.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

6.2.1.0	General

NOTE:	Power classes are specified based on the assumption of certain UE types with specific device architectures. The UE types can be found in Table 6.2.1.0-1.

Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |
| 8 | Low power non-RedCap UE |
| Note: Any power class can be used for Redcap type devices as long as the device can meet the core requirements that are applicable to Redcap devices as defined in clause 4.2.21.1 from TS38.306 [14] |  |

Power class 3 is default power class.

<END OF THE CHANGE 1>

<START OF THE CHANGE 2>

6.2.1.8	UE maximum output power for power class 8

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). In case of initial access and RRC_INACTIVE, the cumulative period of measurement shall equal or exceed 1ms. The minimum output power values for EIRP are found in Table 6.2.1.8-1. The requirement is verified with the test metric of total component of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.8-1: UE minimum peak EIRP for power class 8

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 16.4 |
| n258 | 16.4 |
| n261 | 16.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: Minimum peak EIRP does not apply to initial access and RRC_INACTIVE. |  |

The maximum output power values for TRP and EIRP are found on the Table 6.2.1.8-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and the total component of EIRP (Link=TX beam peak direction, Meas=Link angle.

Table 6.2.1.8-2: UE maximum output power limits for power class 8

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.8-3 below. The requirement is verified with the test metric of the total component of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.8-3: UE spherical coverage for power class 8

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 5.5 |
| n258 | 5.5 |
| n261 | 5.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance in RRC_CONNECTED. NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 3: Minimum EIRP at 50%-tile CDF is defined as the lower limit minus 2 dB in initial access and RRC_INACTIVE |  |

<END OF THE CHANGE 2>

<START OF THE CHANGE 3>

6.2.2.8	UE maximum output power reduction for power class 8

For power class 8, MPR specified in sub-clause 6.2.2.3 applies.

<END OF THE CHANGE 3>

<START OF THE CHANGE 4>

6.2.3.3.8	A-MPR for NS_202 for power class 8

For power class 8, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

<END OF THE CHANGE 4>

<START OF THE CHANGE 5>

6.2.3.4.8	A-MPR for NS_203 for power class 8

For power class 8, AMPR for NS_203 specified in subclause 6.2.3.4.3 applies.

<END OF THE CHANGE 5>

<START OF THE CHANGE 6>

6.2A.2.8	Maximum output power reduction for power class 8

For power class 8, MPR specified in sub-clause 6.2A.2.4.1 applies for intra-band contiguous UL CA.

<END OF THE CHANGE 6>

<START OF THE CHANGE 7>

6.2A.3.3.7	A-MPR for CA_NS_202 for power class 8

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

<END OF THE CHANGE 7>

<START OF THE CHANGE 8>

6.2A.3.4.7	A-MPR for CA_NS_203 for power class 8

For intra-band contiguous CA, A-MPR specified in sub-clause 6.2A.3.4.3 applies.

<END OF THE CHANGE 8>

<START OF THE CHANGE 9>

6.2D.1.7	UE maximum output power for UL MIMO for power class 8

The following requirements define the maximum output power radiated by the PC8 UE. Requirements apply to UEs when configured for 2-layer transmission as well as when configured for single layer uplink full power transmission (ULFPTx), with configuration per clause 6.2D.1.0.

The minimum peak EIRP requirements are found in Table 6.2D.1.7-1 below. The period of measurement shall be at least one sub frame (1 ms). The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.7-1: UE minimum peak EIRP for UL MIMO for power class 8

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 16.4 |
| n258 | 16.4 |
| n261 | 16.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance. NOTE 2: Min Peak EIRP refers to the total EIRP for the UL beams peaks. |  |

The maximum output power values for TRP and EIRP are found in Table 6.2D.1.7-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2D.1.7-2: UE maximum output power limits for UL MIMO for power class 8

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2D.1.7-3 below. The requirement is verified with the test metric of EIRP (Link=spherical coverage grid, Meas=Link angle).

Table 6.2D.1.7-3: UE spherical coverage for UL MIMO for power class 8

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 5.5 |
| n258 | 5.5 |
| n261 | 5.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance |  |

<END OF THE CHANGE 9>

<START OF THE CHANGE 10>

6.2D.2.7	UE maximum output power reduction for modulation / channel bandwidth for UL MIMO for power class 8

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1.7-1 is specified in sub-clause 6.2.2.3. The requirements shall be met with configurations specified in sub-clause 6.2D.1.0.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

<END OF THE CHANGE 10>

<START OF THE CHANGE 11>

6.2D.3.7	UE maximum output power reduction with additional requirements for UL MIMO for power class 8

For UEs configured for 2-layer transmission as well as UEs configured for single layer uplink full power transmission (ULFPTx), the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1.7-1. The requirements shall be met with the configurations specified in clause 6.2D.1.0.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2D.4 apply.

<END OF THE CHANGE 11>

<START OF THE CHANGE 12>

6.3.1.5	Minimum output power for power class 8

The minimum output power shall not exceed the values specified in Table 6.3.1.4-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3.1.5-1: Minimum output power for power class 8

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n261 | 50 | -13 | 47.58 |
|  | 100 | -13 | 95.04 |
|  | 200 | -13 | 190.08 |
|  | 400 | -13 | 380.16 |

<END OF THE CHANGE 12>

<START OF THE CHANGE 13>

6.3A.1.4	Minimum output power for power class 8

For intra-band contiguous carrier aggregation, the minimum output power shall not exceed the values specified in Table 6.3A.1.4-1 for each operating band supported. The minimum power is verified in beam locked mode with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.3A.1.4-1: Minimum output power for CA for power class 7

| Operating band | Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| n257, n258, n261 | 50 | -13 | 47.52 |
|  | 100 | -13 | 95.04 |
|  | 200 | -13 | 190.08 |
|  | 400 | -13 | 380.16 |

<END OF THE CHANGE 13>

<START OF THE CHANGE 14>

6.3D.1.4	Minimum output power for UL MIMO for power class 8

For UE supporting UL MIMO, the minimum output power shall not exceed the sum of the values specified in Table 6.3.1.5-1 and the quantity 10*log10(Number of Layers).

<END OF THE CHANGE 14>

<START OF THE CHANGE 15>

6.4.2.1	Error vector magnitude

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM, the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further equalised using the channel estimates subjected to the EVM equaliser spectrum flatness requirement specified in sub-clauses 6.4.2.4 and 6.4.2.5. For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and one slot for PUCCH and PUSCH in the time domain. The EVM measurement interval is reduced by any symbols that contains an allowable power transient in the measurement interval as as defined in clause 6.3.3.

The RMS average of the basic EVM measurements over 10 subframes for the average EVM case, and over 60 subframes for the reference signal EVM case, for the different modulation schemes shall not exceed the values specified in Table 6.4.2.1-1 for the parameters defined in Table 6.4.2.1-2 or 6.4.2.1-3, depending on UE power class. For EVM evaluation purposes, all 13 PRACH preamble formats and all 5 PUCCH formats are considered to have the same EVM requirement as QPSK modulated.

The requirement is verified with the test metric of EVM (Link=TX beam peak direction, Meas=Link angle).

Table 6.4.2.1-1: Minimum requirements for error vector magnitude

| Parameter | Unit | Average EVM level | Reference signal EVM level |
| --- | --- | --- | --- |
| Pi/2 BPSK | % | 30.0 | 30.0 |
| QPSK | % | 17.5 | 17.5 |
| 16 QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8.0 | 8.0 |
| 256 QAM1 | % | 3.5 | 3.5 |
| NOTE 1: Refer to clause 6.1 for 256 QAM applicability. |  |  |  |

Table 6.4.2.1-2: Parameters for Error Vector Magnitude for power class 1 in FR2-1

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | 4 |
| UE EIRP for UL 16 QAM | dBm | 7 |
| UE EIRP for UL 64 QAM | dBm | 11 |
| UE EIRP for UL 256 QAM | dBm | 18 |
| Operating conditions |  | Normal conditions |

Table 6.4.2.1-2a: Parameters for Error Vector Magnitude for power class 1 in FR2-2

|  |  | Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| UE EIRP | dBm | 4 | 2 | 5 | 8 | 9 |
| UE EIRP for UL 16 QAM | dBm | 7 | 5 | 8 | 11 | 12 |
| UE EIRP for UL 64 QAM | dBm | 11 | 9 | 12 | 15 | 16 |
| Operating conditions | Normal Conditions |  |  |  |  |  |
| NOTE 1: PTRS is configured for 16 QAM and 64 QAM |  |  |  |  |  |  |

Table 6.4.2.1-3: Parameters for Error Vector Magnitude for power class 2, 3, 4, 7 and 8 in FR2-1

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | -13 |
| UE EIRP for UL 16 QAM | dBm | -10 |
| UE EIRP for UL 64 QAM | dBm | -6 |
| UE EIRP for UL 256 QAM1 | dBm | 1 |
| Operating conditions |  | Normal conditions |
| NOTE 1: Refer to clause 6.1 for 256 QAM applicability. |  |  |

Table 6.4.2.1-3a: Parameters for Error Vector Magnitude for power class 3 in FR2-2

|  |  | Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| UE EIRP | dBm | -13 | -11 | -8 | -5 | -4 |
| UE EIRP for UL 16 QAM | dBm | -10 | -8 | -5 | -2 | -1 |
| UE EIRP for UL 64 QAM | dBm | -6 | -4 | -1 | 2 | 3 |
| Operating conditions | Normal Conditions |  |  |  |  |  |
| NOTE 1: PTRS is configured for 16 QAM and 64 QAM |  |  |  |  |  |  |

Table 6.4.2.1-3b: Parameters for Error Vector Magnitude for power class 2 in FR2-2

|  |  | Level |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| UE EIRP | dBm | -13 | -11 | -8 | -5 | -4 |
| UE EIRP for UL 16 QAM | dBm | -10 | -8 | -5 | -2 | -1 |
| UE EIRP for UL 64 QAM | dBm | -6 | -4 | -1 | 2 | 3 |
| Operating conditions | Normal Conditions |  |  |  |  |  |
| NOTE 1: PTRS is configured for 16 QAM and 64 QAM |  |  |  |  |  |  |

Table 6.4.2.1-4: Parameters for Error Vector Magnitude for power class 5 in FR2-1

| Parameter | Unit | Level |
| --- | --- | --- |
| UE EIRP | dBm | -6 |
| UE EIRP for UL 16 QAM | dBm | -3 |
| UE EIRP for UL 64 QAM | dBm | 1 |
| UE EIRP for UL 256 QAM | dBm | 8 |
| Operating conditions |  | Normal conditions |

<END OF THE CHANGE 15>

<START OF THE CHANGE 16>

6.4.2.2	Carrier leakage

<Unchanged Sections Skipped>

6.4.2.2.9	Carrier leakage for power class 8

When carrier leakage is contained inside the spectrum occupied by the configured UL CCs and DL CCs, the relative carrier leakage power specified in subclause 6.4.2.2.4 applies.

<END OF THE CHANGE 16>

<START OF THE CHANGE 17>

6.4.2.3.9	In-band emissions for power class 8

The average of the in-band emission specified in subclause 6.4.2.3.4 applies.

<END OF THE CHANGE 17>

<START OF THE CHANGE 18>

6.4A.2.2.8	Carrier leakage for power class 8

For intra-band contiguous carrier aggregation, when carrier leakage is contained inside the spectrum occupied by all configured UL and DL CCs, the relative carrier leakage power shall not exceed the values specified in Table 6.4A.2.2.8-1 for power class 8 UEs.

Table 6.4A.2.2.8-1: Minimum requirements for relative carrier leakage power class 8 in FR2-1

| Parameters | Relative limit (dBc) |
| --- | --- |
| Output power > 0 dBm | -25 |
| -13 dBm ≤ Output power EIRP ≤ 0 dBm | -20 |

<END OF THE CHANGE 18>

<START OF THE CHANGE 19>

6.4A.2.3.8	Inband emissions for power class 8

For intra-band contiguous carrier aggregation, the average of the in-band emission specified in clause 6.4A.2.3.4applies

<END OF THE CHANGE 19>

<START OF THE CHANGE 20>

6.6.9	Beam correspondence for power class 8

6.6.9.1	General

The beam correspondence requirement for power class 8 UEs consists of two components: UE minimum peak EIRP (as defined in Clause 6.2.1.8), and UE spherical coverage (as defined in Clause 6.2.1.8).

Power class 8 UE shall mandatorily support IE beamCorrespondenceWithoutUL-BeamSweeping, as defined in TS 38.306 [14]. The UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.8-1 and spherical coverage requirement according to Table 6.2.1.8-3.

-	If beamCorrespondenceSSB-based-r16 is supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.8-1 and spherical coverage requirement according to Table 6.2.1.8-3 using the side conditions for SSB based enhanced beam correspondence requirements as defined in Clause 6.6.8.3.2.

-	If beamCorrespondenceCSI-RS-based-r16 is supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.8-1 and spherical coverage requirement according to Table 6.2.1.8-3 using the side conditions for CSI-RS based enhanced beam correspondence requirements as defined in Clause 6.6.8.3.3.

The beam correspondence requirement for power class 8 UEs in initial access and in RRC_INACTIVE consists of UE spherical coverage only:

-	UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.8-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.8.3.4.

<END OF THE CHANGE 20>

<START OF THE CHANGE 21>

7.3.2.8	Reference sensitivity power level for power class 8

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 7.3.2.8-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 7.3.2.8-1: Reference sensitivity

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -85.3 | -82.3 | -79.3 | -76.3 |
| n258 | -85.3 | -82.3 | -79.3 | -76.3 |
| n261 | -85.3 | -82.3 | -79.3 | -76.3 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth of 50MHz,100MHz, 200MHz and 400MHz specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

<END OF THE CHANGE 21>

<START OF THE CHANGE 22>

7.3.4.8	EIS spherical coverage for power class 8

The reference measurement channels and throughput criterion shall be as specified in clause 7.3.2.8.

The maximum EIS at the 50th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.8-1 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.8-1: EIS spherical coverage for power class 8

| Operating band | EIS at 50th %-tile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -74.4 | -71.4 | -68.4 | -65.4 |
| n258 | -74.4 | -71.4 | -68.4 | -65.4 |
| n259 | -74.4 | -71.4 | -68.4 | -65.4 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in Annex E.2.1. |  |  |  |  |

The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth of 50MHz, 100MHz and 200MHz specified in Table 7.3.2.1-2.

Unless given by Table 7.3.2.1-3, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3-1) configured.

<END OF THE CHANGE 22>