3GPP TSG-RAN WG4 Meeting #112bis	R4-2416109

# Hefei, Anhui, October 14 – 18, 2024

Title:	Considerations on grouping rules for inter-band DC configuration tables

Source:	ZTE Corporation, Sanechips

Agenda item:	8.1.1.4

Document for:	Approval

- 1.	Introduction

In current RAN4 specifications, to reduce the DC configuration table size, some guidelines have been made for DC configuration tables. The related rules have been collected in TR 38.846 [1], which specifies how to organize the DC configuration tables with different constituent bands. For the uplink support in the configuration table, the valid uplink configurations are specified that uplink does not have more carriers than downlink. For the UL configuration type, it should be consistent within one row, i.e., there should not be a mixture of contiguous and non-contiguous UL CA within a row. If multiple UL DC configurations are indicated with multiple DL DC configurations, only UL DC configurations with the same or a lower number of carriers in the same fallback group are valid UL configurations. Apart from the above rules for DC configurations, the following rules are also applied to the grouping of the DC configurations.

-	Grouping of DC configurations is based on common band combination.

-	In case E-UTRA or/and NR has non-contiguous CA, it will be on a separate row compared to cases when DC configuration has only single carrier or contiguous CA operation.

-	Common band combination should be considered as the configurations having the same band sequence, such as DC_x-y-y_nz and DC_x-x-y_nz are different band combinations, while all configurations with DC_x-y_nz(*) having non-contiguous parts in band nz are considered as common band combination.

In this proposal, we discuss how the grouping rules for inter-band DC configurations are applied in the current RAN4 specifications. Some following-up Draft CRs for EN-DC configurations within FR1 are provided in [2-5] in this meeting for endorsement. Some other CRs such as EN-DC configurations including FR2 etc. will be provided in the next meeting due to the limited time.

- 2.	Discussion

Based on the above discussion, we have the following observation related to the rules of DC configuration grouping.

Observation 1.  It is observed that in the current RAN4 spec, for the grouping of the DC configurations, the following rules are applied.

-	Grouping of DC configurations is based on common band combination.

-	In case E-UTRA or/and NR has non-contiguous CA, it will be on a separate row compared to cases when DC configuration has only single carrier or contiguous CA operation.

-	Common band combination should be considered as the configurations having the same band sequence, such as DC_x-y-y_nz and DC_x-x-y_nz are different band combinations, while all configurations with DC_x-y_nz(*) having non-contiguous parts in band nz are considered as common band combination.

However, while looking at the current spec in TS 38.101-3 for EN-DC configurations, the implementation of some EN-DC configuration grouping seems quite confusing, some of which are even contradictory. For example,

Example 1 (EN-DC configurations with different contiguous group).

According to the rules in TR 38.846, all DC configurations having the common component bands should be divided into two different categories, i.e., the contiguous group and the non-contiguous group. For the DC configurations with only contiguous CA parts, if the E-UTRA CA part and/or NR CA part have the same common constituent bands, they should be categorized into the same group.

In the below EN-DC configurations for DC_3-7_n26-n78, the configurations in yellow (DC_3A-7A_n26A-n78A, DC_3A-7C_n26A-n78A, DC_3C-7A_n26A-n78A and DC_3C-7C_n26A-n78A) are specified in different rows in the current spec which should be categorized into one group based on the grouping rules in TR 38.846.

| DC_3A-7A_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_3A_n26A DC_7A_n26A |
| --- | --- |
| DC_3C-7A-26A_n78(2A) DC_3C-7C-26A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7C_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_7A_n26A DC_7C_n26A |
| DC_3C-7A_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A |
| DC_3C-7C_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |

Observation 2.  It is observed that in the current RAN4 spec, some DC configurations having the same constituent bands and with only contiguous CA parts in the configurations have been specified into different groups which should be corrected to the same contiguous group.

Example 2 (EN-DC configurations with same non-contiguous group as contiguous group).

According to the rules in TR 38.846, for the DC configurations with non-contiguous CA parts, if the E-UTRA CA part and/or NR CA part have the same common constituent bands, they should be categorized into the different groups as the configurations with only contiguous CA parts.

However, some EN-DC configurations fail to comply with the above guidelines. For example, DC_1A-3A_n8A-n77A and DC_1A-3A_n8A-n77(2A) have the same constituent bands, but they should be moved to different rows

| DC_1A-3A_n8A-n77A DC_1A-3A_n8A-n77(2A) | DC_1A_n8A DC_1A_n77A DC_3A_n8A DC_3A_n77A |
| --- | --- |

Observation 3.  It is observed that in the current RAN4 spec, some DC configurations with non-contiguous CA parts have been specified into the same group as DC configurations with only contiguous CA parts. They should be corrected to different groups.

Example 3 (EN-DC configurations with different constitute bands in the same group).

According to the rules in TR 38.846, all the DC configurations in the same group should have the “common band combination” which means the constituent bands having the same band sequence. However, in the below example for DC_2A-66B_n66A, it is specified into the same group as for DC_2A-66A-66A_n66A in which the constituent bands having no common band sequence, i.e. one for DC_2-66_n66 and the other for DC_2-66-66_n66.

| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| --- | --- |
| DC_2A-(n)66AA | DC_2A_n66A DC_(n)66AA2 |
| DC_2A-2A-(n)66AA | DC_2A_n66A DC_(n)66AA2 |
| DC_2A-66A-66A_n66A DC_2A-66B_n66A | DC_2A_n66A DC_66A_n66A2 |

Observation 4.  It is observed that in the current RAN4 spec, some DC configurations in the same group do not have the “common band combination” basis, which means the constituent bands in the same group having the different band sequence.

Based on the above observations, we see that in the current RAN4 spec, some EN-DC configurations do not comply with the grouping guidelines specified in TR 38.846 which should be corrected.

Proposal 1.  It is suggested to regroup the inter-band DC configurations with the guidelines in Observation 1 and take the following aspects into consideration.

-	To categorize DC configurations having only contiguous CA parts into one group.

-	To differentiate the DC configurations having non-contiguous CA parts with the DC configurations having only contiguous CA parts.

-	To group the DC configurations on the basis of “common band combination”, i.e. the constituent bands having the same band sequence.

Proposal 2.  It is suggested to endorse the Draft CRs in [2-4] for regrouping the inter-band EN-DC configurations within FR1.

- 3. Conclusion

In this paper, we discuss the issues about the grouping of inter-band DC configurations in RAN4 specifications. Some examples in the current specifications that do not comply with the grouping guidelines specified in TR 38.846 have been proposed. The following observations and proposals are provided.

Observation 1.  It is observed that in the current RAN4 spec, for the grouping of the DC configurations, the following rules are applied.

-	Grouping of DC configurations is based on common band combination.

-	In case E-UTRA or/and NR has non-contiguous CA, it will be on a separate row compared to cases when DC configuration has only single carrier or contiguous CA operation.

-	Common band combination should be considered as the configurations having the same band sequence, such as DC_x-y-y_nz and DC_x-x-y_nz are different band combinations, while all configurations with DC_x-y_nz(*) having non-contiguous parts in band nz are considered as common band combination.

Observation 2.  It is observed that in the current RAN4 spec, some DC configurations having the same constituent bands and with only contiguous CA parts in the configurations have been specified into different groups which should be corrected to the same contiguous group.

Observation 3.  It is observed that in the current RAN4 spec, some DC configurations with non-contiguous CA parts have been specified into the same group as DC configurations with only contiguous CA parts. They should be corrected to different groups.

Observation 4.  It is observed that in the current RAN4 spec, some DC configurations in the same group do not have the “common band combination” basis, which means the constituent bands in the same group having the different band sequence.

Proposal 1.  It is suggested to regroup the inter-band DC configurations with the guidelines in Observation 1 and take the following aspects into consideration.

-	To categorize DC configurations having only contiguous CA parts into one group.

-	To differentiate the DC configurations having non-contiguous CA parts with the DC configurations having only contiguous CA parts.

-	To group the DC configurations on the basis of “common band combination”, i.e. the constituent bands having the same band sequence.

Proposal 2.  It is suggested to endorse the Draft CRs in [2-4] for regrouping the inter-band EN-DC configurations within FR1.

# 4. References

- TR 38.846, Study on simplification of band combination specification for NR and LTE.

- R4-2416110, (DC_R18_2BLTE_1BNR_3DL2UL) Draft CR for TS 38.101-3 on 2L and 1N bands EN-DC configuration grouping, ZTE, Sanechips.

- R4-2416111, (DC_R18_xBLTE_1BNR_yDL2UL) Draft CR for TS 38.101-3 on (3-5)L and 1N bands EN-DC configuration grouping, ZTE, Sanechips.

- R4-2416112, (DC_R18_xBLTE_2BNR_yDL2UL) Draft CR for TS 38.101-3 on (1-4)L and 2N bands EN-DC configuration grouping, ZTE, Sanechips.

- R4-2416113, (DC_R18_xBLTE_yBNR_zDL2UL) Draft CR for TS 38.101-3 on (1-3)L and (3-5)N bands EN-DC configuration grouping, ZTE, Sanechips.