3GPP TSG-RAN WG4 Meeting#112bis                             R4-2416236

Hefei, China, 14th – 18th Oct, 2024

Agenda item:	6.10.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for Inter-band CA for ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we would like to share some further views on RF requirements for ATG UE supporting inter-band DL CA.

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A Agreement: - Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input. - The filter with [X] MHz frequency gap can be considered. Issue 3-2-2: BCS for DL CA_n3_n39 Agreement: - Table 1: BCS for CA_n3A-n39A NR CA configuration; Uplink CA configuration or single uplink carrier; NR Band; Channel bandwidth (MHz); Bandwidth combination set. CA_n3A-n39A; -; n3; 5, 10, 15, 20, 25, 30; 0. n39; 5, 10, 15, 20, 25, 30, 35, 40. Rx requirement for inter-band CA_n3A-n39A Issue 3-4-1: delta RIB for NR CA_n3A-n39A could be used as the baseline Agreement: Table 3: ΔRIB,c Inter-band CA combination; ΔRIB,c for NR bands (dB)*. Component band in order of bands in configuration**. CA_n3-n39; -; -. NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77.. Issue 3-4-2: MSD for CA_n3A-n39A Agreement: - There is no MSD issue for n3 impact n39. Issue 3-4-3: OOB exception for CA_n3A-n39A Agreement: - No need to specify OOB blocking exception for CA_n3-n39 since the second order intermodulation product of the n3 UL carrier and the CW interfering signal will not overlaps with the n39 DL carrier.

Discussion

- 2.1. BCS for CA_n3-n39

- During the last RAN4 meeting, there were some discussions on whether n39 UL could be enabled. Considering the DL frequency range of n3 is immediately closed to n39 UL, so it is proposed that only n3 is supported in the uplink by default assumption.

Table 2.2-1:  CA band combination of band n3+n39

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 | – | 1785 | 1805 | – | 1880 | FDD |
| n39 | 1880 | – | 1920 | 1880 | – | 1920 | TDD |

- In addition, during the last RAN4 meeting, it is suggested have some further studies on the dedicated filtering between band n3 DL and n39 UL with some gap considered. The feasibility of this kind of filtering and also its insertion loss is under the evaluation, we propose to postpone the decision for this advanced hardware assumption for it.

- Proposal 1. The BCS for inter-band DL CA_n3A-n39A is proposed in table 1 by default assumption and further study on the feasibility of simultaneous n39 UL transmission and band n3 DL reception.

- Table 1.  Supported bandwidths per CA band combination of band n3+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39AX | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
| [NOTE X: Uplink is only in n3 for CA_n3-n39] |  |  |  |  |

- 2.2. RF requirements for Inter-band CA

In this section, we provide some initial analysis on the overall requirement for ATG UE supporting inter-band DL CA and also provided the summary of proposals for it.

Table 2. Analysis and summary of proposals for ATG UE supporting inter-band DL CA

| The existing RF requirement for inter-band CA | ATG UE supporting inter-band CA |
| --- | --- |
| 6.2A.1.3 UE maximum output power for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.2A.2.3 UE maximum output power reduction for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.2A.3.1.3 UE additional maximum output power reduction for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.2A.4.1.3 Configured transmitted power for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.3A.1.3 Minimum output power for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.3A.2.3 Transmit OFF power for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.3A.3.3 Transmit ON/OFF time mask for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.3A.4.3 Power control for inter-band CA No requirements unique to CA operation are defined. | No impacts, to follow Rel-18 single carrier requirements |
| 6.4A.1.3 Frequency error for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.4A.2.3 Transmit modulation quality for inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.5A.1.3 Occupied bandwidth for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.5A.2.2.3 Spectrum emission mask for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.5A.2.3.3 Additional spectrum emission mask for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 6.5A.3.2.3 Spurious emissions for UE co-existence for Inter-band CA | No impacts, to follow Rel-18 single carrier requirements |
| 7.3A.2.3 Reference sensitivity power level for Inter-band CA | Reference sensitivity power level for Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1, Table 7.3.2-2 and Table 7.3.2-3 modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4. |
| 7.3A.3.2 ΔRIB,c for Inter-band CA | - The delta TIB, delta RIB, MSD for CA_n3-n39 with single uplink n3 transmission captured in TR 38.718-02-01 clause 5.90 could be reused for ATG. - The delta TIB, delta RIB, MSD for CA_n3-n39 captured in TR 38.718-02-01 clause 5.90 could be reused for ATG. |
| 7.4A.3 Maximum input level for Inter-band CA | Maximum input level for Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the maximum input level is defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.4J for each component carrier while all downlink carriers are active. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) for each component carrier. |
| 7.5A.3 Adjacent channel selectivity Inter-band CA | Adjacent channel selectivity Inter-band CA For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the adjacent channel requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The ATG UE shall meet the requirements specified in clause 7.5J, for each component carrier while all downlink carriers are active. The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). |
| 7.6A.2.3 In-band blocking for Inter-band CA | To following the existing requirements defined in Clause 7.6A.2.3. |
| 7.6A.3.3 Out-of-band blocking for Inter-band CA | To following the existing requirements defined in Clause 7.6A.3.3. And no OOB exception for CA_n3-n39 |
| 7.6A.4.3 Narrow band blocking for Inter-band CA | N/A |
| 7.7A.3 Spurious response for Inter-band CA | To following the existing requirements defined in Clause 7.7A.3. |
| 7.8A.2.3 Wide band intermodulation for Inter-band CA | To following the existing requirements defined in Clause 7.8A.2.3. |
| 7.9A.3 Spurious emissions for Inter-band CA | To following the existing requirements defined in Clause 7.9A.3 |

Conclusions

In this contribution, we shared some initial views on the RF requirements for ATG UE supporting inter-band DL CA in Rel-19 and proposals are made as following:

- Proposal 1. The BCS for inter-band DL CA_n3A-n39A is proposed in table 1 by default assumption and further study on the feasibility of simultaneous n39 UL transmission and band n3 DL reception.

- Table 1. Supported bandwidths per CA band combination of band n3+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39AX | - | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
| [NOTE X: Uplink is only in n3 for CA_n3-n39] |  |  |  |  |

Proposal 2: for RF requirements for inter-band DL CA, refer to the proposals in table 2.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.

- R4-2410687,WF on Rel-19 ATG UE RF requirements, Approved.

- TR 38.718-02-01