3GPP TSG-RAN WG4 Meeting #112bis	R4-2416465

# Hefei, China, 14 – 18 October 2024

Agenda item:		6.8.3.2

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for NTN less than 5 MHz

Document for:		Approval

## 1	Introduction

3 MHz channel bandwidth was specified for TN in rel-18. In the work item approved for NTN, 3 MHz channel bandwidth is to be added with RAN1-design based on rel-18, with no expected RAN1 impact. In this contribution UE RF requirements for 3 MHz channel bandwidth are discussed.

## 2	Discussion

The agreements reached in RAN4#112 are captured in [1]. This document addresses multiple requirements which were agreed to be further studied.

SEM, general spurious emissions and MPR

In [1] SEM and out-of-band boundary were left open. SEM and general spurious emissions are general limits that apply for all operating bands. Given that the waveforms for NTN and TN variants of NR are the same and challenges to meet the emission requirements remain the same, it is suggested to apply the same SEM and out of band boundary for 3 MHz channel bandwidth if NTN as already specified for TN.

Proposal 1: Adopt the SEM mask as shown in Table 1 with out-of-band boundary of 6 MHz, i.e. align the requirements between TN and NTN for 3 MHz channel bandwidth.

Table 1: SEM mask for 3 MHz channel bandwidth

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 |  |
| ± 0-1 | -13 | 1 % of channel BW |
| ± 1-5 | -10 | 1 MHz |
| ± 5-6 | -25 |  |

Given that the SEM and general spurious emissions requirements are aligned with TN operation, the required MPR can can also be the same. For example minimum RF impairment levels such as DC leakage and IQ-image levels are the same in TN and NTN specifications. We also assume PA calibration point can be assumed to be the same, and therefore the backoff required to meet the general requirements is the same between TN and NTN. No specification update is needed, as TS 38.101-5 already refers to TS 38.101-1 for MPR.

Proposal 2: Same MPR applies for 3 MHz CBW in TN and NTN. No specification update is needed.

A-MPR

Initial evaluations for AMPR were done. The following assumption were used in simulations

- PA calibrated to meet 30 dBc ACLR 1 dB MPR in 20 MHz channel and 100 RB DFT-s-OFDM signal

- DC-leakage and IQ-image set to -28 dBc

- CIM3 of -60 dBc

NS_01

NS_01 is the general NS with no additional emission requirements. No A-MPR is needed

NS_100

NS_100 can be signaled for protection of UTRA services. NS_100 applies for band n256, given that n256 is immediately adjacent to UTRA band I. In TS 38.101-1, NS_100 and 3 MHz channel bandwidth are both specified for n26. However, we failed to find any 3GPP document which would have analysed the A-MPR need.

UTRA ACLR is specified for LTE, with no A-MPR allowed. Therefore, it is safe to conclude at least DFT-s-OFDM waveforms can been UTRA ACLR without additional power backoff. With CP-OFDM waveforms the general MPR allows more backoff. This backoff is sufficient to meet UTRA ACLR. Therefore no A-MPR is needed for NS_100

NS_02N

NS_02N is specified for n255 with stringent emission requirements below 1610 MHz. No A-MPR is allowed for wider channel bandwidths. With 3 MHz channel bandwidth the emissions will be contained closer to the transmission, and no A-MPR is needed also for 3 MHz.

NS_03N

NS_03N is specified for n254 for operation under FCC regulatory domain. n254 UL is immediately adjacent to the emission limit, and therefore meeting the requirement is hard. A-MPR is specified for 5 MHz and wider channel bandwidths.

Based on the initial evaluations small A-MPR is needed to meet the requirements. The new requirement is highlighted in yellow in below tables. Given that the emission limit is stringent and close to the carrier, the values are provided only as initial once and would be further confirmed until RAN4#113.

Table 6.2.3.2-1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3 MHz | fc < [1613.9] |  | [>= 1.8] | [A3] |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 6.2.3.2-2: A-MPR for NS_03N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
|  | 256QAM | 4.5 | 6 | 4.5 | 8 | 9 | 4 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 256QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

NS_04N

NS_04N is specified for n254 for operation in ETSI regulatory domain. For NS_04 there is a both additional spectrum emission mask as well as additional spurious emission limits both at low and high edge of the operating band. The spurious emission limit above the band is not limiting output power for NS_04N, as NS_04N is intended to be valid only when carrier is contained within 1610 – 1618.25 MHz. Based on our studies the additional SEM mask is not limiting the output power. Therefore, the required A-MPR for 3 MHz is the same for NS_03N.

NS_05N

NS_04N is specified for n254 for operation in ETSI regulatory domain. For NS_05 there is a both additional spectrum emission mask as well as additional spurious emission limits both at low and high edge of the operating band. The spurious emission limit below the band is not limiting output power for NS_05N, as NS_05N is intended to be valid only when carrier is within 1618.25 – 1626.5 MHz. Therefore, simulations were done at the top end of the band center frequency of 1625.0 MHz. No need for A-MPR was observed in the initial evaluations.

NS_24

NS_24 is specified for n256 for protection of band 34. The emission limit is very stringent at -50 dBm/MHz for 2010 to 2025 MHz. The requirement applies only when there is at least 5 MHz offset from the upper edge of the channel bandwidth to the protected frequencies. Therefore, 2003.5 MHz was used as center frequency for the initial evaluation. It seems A-MPR will be needed for DFT-s-OFDM Pi/2 BPSK.

When NS_24 was originally defined for n65, it was observed that simulated A-MPRs were too optimistic [2]. This is because the simulation model typically captures the PA using a fixed bias voltage, meaning the PA is optimized for best linearity. However, with large backoffs this means that the efficiency is very low. For this reason PA bias is in reality dynamically adjusted during operation. In our view measurements are needed also for 3 MHz CBW to confirm that A-MPR is appropriate.

In addition to potentially optimistic simulations, NS_24 A-MPR went through corrections [3]. It was reported that unless CIM5 is appropriately taken into account, A-MPR for narrow allocations may be too small.

Taking into account these factors, we aim to provide A-MPR proposal for NS_24 in RAN4#113.

Observation 1: Based on initial evaluations NS_01, NS_02N, NS_05N and NS_100 may not need any A-MPR for 3 MHz CBW.

Observation 2: Based on initial evaluations NS_03N and NS_04N need A-MPR at low edge of band n254. Legacy A-MPR A3 can be re-used and a new A-MPR region is needed.

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3 MHz | fc < [1613.9] |  | [>= 1.8] | [A3] |

Observation 3: NS_24 A-MPR needs to confirmed by measurements given expected backoff is large. CIM5 needs to be taken into account.

Proposal 3: Continue A-MPR evaluations at least until RAN4#113 to ensure A-MPR specification are reliable.

Refsens

With 3 MHz channel bandwidth there are 15 resource blocks available and therefore total received power with same PSD is reduced by 2.2 dB compared to 5 MHz channel bandwidth. The reduced total power also means that any impairments have more impact to the signal quality. For NR NTN, maximum input power and in-band blocking interferer signal level are reduced to -40 dBm. This means that NR NTN may not be as well optimized for maximum linearity as receiver for TN bands.

For 3 MHz channel bandwidth, most LTE bands apply 1.7 dB lower power for 3 MHz channel bandwidth than for 5 MHz channel bandwidth. This is because Rx IIP2 effects become more impactful. Rx IIP2 needs to be considered in setting the wanted signal power level.

Observation 4: Rx IIP2 needs to be considered in setting wanted signal level for 3 MHz refsens.

Proposal 4: Define refsens for 3 MHz channel bandwidth as shown in Table 2:

Table 2: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -101.7 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

Optional vs. mandatory support for 3 MHz

NR NTN was originally defined in rel-17 when the narrowest channel bandwidth was 5 MHz. UEs supporting NR NTN based on the rel-17 specifications will not find NTN cells from the new sync raster nor will they support configuration of 3 MHz channel bandwidth. For these reasons, networks will need to operate with the assumption that e.g. initial access would take place with the legacy sync raster, as otherwise some UEs will not be able to connect. Additionally, support for 3 MHz does not add any critical functionality which would be essential for NR NTN system to work.

For these reasons, it is preferable to allow optional support for 3 MHz channel bandwidth like is typical for new features added in later releases. RAN2 has already defined capabilities how UE indicates support of 3 MHz channel bandwidth, and it is possible that no updates in RAN2 are needed.

Proposal 5: Support of 3 MHz channel bandwidth is optional for UE.

## 3	Conclusion

In this contribution UE RF requirements for less than 5 MHz for NTN were discussed. Following proposals were made

Proposal 1: Adopt the SEM mask as shown in Table 1 with out-of-band boundary of 6 MHz, i.e. align the requirements between TN and NTN for 3 MHz channel bandwidth.

Table 1: SEM mask for 3 MHz channel bandwidth

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 |  |
| ± 0-1 | -13 | 1 % of channel BW |
| ± 1-5 | -10 | 1 MHz |
| ± 5-6 | -25 |  |

Proposal 2: Same MPR applies for 3 MHz CBW in TN and NTN. No specification update is needed.

Observation 1: Based on initial evaluations NS_01, NS_02N, NS_05N and NS_100 may not need any A-MPR for 3 MHz CBW.

Observation 2: Based on initial evaluations NS_03N and NS_04N need A-MPR at low edge of band. Legacy A-MPR A3 can be re-used and a new A-MPR region is needed.

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3 MHz | fc < [1613.9] |  | [>= 1.8] | [A3] |

Observation 3: NS_24 A-MPR needs to confirmed by measurements given expected backoff is large. CIM5 needs to be taken into account.

Proposal 3: Continue A-MPR evaluations at least until RAN4#113 to ensure A-MPR specification are reliable.

Observation 4: Rx IIP2 needs to be considered in setting wanted signal level for 3 MHz refsens.

Proposal 4: Define refsens for 3 MHz channel bandwidth as shown in Table 2:

Table 2: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -101.7 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 5: Support of 3 MHz channel bandwidth is optional for UE.

## 4	References

[1]  R4-2413528, WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi

[2]  R4-1812118, A- MPR for n65, Qualcomm Incorporated

[3]  R4-2111733, n65 NS_24 AMPR, Qualcomm Incorporated