3GPP TSG-RAN WG4 Meeting #114	R4-2500049

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.18.2

Source:	MediaTek Inc.

Title:	Simulations for UE demodulation requirements with 8Rx MMSE-IRC receiver

Document for:	Discussion

- Introduction

A WF for UE 8Rx MMSE-IRC receiver requirements was agreed in RAN4#113 meeting [1]. In this contribution, we provide our simulation results.

- Simulation results for PDSCH requirements with inter-cell interference

Table 1. Simulation results for 8Rx MMSE-IRC receiver with inter cell interference

|  | MIMO | Rank | MCS | INR1 | INR2 | Channel Model | MMSE-IRC | MMSE | Gain over MMSE | Corresponding SINR (IRC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | 2T8R | 1 | 17 | 7.77 | 2.29 | TDLC300-100 | 6.4 | 10.4 | 4.0 | -3.0 |
|  | 2T8R | 2 | 17 |  |  | TDLA30-10 | 8.2 | 12.9 | 4.7 | -1.2 |
|  | 4T8R | 4 | 13 |  |  |  | 8.8 | 14.4 | 5.6 | -0.6 |
|  | 2T8R | 1 | 17 | 7.58 | N/A | TDLA30-10 | 3.8 | 7.9 | 4.1 | -4.5 |
|  | 2T8R | 2 | 17 |  |  |  | 7.7 | 11.9 | 4.2 | -0.6 |
|  | 4T8R | 4 | 13 |  |  |  | 8.3 | 13.4 | 5.1 | 0.0 |
| Simplified | 2T8R | 1 | 17 | 7.77 | 2.29 | TDLC300-100 | 8.2 | 11.2 | 3.0 | -1.2 |
|  | 2T8R | 2 | 17 |  |  | TDLA30-10 | 10.8 | 14.2 | 3.4 | 1.4 |
|  | 4T8R | 4 | 13 |  |  |  | 15.0 | 17.1 | 2.1 | 5.6 |
|  | 2T8R | 1 | 17 | 7.58 | N/A | TDLA30-10 | 5.5 | 9.1 | 3.6 | -2.8 |
|  | 2T8R | 2 | 17 |  |  |  | 10.3 | 13.4 | 3.1 | -5.2 |
|  | 4T8R | 4 | 13 |  |  |  | 14.4 | 16.1 | 1.7 | 6.1 |

- Simulation results for PDSCH requirements with intra-cell interference

Table 2. Simulation results for 8Rx MMSE-IRC receiver with intra-cell inter-user interference

|  | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Channel Model | Antenna correlation | Modulation order for the co-scheduled UE | MMSE-IRC | MMSE | Gain over MMSE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | NA | 10.3 | Inf |
|  |  |  | MCS 19 |  |  |  |  | NA | 12.0 | Inf |
| Simplified | 4T8R | 2+2 | MCS 17 | Orthogonal | TDLA30-10 | ULA Low | 16QAM | NA | 12.5 | Inf |
|  |  |  | MCS 19 |  |  |  |  | NA | 14.5 | Inf |

- Simulation results for CQI requirements with inter-cell interference

The simulation results are based on Table C in Appendix.

| Figure 1. Throughput ratio (gamma) versus SNR | Figure 2. BLER versus SNR |
| --- | --- |

- Conclusion

In this contribution, we shared simulation results for the PDSCH for both inter-cell and intra-cell interference scenarios and CQI for inter-cell interference scenarios.

- Reference

- R4-2419784, Way forward for [113][322] NR_demod_Ph5_Part2_UE, Nokia

- Appendix: Simulation Assumptions

Table A. Simulation assumptions for PDSCH requirements with inter cell interference

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for HomoNet Disabled for HetNet |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1,2,4 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for HomoNet N/A for HetNet |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 in 38.101-4 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for HomoNet 7.58 for HetNet | 2.29 for HomoNet N/A for HetNet |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 in 38.101-4 |  |  |  |  |  |

Table B. Simulation assumptions for PDSCH requirements with intra cell interference

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | 1000 | 1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 | 1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Single Panel Type I, For Rank 1+1: Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12].Any column of precoder matrix is not equal to any column of precoder matrix of Target UE. For Rank 2+2: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 in 38.101-4 |  |
| Number of HARQ Processes |  |  | 4 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

Table C. Simulation assumptions for CQI requirements with inter cell interference

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  | FDD | FDD |
| Subcarrier spacing |  |  | kHz | 15 | 15 |
| SINR |  |  | dB |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 | Row 5,6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | - |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-1 | - |
| INR (Note 6) |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2x8 | 1x8 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1. |  |  |  |  |  |