3GPP TSG-RAN WG4 Meeting #114	R4-2500050

Athens, GR, 17th – 21st February 2025

Agenda item:	7.1.1.3.2

Source:	MediaTek Inc.

Title:	Feasibility evaluation of MIMO layer for 6Rx UE

Document for:	Discussion

# Introduction

The WI NR_ENDC_RF_Ph4 [1] was approved during the RP#103 meeting with the objective of "6Rx for handheld and FWA UE." One of the tasks assigned to RAN4 is to investigate the benefits and feasibility of implementing 6 MIMO layers, specifically aimed at supporting handheld UEs.

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this paper, we conduct link-level simulations to assess the the gain and feasibility of implementing 6 layers for 6Rx UE. In addition, we share field data regarding SNR distribution in real field scenario.

# Field Data

We provide distributions of RSRP and SNR obtained from field data via SSB. Figure 1 shows the relationship between RSRP and SNR, while Figure 2 displays the CCDF of SNR. The data reveals that about 5% of SNR values exceed 25dB. We expect that the 6L will only demonstrate better performance than the 4L under high SNR conditions. Some companies have presented simulation results indicating that the 6L can outperform the 4L when the SNR is sufficiently high. In rare cases, the SNR is around 25dB, but in most cases, the SNR where the 6L outperforms the 4L is well over 30dB. Therefore, we believe the benefits of implementing 6L6R technology are limited.

Observation 1: From our field data, it shows that approximately 5% of SNR values measured through SSB exceed 25dB.

Observation 2: The advantages of implementing 6L6R over 4L6R are limited, given the infrequent occurrence of UE with SNR exceeding 30dB.

| Figure 1. RSRP and SNR distribution | Figure 2. CCDF of measured SNR |
| --- | --- |

- Link level simulation

## 2.1 Discussion for simulation assumptions

After RAN4#113 meeting, the evaluation assumptions are documented in the WF [2] as shown below.

Issue 2-1-1: Tx EVM assumptions for 6-layer feasibility evaluation Way Forward: RAN4 to further discuss the following options. - Option 1: Use Tx EVM = 1% - Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation - Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM Issue 2-1-2: 6-Layer Feasibility Evaluation Assumptions Way Forward: RAN4 to further discuss and align on simulation assumptions provided in Annex A and discuss possible options for refinement and down selection for key parameters as identified in company proposals as summarised below. The bullet list included is not intended to be a complete set of options and it does not specifically down select or preclude other options for refinement. It is only intended to summarize the options presented. - Antenna Configuration: - Limit the transmit/receive antenna configuration to 8Tx/6Rx and exclude 16Tx/6Rx and 32Tx/6Rx antenna configurations. - Consider both 8Tx/6Rx and 32Tx/6Rx antenna configurations - DMRS Configuration: - Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) from Annex A table. - Consider DMRS Configuration Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table. - Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) and Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table. - Consider additional DMRS Configuration Option 1a: Rel-18 1+1(6L), Rel-18 1+1(4L) - Rank - Include only Rank 4 and Rank 6 configurations - Include Rank 5, in addition to Rank 4 and Rank 6 - Modulation and Coding Schemes for Adaptive MCS - Consider Table 2 only, 64QAM + 256QAM - Consider Table 1 only, 64QAM - Consider Table 1 and Table 2 - Additional margins - Apply at least 3dB correction to the SNR values reported by companies in baseband simulation results - A 2~3 dB additional margin or 2% Rx EVM shall be considered - BS Antenna Correlation - RAN4 to use BS correlation matrix with alpha = 0.1 - RAN4 shall use no correlation on BS Tx Ports - RAN4 shall not use BS correlation matrix other than 3GPP specified ones

We provide our views for on simulation assumptions.

Tx EVM

According to Table 6.5.2.2-1 of RF specification TS 38.104, the BS EVM requirements is 3.5% for 256QAM.

Table 6.5.2.2-1: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

In the RAN4 PDSCH and SDR requirements, RAN4 utilizes Tx EVM values that are more stringent than those specified in TS 38.104. The following Tx EVM values for different modulation orders were agreed upon in [3]:

- Use the following RF impairments models to define the minimum UE performance requirements for FR1 - TX EVM = 6% for QPSK/16QAM/64QAM - TX EVM = 3% for 256QAM

Observation 3: For demodulation requirements, the assumed Tx EVM for 256 QAM is 3%, which is lower than the 3.5% requirement defined in TS38.104.

Observation 4: 256QAM will be used in high SNR regime when evaluating the performance of 6L.

Based on the prior agreements in the demodulation session and considering that 256QAM will be utilized in high SNR scenarios, we suggest adopting the Tx EVM as 3% for all modulation orders in MCS Table 2.

Proposal 1: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for evaluation.

DMRS configuration

Different configurations of DMRS are captured in the WF [2]:

- Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) from Annex A table.

- Consider DMRS Configuration Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table.

- Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) and Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table.

- Consider additional DMRS Configuration Option 1a: Rel-18 1+1(6L), Rel-18 1+1(4L)

RAN4 introduced the optional UE feature of enhanced DMRS in Release 18. However, to our knowledge, this feature has not seen widespread adoption yet. Additionally, if 6L can only provide gains over 4L with the enhanced DMRS, it would limit the advantages of UE supporting 6L. Therefore, we propose to consider only Rel-15 1+1 (4L) and Rel-15 2+2 (6L) for the 6L evaluation.

Somes companies have suggested the inclusion of Rel-15 2+0 (6L) for consideration. The additional DMRS can improve channel estimation accuracy, potentially leading to increased throughput, especially in scenarios of medium to high-speed mobility where channel conditions fluctuate rapidly. The quantity of extra DMRS is determined by the dmrs-AdditionalPosition parameter, which is set through an RRC message and therefore cannot be adjusted swiftly. Considering the dynamics of user equipment (UE) mobility, we anticipate Rel-15 2+0 is not the most prevalent and standard scenarios in practical applications.

Observation 5: Enhancement DMRS was introduced in Rel-18 and not widely adopted now by our understanding.

Observation 6: The benefit of UE to support 6L6R is limited if 6L6R can provide gain over 4L6R with enhancement DMRS only.

Observation 7: The number of additional DMRS is configured via the dmrs-AdditionalPosition parameter through an RRC message, and therefore, it cannot change rapidly. we anticipate Rel-15 2+0 is not the most prevalent and standard scenarios in practical applications.

Proposal 2: Use DMRS configuration [1+1 (4L), 2+2 (6L)] introduced from Rel-15 for feasibility evaluation.

Rank

Regarding rank, we support considering only Rank 4 and Rank 6. With 2 symbols for PDCCH and 12 symbols for PDSCH, the TBS will be approximately the same for 1+1 (4L) and 2+2 (5L), indicating that the maximum throughput will be similar. Additionally, the primary objective of the WID is to evaluate whether 6L offers a performance advantage over 4L.

Observation 8: With 2 symbols for PDCCH and 12 symbols for PDSCH, the TBS will be approximately the same for 1+1 (4L) and 2+2 (5L), indicating that the maximum throughput will be similar.

Observation 9: The main goal of the WID is to determine if 6L can provide gain over 4L.

Proposal 3: Include only Rank 4 and Rank 6 for evaluation.

Modulation and Coding Schemes for Adaptive MCS

As we are now evaluating the 6L configuration, we believe that achieving 6L is only feasible in high SNR regions. However, with 64QAM Table, the throughput will not increase even in very high SNR conditions, as the maximum modulation order is limited to 6. Consequently, we support the consideration of the 256QAM Table to achieve higher throughput. By adopting 256QAM, we can take full advantage of the high SNR environment, thereby enhancing overall system performance.

Observation 10: 6L is only feasible in high SNR regions, but MCS Table 1 (maximum 64 QAM) limits throughput.

Proposal 4: Consider MCS Table 2 (support 256QAM) only.

Antenna efficiency and MIMO correlation

The MIMO correlation options from Option 1 to Option 4 in Appendix of WF [2] are represented by ECC (envelope correlation coefficient) values, which indicate the correlation between the radiation patterns of two different antennas [4]. In RAN4 simulations, the diagonal terms in the MIMO correlation matrix are always set to 1, which is acceptable for conducted tests. However, in the current feasibility study of 6L, companies have proposed ECC measurement results for each antenna and suggested using these results for evaluation. In this scenario, the assumption of all-1 diagonal terms is no longer valid because it is unrealistic for all antennas to have perfect efficiency. Therefore, it is reasonable to also consider antenna efficiency in this feasibility study. According to [5], the correlation coefficient for antenna pairs depends on both ECC and antenna efficiency,

ρpp=ηpp,   if p=qρpq=ηpηqECCp, q,   if p≠q                                                                   (1)

Here, ρpq represents the correlation coefficient between antennas p and q, ηp and ηq denote the efficiency of antenna p and q, respectively, and ECC(p, q) is the envelope correlation coefficient between antenna p and q. In Table 1, we present our measured antenna efficiencies and use these values to calculate the MIMO correlation coefficients for the feasibility study. It is evident that there is at least a 3dB efficiency loss for each antenna.

Table 1. Measured antenna efficiency

|  | ANT0 | ANT1 | ANT2 | ANT3 | ANT4 | ANT5 |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna efficiency (dB) | -7.8 | -6.7 | -5.5 | -3.5 | -4.4 | -3.7 |

With the measured antenna efficiency, equation (1) and ECC Option 1 in Table 2, we derive the MIMO correlation as below:

ρ=0.1660.0190.0650.0410.0320.0320.0190.2140.0250.0430.0840.060.0650.0250.2820.0430.0640.0420.0410.0430.0430.4470.0440.1310.0320.0840.0640.0440.3630.0390.0320.060.0420.1310.0390.427                                                  (2)

Observation 11: The UE correlation matrices Option 1 to Option 4 in Appendix of WF [2] are ECC, not correlation coefficient.

Observation 12: As RAN4 incorporates measured ECC values into link-level simulations to reflect real antenna performance, it is also necessary to consider antenna efficiency.

Proposal 5: Use following equations to derive MIMO correlation, where ρpq is the correlation coefficient between antenna p and q, ηp and ηq are the antenna efficiency of antenna p and q, and ECC(p, q) is the ECC between antenna p and q.

ρpp=ηpp,   if p=qρpq=ηpηqECCp, q,   if p≠q

## 2.2 Simulation results

We provide simulation results based on simulation assumptions in Table 2.

Table 2. Simulation assumptions for 6Rx evaluation

|  |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] |
|  | DMRS Configurations | Rel-15 1+1(4L), Rel-15 2+2(6L) |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) |
| UE Correlation matrix |  | Option 1 (from HW R4-2412572): 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 2 (option 1 considering antenna efficiency): 0.1660.0190.0650.0410.0320.0320.0190.2140.0250.0430.0840.060.0650.0250.2820.0430.0640.0420.0410.0430.0430.4470.0440.1310.0320.0840.0640.0440.3630.0390.0320.060.0420.1310.0390.427 |

Figure 1 illustrates the throughput versus SNR for 4L and 6L transmissions, taking into account MIMO correlation Options 1 and 2 as specified in Table 2.

Figure 3. Simulation results for MIMO correlation Option 1 in Table 2.

Our simulation results show that the throughput for 6L is lower than that for 4L when considering a 3% Tx EVM and MIMO correlation Option 1. With MIMO correlation Option 2 (Option 1 adjusted for antenna efficiency), our results show that achieving the same throughput as MIMO correlation Option 1 (assuming ideal 0dB antenna efficiency) requires more than 5dB additional signal power. As we are now focusing on the high SNR region where 6L6R may outperform 4L6R, this indicates that it is challenging to achieve the necessary SNR for 6L6R to outperform 4L6R in real-world conditions when considering realistic antenna efficiency.

Observation 13: No performance gain was observed for 6L6R compared to 4L6R when considering a 3% Tx EVM and MIMO correlation Option 1.

Observation 14: Taking realistic antenna efficiency into account, it is challenging to achieve the necessary SNR for 6L6R to outperform 4L6R in the field, as it requires significantly more signal power (over 5dB).

Based on observations 10 and 11, we concluded that there is no need to introduce requirements for 6 MIMO layers, as our simulation results showed no benefits when considering measured ECC and antenna efficiency.

Proposal 6: Do not introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results based on realistic antenna correlation matrixes.

# Conclusion

In this contribution, we provide our evaluation results and views on the feasibility of 6 layers for 6 Rx UE. Observations and proposals are summarized as follows.

Observation 1: From our field data, it shows that approximately 5% of SNR values measured through SSB exceed 25dB.

Observation 2: The advantages of implementing 6L6R technology are limited, given the infrequent occurrence of UE with SNR exceeding 30dB.

Observation 3: For demodulation requirements, the assumed Tx EVM for 256 QAM is 3%, which is lower than the 3.5% requirement defined in TS38.104.

Observation 4: 256QAM will be used in high SNR regime when evaluating the performance of 6L.

Proposal 1: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for evaluation.

Observation 5: Enhancement DMRS was introduced in Rel-18 and not widely adopted now by our understanding.

Observation 6: The benefit of UE to support 6L6R is limited if 6L6R can provide gain over 4L6R with enhancement DMRS only.

Observation 7: The number of additional DMRS is configured via the dmrs-AdditionalPosition parameter through an RRC message, and therefore, it cannot change rapidly. we anticipate Rel-15 2+0 is not the most prevalent and standard scenarios in practical applications.

Proposal 2: Use DMRS configuration [1+1 (4L), 2+2 (6L)] introduced from Rel-15 for feasibility evaluation.

Observation 8: With 2 symbols for PDCCH and 12 symbols for PDSCH, the TBS will be approximately the same for 1+1 (4L) and 2+2 (5L), indicating that the maximum throughput will be similar.

Observation 9: The main goal of the WID is to determine if 6L can provide gain over 4L.

Proposal 3: Include only Rank 4 and Rank 6 for evaluation.

Observation 10: 6L is only feasible in high SNR regions, but MCS Table 1 (maximum 64 QAM) limits throughput.

Proposal 4: Consider MCS Table 2 (support 256QAM) only.

Observation 11: The UE correlation matrices Option 1 to Option 4 in Appendix of WF [2] are ECC, not correlation coefficient.

Observation 12: As RAN4 incorporates measured ECC values into link-level simulations to reflect real antenna performance, it is also necessary to consider antenna efficiency.

Proposal 5: Use following equations to derive MIMO correlation, where ρpq is the correlation coefficient between antenna p and q, ηp and ηq are the antenna efficiency of antenna p and q, and ECC(p, q) is the ECC between antenna p and q.

ρpp=ηpp,   if p=qρpq=ηpηqECCp, q,   if p≠q

Observation 13: No performance gain was observed for 6L6R compared to 4L6R when considering a 3% Tx EVM and MIMO correlation Option 1.

Observation 14: Taking realistic antenna efficiency into account, it is challenging to achieve the necessary SNR for 6L6R to outperform 4L6R in the field, as it requires significantly more signal power (over 5dB).

Based on observations 10 and 11, we concluded that there is no need to introduce requirements for 6 MIMO layers, as our simulation results showed no benefits when considering measured ECC and antenna efficiency.

Proposal 6: Do not introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results based on realistic antenna correlation matrixes.

# Reference

- RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon

- R4-2420376, WF on 6Rx UE requirements, AT&T

- R4-1811394, Way forward on NR UE PDSCH demodulation requirements and General aspects, Intel

- Y. Kabiri, A. L. Borja, J. R. Kelly, and P. Xiao, ‘‘A technique for MIMO antenna design with flexible element number and pattern diversity,’’ IEEE Access, vol. 7, pp. 86157–86167, 2019.

- X. Mei and K.-L. Wu, “Capacity-oriented envelope correlation coefficient for multiple antennas of mobile devices,” IEEE Trans. Antennas Propag., vol. 70, no. 10, pp. 9785–9794, Oct. 2022.