3GPP TSG-RAN WG4 Meeting #114	R4-2500051

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.10.4

Source:	MediaTek Inc.

Title:	Discussion on NTN testing for NGSO

Document for:	Discussion

# Introduction

At RAN4#113 meeting, a WF for NGSO testing was agreed [1]. In this contribution, we provide our views on outstanding issues.

# Discussion

Issue 1-1-3-3: Assumption of updated granularity for propagation delay and Doppler shift calculation - WF - Option 1 (WF in the last meeting) - The Doppler shift and propagation delay should be calculated and updated based on agreed TE emulated channel model per slot - TE vendors are encouraged to provide the feedback about the implementation feasibility to update the Doppler shift and propagation delay per slot Issue 1-1-3-4: Assumption of updated granularity for Satellite ephemeris information - WF - Unless otherwise stated on existing test configuration, the TE is expected to refresh satellite ephemeris in SIB 19 at least once every 10.24s - For RF and demodulation requirement, use 10.24s to update the satellite ephemeris information - For RRM requirement, use 2.56s to update the satellite ephemeris information

According to the WF, when deriving the time-varying TE emulated NGSO channel, there are three distinct types of time steps: T_kelper, T_slot, and T_ephemeris as shown in Figure 1.

Figure 1. Illustration for T_kepler, T_slot and T_ephemeris

Table 1. Types of time steps when deriving the time-varying TE emulated NGSO channel

| T_kelper | The position and velocity state vector are calculated every T second based on Keplerian model. Based on the given UE location, the Doppler shift and propagation delay are then derived accordingly. |
| --- | --- |
| T_slot | RAN4 assume that the Doppler shift and propagation delay should be calculated and updated per slot. |
| T_ ephemeris | RAN4 assume TE is expected to refresh satellite ephemeris in SIB 19 at least once every 10.24s - For RF and demodulation requirement, use 10.24s to update the satellite ephemeris information - For RRM requirement, use 2.56s to update the satellite ephemeris information |

Should T_kepler differs from T_slot, the Doppler shift and propagation delay for each slot are obtained through interpolation techniques. We used the procedures and formulas in Appendix for simulations. Preliminary evaluations of various interpolation methods in relation to T_kepler are presented in Table 2 and 3. We present the highest absolute error values for Doppler and delay along the satellite trajectory, referencing Table 6.4.1_2.4.3-1 in TS 38.521-5 for the initial ephemeris data. The absolute error is calculated as the absolute difference between the scenarios "without interpolation" (where T_kepler equals T_slot) and "with interpolation" (where each T_slot within T_kepler is interpolated).

Table 2. Maximum absolute error in doppler and delay with interpolation

|  | maximum absolute error in doppler with interpolation |  | maximum absolute error in delay with interpolation |  |
| --- | --- | --- | --- | --- |
|  | cubic | linear | cubic | linear |
| T_kepler = 64 ms T_slot = 1ms | 5.384208634495e-10 | 0.0029723538900725 | 1.6479873021779e-17 | 1.457767500462e-10 |
| T_kepler = 512 ms T_slot = 1ms | 1.3645058061229e-06 | 0.19022978622160 | 2.146546829173e-14 | 9.329605036873e-09 |
| T_kepler = 1024ms T_slot = 1ms | 2.1848600226803e-05 | 0.7608848794407 | 3.4346570726428e-13 | 3.731680531205e-08 |

Table 2 indicates that the error with linear interpolation is significantly larger compared to the cubic method. Specifically, when T_kepler is 1024ms, the error in Doppler using linear interpolation is approximately 0.7 Hz and the error using cubic is approximately 2.18e-5 Hz.

Given the varying computational complexities among different TE vendors, not all may be able to calculate the position and velocity state vector for every slot using the Keplerian model. As a result, TE vendors might implement their own solutions with different T_kepler values and interpolation methods. We are open to not restricting the implementation on the TE side. However, RAN4 should discuss and determine an acceptable error value introduced by interpolation.

Observation 1: Based on the initial ephemeris data in Table 6.4.1_2.4.3-1 of TS 38.521-5, and assuming the position and velocity state vector are calculated every 1024ms using the Keplerian model. The error in Doppler using linear method is approximately 0.7 Hz and the error using cubic method is approximately 2.18e-5 Hz

Proposal 1: We are open to not restricting the implementation on the TE side. However, RAN4 should discuss and determine an acceptable error value introduced by interpolation.

# Conclusion

In this contribution, we provide our views on NTN testing for NGSO. Observations and proposals are summarized as below.

Observation 1: Based on the initial ephemeris data in Table 6.4.1_2.4.3-1 of TS 38.521-5, and assuming the position and velocity state vector are calculated every 1024ms using the Keplerian model. The error in Doppler using linear method is approximately 0.7 Hz and the error using cubic method is approximately 2.18e-5 Hz

Proposal 1: We are open to not restricting the implementation on the TE side. However, RAN4 should discuss and determine an acceptable error value introduced by interpolation.

# Reference

- R4-2413516, Way Forward for [113][322] NTN_testing_NGSO_channel_model, Samsung