3GPP TSG-RAN WG4 #114	R4-2500057

Athens, Greece, February 17-21, 2025

Agenda item:	7.25.2.2		Adaptation of Common Signal/Channel Transmission

Source: 	Qualcomm Incorporated

Title: 	On RRM requirements for adaptation of common signal/channel transmission

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 NES was started. Several agreements have been made for SSB adaptation in RAN4 #113 in [1] as listed below. The agreements are as follows:

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

    - No RRM impacts for SSB adaptation requirement for IDLE mode.

    - No RRM impacts for Cell DTX.

Issue 1-2-1: L1 requirements at transition

Agreement:

  - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation.

    - FFS the applicable L1 requirements.

Issue 1-1-3: Scenario clarification-  SSB adaptation triggering

Agreement:

  - RAN4 to focus on NW controlled SSB adaptation firstly.

Issue 1-2-2: L1 requirements subsequent to “transition period”

Agreement:

  - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity.

  - Note: The transition period is discussed in other issues.

Several additional open issues have been identified in RAN4 #113 in [1] that depend on further RAN1 input.

Issue 1-1-2: Scenario clarification- SSB periodicity

Agreement:

  - Postpone the discussion and wait for RAN1 progress.

Issue 1-1-4: Scenario clarification-  SSB adaptation mechanism

Agreement:

  - Postpone the discussion and wait for RAN1 progress.

Several additional open issues have been identified in RAN4 #113 in [1] and depend on further RAN2 input.

Issue 1-3-1: L3 requirements for SSB adaptation – general

Agreement:

  - Wait for LS reply on whether there is impact on SMTC.

In this paper we provide our views for the issues identified in the way forward.

# Discussion

In RAN4 #112bis it was agreed that RAN4 takes as a starting point for SSB adaption: CONNECTED mode, SSB adaptation for SCell, SSB adaptation for NCD-SSB and SSB periodicity adaptation in time domain. In RAN4 #112bis an LS has been sent to RAN1 to get further information [3]. The Reply LS has been received in [4].

RAN1 confirmed that so far it was only agreed to adapt SSB burst periodicity. Also, there is no change in frequency before and after SSB adaptation. The QCL relation for the same SSB index before and after SSB adaptation is unchanged. Finally, RAN1 confirmed that at least the case is supported that there is no change in transmitted SSB before and after SSB adaptation.

Observation 1: RAN1 confirmed that SSB adaptation means adaptation of burst periodicity. There is no change in frequency before and after SSB adaptation. The QCL relation for the same SSB index before and after SSB adaptation is unchanged. It was confirmed that there is no change in transmitted SSB before and after SSB adaptation.

Since no additional agreements has been achieved, RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged.

Proposal 1: RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged.

In addition, in RAN1 #119 no consensus was achieved to support SSB adaptation in time domain for Rel-19 NES-capable PCell in connected mode. Hence, RAN4 should not define requirements for PCell.

Proposal 2: RAN4 should not define requirements for SSB adaptation for PCell

To get further information on the impact to SMTC, a LS has been sent to RAN2 in [2]. No information has yet been received from RAN2. Hence, discussions about requirements impacting L3 should be postponed till the RAN2 Reply LS has been received.

Observation 2: No information has yet been received from RAN2 regarding the impact on SMTC.

Proposal 3: RAN4 should postpone discussions about impacts on L3 requirements and focus on L1 requirements.

In RAN4 #113 it was agreed that during the transition period of SSB adaptation the maximum corresponding to the SSB before and after adaptation is used for defining requirements for the evaluation/measurement period. After the transition period, the periodicity after the SSB adaptation is applied again.

Figure 2-1 illustrates the situation for SSB periodicity adaptation from P1  P2. In our understanding the RAN4 agreement means that during the transition period, i.e., direct after the SSB burst periodicity has been changed, the requirements apply with a periodicity of max{P1, P2} instead of P1.

Observation 3: In our understanding the RAN4 agreement means that during the transition period, i.e., direct after the SSB burst periodicity has been changed, the requirements apply with a periodicity of max{P1, P2} instead of P1. That means the requirement is potentially relaxed.

Figure 2-1: Illustration of Transition in Measurement Period

What remains to be clarified is the duration of the transition period during which the requirements are based on max{P1, P2} before the requirement based on P2 applies again. The scenario is illustrated in Figure 2-2. At a certain point in time, a switch command is received base don DCI, MAC CE or RRC. Then it takes a certain time Tfirst_SSB till the first SSB with the new period P2 is received. This time depends on the type of the switching command, i.e., whether the switching is done by DCI, MAC CE or RRC. Afterwards the measurement can be defined based on periodicity P2, called meas_period(P2). The transition period can be defined as Tfirst_SSB + meas_period(P2). The measurement period based on P2 depends on the type of the requirement, i.e., whether it is L1-RSRP, CBD, etc.

Figure 2-2: Illustration of Transition Duration

Observation 4: The transition period can be defined as Tfirst_SSB + meas_period(P2). Tfirst_SSB is the time till the first SSB with the new period P2 is received. meas_period(P2) is the measurement period based on the new periodicity P2.

Proposal 4: RAN4 should analyze whether Tfirst_SSB + meas_period(P2) can serve as transition period for all requirements. RAN4 should also discuss whether a general definition of the transition time is sufficient for ich relaxed requirements apply without defining new test cases.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #112bis. Our observations and proposals are summarized as follows:

Observation 1: RAN1 confirmed that SSB adaptation means adaptation of burst periodicity. There is no change in frequency before and after SSB adaptation. The QCL relation for the same SSB index before and after SSB adaptation is unchanged. It was confirmed that there is no change in transmitted SSB before and after SSB adaptation.

Proposal 1: RAN4 should focus on adaptation of SSB periodicity for connected mode, SCell and NCD-SSB under the assumption that frequency location and QCL relation before and after SSB adaptation is unchanged.

Proposal 2: RAN4 should not define requirements for SSB adaptation for PCell

Observation 2: No information has yet been received from RAN2 regarding the impact on SMTC.

Proposal 3: RAN4 should postpone discussions about impacts on L3 requirements and focus on L1 requirements.

Observation 3: In our understanding the RAN4 agreement means that during the transition period, i.e., direct after the SSB burst periodicity has been changed, the requirements apply with a periodicity of max{P1, P2} instead of P1. That means the requirement is potentially relaxed.

Observation 4: The transition period can be defined as Tfirst_SSB + meas_period(P2). Tfirst_SSB is the time till the first SSB with the new period P2 is received. meas_period(P2) is the measurement period based on the new periodicity P2.

Proposal 4: RAN4 should analyze whether Tfirst_SSB + meas_period(P2) can serve as transition period for all requirements. RAN4 should also discuss whether a general definition of the transition time is sufficient for ich relaxed requirements apply without defining new test cases.

# References

- R4-2420242, WF on RRM requirements for Netw_NR_enh_part2, RAN4 #113, November 2024

- R4-2416911, LS on SSB adaptation, RAN4 #112bis, October 2024

- R4-2416913, LS on SSB relation in on-demand SSB and SSB adaptation on SCell, RAN4 #112bis, October 2024

- R1-2410917, Reply LS on SSB relation in On-demand SSB and SSB adaptation on SCell, RAN1 #119, November 2024