3GPP TSG-RAN WG4 Meeting # 114	R4-2500074

Athens, Greece, 17th – 21st February 2025

Agenda Item:	7.13.4.3

Source: 	Nokia

Title: 	Simplification of BS TRP test methods

Document for:	Approval

# 1.	Introduction

The revised WI on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at TSG RAN#106 [1]. The objective of this WI on simplification of BS TRP test methods is revised as follow:

- TRP simplification: Simplify BS TRP estimation methods (e.g. Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods.

This topic was discussed at TSG RAN4#113 [2] and the WF was agreed [3]. This contribution provides proposals to progress this topic in RAN4.

# 2.	Discussion

As discussed in [4], work related to this topic is also being performed in ETSI ERM WGRM with the findings being captured in the ETSI TR 103 974 [5]. As stated in [5], this work was initiated by a letter from the European Commission, Directorate for Internal market, Entrepreneurship and SMEs, in which it is stated [5, Annex A]:

Regarding the presence of alternative testing methods, as indicated in the last radio equipment expert group, in general, the one testing method for each requirement is the preferred option. However, there can be cases where in order to foster innovation, or to ensure technological neutrality, two or more methods are proposed by European Standardisation Organisations. In these circumstances, it is important to get assurances that the various methods proposed are equivalent in terms of accuracy and reproducibility and reflect the state of the art. This is important to ensure that the different methods lead to comparable test results for establishing compliance. In this context, the Commission may undertake studies to assess if the various methods proposed meet these criteria and draw the necessary consequences.

Consequently, the objective of this work in ETSI ERM WGRM is stated as follows [5]:

To investigate the impact of this letter on the work in ETSI, this Technical Report will provide a discussion on the conditions for harmonised standards to offer different test methods for one requirement, with the focus on typical radio parameters. One question to be answered is for example, how to ensure equivalent results of a radiated power measurement on different test sites (FAR, SAR, OATS).

The European Commission, Directorate for Internal market, Entrepreneurship and SMEs acknowledged in the letter that two or more methods can be proposed to foster innovation or to ensure technological neutrality. Test sites should be allowed to select among different test methods for each test, provided that the different test methods are equivalent in term of accuracy and reproducibility and reflect the state of the art. Therefore, it is proposed that RAN4 work should not be aimed to limit to one test method for each requirement, since this may not foster innovation or ensure technological neutrality as all test sites will be mandated to use the same test method.

On the other hand, both and ETSI ERM WGRM and the European Commission, Directorate for Internal market, Entrepreneurship and SMEs emphasized the importance to ensure that the different test methods allowed for each requirement lead to comparable/equivalent test results for establishing compliance. Therefore, it is proposed that RAN4 work should be aimed to ensure that the different test methods applicable to each requirement lead to comparable/equivalent test results, by improving their applicability and description in TR 37.941 [6] and corresponding test specifications.

Moreover, the test requirements for establishing compliance are calculated as the core requirements plus the test tolerances, and the test tolerances are individually determined for each requirement considering the regulatory requirements (tests for regulatory requirements have 0 dB test tolerance) and the allowed measurement uncertainties for the different test methods. Note that the actual measurement uncertainty of each test equipment may still be different even for the same test method, and test sites should be allowed to select different test equipment vendors provided that the actual measurement uncertainty is lowered than the allowed one for each test method, but the same test requirement shall apply whichever test method\equipment is used in the test. Although the allowed measurement uncertainties for the test methods are different, and some of them have been revised since they are defined in the earliest releases of TR 37.941, the allowed measurements uncertainties and test tolerances (and hence the test requirements) in the test specifications have not been changed since they are defined in the earliest releases. Therefore, it is proposed that RAN4 work should not change any allowed measurements uncertainties and test tolerances (and hence test requirements) defined in the test specifications.

# 3.	Conclusion

This contribution has provided proposals to progress the objective of the WI for TRP simplification in RAN4.

Proposal 1: RAN4 work should not be aimed to limit to one test method for each requirement, since this may not foster innovation or ensure technological neutrality as all test sites will be mandated to use the same test method.

Proposal 2: RAN4 work should be aimed to ensure that the different test methods applicable to each requirement lead to comparable/equivalent test results, by improving their applicability and description in TR 37.941 and corresponding test specifications.

Proposal 3: RAN4 work should not change any allowed measurements uncertainties and test tolerances (and hence test requirements) defined in the test specifications.

# References

[1]	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC.

[2]	R4-2419591, “Topic summary for [113][304] NR_BS_RF_Part3_OTA_TRP”, ZTE.

[3]	R4-2419828, “Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP”, ZTE Corporation.

[4]	R4-2419678, “Discussion on potential simplification of BS TRP test methods”, Huawei, HiSilicon.

[5]	ETSI TR 103 974 v0.0.4: “On the equivalence of measurement results with different test methods, v.0.0.4”.