3GPP TSG-RAN WG4 Meeting #114	R4-2500110

Athens, GR, Feb. 2025

Source	:	Qualcomm, Sony

Title	:	On the UE RF requirement for the FR2 asymmetric connection scenario

Agenda item	:	7.22.2

Document for	:	Approval

# 1.	Introduction

RAN4 has been discussing an asymmetric connection scenario for FR2 with an aim to improving the UL budget. This scheme assumes ‘UL TRPs’ are deployed in higher density than legacy TRPs (UL+DL), and that the UE can expect to receive SSBs from these UL TRPs. RAN4 decided to check with RAN1 on inclusion of the latter in the WI scope. In this contribution we develop the proposal based on confirmation of the same from RAN1.

# 2. 	FR2 enhancement discussion

## 2.1	Confirmation that RAN4 proposal is within WI scope

For Rel-19, RAN4 has been investigating a proposal [2,3] for the FR2 asymmetric connection feature where UL and DL are associated with different TCI states, each linked to different DL RSs from the respective TRPs. Implicit in this proposal is that the UE can expect to receive SSBs from UL TRPs, but this aspect was not clear in RAN4. Consequently, RAN4 enquired if the SSB assumption (DL RS) of the proposal was in the scope of RAN1 work [4]:

The reply from RAN1 confirmed that the RAN4 proposal based on reception of SSBs from the UL TRP was indeed within scope of the Rel-19 work in RAN1 [5]

Based on the RAN1 reply, RAN4 can now proceed with refining the technical details of the proposal for the Rel-19 FR2 asymmetric connection feature where UL and DL are associated with different TCI states, each linked to different SSBs from the respective TRPs.

UL + DL TRP, legacy

UL TRP (*)

DL

SSB + PDSCH

UL

(*) – SSB available in DL

SSB only DL

UL TRP (*) configured to UE for UL

UL TRP (*)

UL TRP (*)

2.2	The scenario

For convenience, the enhancement scenario is reproduced here from [2,3]. The basic premise is the existence of UL TRPs (with SSB capability), and that these can be deployed at higher density compared to the legacy TRP (UL +DL). This higher density increases the probability of LOS as well as reduced link distances, providing much need relief to the UL budget. To exploit this scheme the system would configure the UE to communicate with an UL TRP for UL alone. In more precise terms, the UL and DL are associated with different TCI states, corresponding to the UL TRP and the TRP associated with initial access respectively.

The RACH procedure and timing issues outlined in a prior contribution [2] highlighted the need for independent TAs for each TRP:

Observation 4 [2]: The proposed scenario (different TCI states for UL and DL) benefits UEs with limited UL capability towards the DL TRP. The different TCI states are expected to have different physical locations, consequently, 2 TA capability is required.

This observation is worth reiterating for a self-contained summary in this contribution:

Observation 1: It is crucial for the UE to be enabled for 2TA operation for the FR2 asymmetric connection feature.

Two TAs can be configured to the UE currently using mTRP mDCI framework, but mTRP is a very heavy feature and would make the proposed Rel-19 enhancement unattractive. For this reason, RAN1 is actively pursuing a way to enable 2TA operation in the UE outside the mTRP framework.

Observation 2: RAN1 is pursuing a method to configure 2TA operation in the sTRP framework under NR_MIMO_Ph5.

## 2.3	Capabilities gating support of the feature

While this feature is presented as a simultaneous connection with 2 physically different TRPs, in detail it is a connection with one TRP at a time due to TDD. From RAN1 point of view it is a simple multi-beam operation (sTRP with different TCI states). As such the pre-requisite capability is support of Rel-17 unified TCI state framework and specifically the ability to have different TCI states for UL and DL (for FR2). This capability is unifiedSeparateTCIstate. The network in turn must configure the UE with unifiedTCI-StateType as ‘separate’. Consequently, configuration of unifiedTCI-StateType as ‘separate’ guarantees that the UE supports the necessary unified TCI capabilities and is correctly configured.

In RAN4 context, the UE must track two different beam management reference signals (QCL type D RS), but this is not a new skill; the UE can support inter-band CA in FR2 since Rel-16 and 17 for DL and UL+DL respectively. Consequently, no unique capability is required in RAN4 for this feature. Note however that the RAN1 LS reply [5] only confirms SSB transmissions from UL TRPs when PLoffset is not configured.

Observation 3: For the Rel-19 FR2 asymmetric connection feature, the UE must be configured with the IE unifiedTCI-StateType as ‘separate’ for FR2 and must not be configured with [PL offset] in the UL TCI state.

Any further capabilities depend on whether RAN1 define a new capability associated with the effort referenced in observation 2 (‘way to configure 2 TAs without reliance on mTRP’).

Proposal 1: Any new capability for the Rel-19 FR2 asymmetric connection feature depend only on RAN1, no new capability is required in RAN4.

## 2.4	The UE RF requirement proposal

The proposed feature in essence is to retain the legacy UL EIRP capability, only here the UL is opportunistically directed by the network at a more suitable reception point than the TRP of initial access. As such, the goal is to ensure that the UE has the same EIRP statistics as legacy UL operation, but with a second TCI state (i.e. in the direction of the more suitable reception point). The requirement is therefore one of beam correspondence: the UE must demonstrate legacy UL EIRP statistics with an UL-specific QCL type D reference signal.

Observation 4: The UE RF requirement for the FR2 asymmetric connection feature is one of beam correspondence:

- The UE must demonstrate legacy UL EIRP statistics with a QCL type D DL reference signal from the UL TRP.

- The latter is distinct from the QCL type D DL reference signal transmitted from the DL TRP which is used for the purposes of refining the UE’s Rx beams

Another aspect for the requirement is the relative spatial locations of the ‘legacy TRP’ and the ‘UL TRP’. For a test condition that represents field conditions, the TRPs must ideally be spatially separated. While such a test capability exists from the test set developed for the legacy Rel-18 multiRx feature, it is complex and expensive. Alternatively, a simpler precedent can be used: in Rel-16 DL interband CA, there was no spatial separation between the different beam management reference signals, although the reference signals are self-evidently different. In this Rel-19 feature, the UE must track the RS associated with each TCI state independently, so the UE itself should have no preference on spatial separation. Consequently, zero separation is equivalent to any other choice, from the perspective of verifying UE functionality. This scheme is beneficial because it re-uses the legacy single AoA FR2 test setup, a strong practical consideration.

Proposal 2: The legacy FR2 single AoA test set up is used for the UE RF requirement for the FR2 asymmetric connection feature.

A closing consideration is spec. implementation – unfortunately none of the existing suffixes are applicable to the mode of operation in this proposal. While there is loss of tracking between -1 and -2 on suffixes, it may be safer to claim the next available letter across both, ‘M’

Proposal 3: The UE RF requirement for the FR2 asymmetric connection feature is captured in a new suffix, ‘M’.

The accompanying draftCR [6] is provided for visualization of the RF requirement wording.

# 4.	Conclusions

Observation 1: It is crucial for the UE to be enabled for 2TA operation for the FR2 asymmetric connection feature.

Observation 2: RAN1 is pursuing a method to configure 2TA operation in the sTRP framework under NR_MIMO_Ph5.

Observation 3: For the Rel-19 FR2 asymmetric connection feature, the UE must be configured with the IE unifiedTCI-StateType as ‘separate’ for FR2 and must not be configured with [PL offset] in the UL TCI state.

Observation 4: The UE RF requirement for the FR2 asymmetric connection feature is one of beam correspondence:

- The UE must demonstrate legacy UL EIRP statistics with a QCL type D DL reference signal from the UL TRP.

- The latter is distinct from the QCL type D DL reference signal transmitted from the DL TRP which is used for the purposes of refining the UE’s Rx beams

Proposal 1: Any new capability for the Rel-19 FR2 asymmetric connection feature depend only on RAN1, no new capability is required in RAN4.

Proposal 2: The legacy FR2 single AoA test set up is used for the UE RF requirement for the FR2 asymmetric connection feature.

Proposal 3: The UE RF requirement for the FR2 asymmetric connection feature is captured in a new suffix, ‘M’.

# 5.	References

- RP-242394, ‘WID revision: NR MIMO Phase 5’, Samsung, RAN#105

- R4-2415724, ‘FR1 3Tx power scaling and FR2 asymmetric connection scenario’, Qualcomm et al., RAN4#112-Bis

- R4-2417632, ‘dCR on RF requirements for a UE with different TCI states for UL and DL’, Qualcomm, RAN4#113

- R4-2417117, ‘LS on UE RF issues for Rel-19 MIMO enhancement’, RAN4 (Huawei), RAN4#112-Bis

- R4-2500005, ‘Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement’, RAN1 (Interdigital et al.), RAN4#112-Bis

- R4-2500111, ‘dCR on RF requirements for a UE with different TCI states for UL and DL’, Qualcomm, RAN4#114