3GPP TSG-RAN4 #114																					R4-2500167

Athens, Greece, 17th Feb. – 21st Feb. 2025

Title:	FR2 test setup for AI/ML beam management evaluation

Source:			Anritsu Corporation

Agenda Item:			7.20.2

Document for:			Approval

# 1.	Introduction

At the RAN4 #113 meeting in Orlando, we continued an extensive discussion on a test setup for the AI/ML beam management test setup. The associated way forward [1] was created, extracted below are few issues for reference.

- Agreements in main session: Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. - Agreements in ad-hoc session (R4-2420335): Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

In this contribution, we share our findings regarding the associated discussion on AI/ML beam management evaluation among RAN1 and RAN4, and we provide our views on them.

# 2.	Discussion

# 2.1. Analysis of AI model inputs

During the meeting #113, one of the controversial points in the group was whether the UE Rx beam knowledge was necessary as the AI model input during a beam prediction process. From a viewpoint of this performance test procedure, characteristics of the UE Rx beam knowledge (e.g. Rx beam gain, direction during the test) and their necessity for AI model input have a huge impact on the 1AoA test procedure and setup with wireless (virtual) cable connection. In a case the UE Rx beam information is one of the AI model inputs in addition to the L1-RSRPs of set B, there is a need that we measure an actual UE Rx beam pattern to include it in the simulation by a fading simulator.

Observation 1: It is necessary to clarify characteristics of the UE Rx beam knowledge and their necessity for AI model input since they have a huge impact on the 1AoA test procedure and setup with wireless (virtual) cable connection.

To clarify the question, we first checked the evaluation results which is included in TR 38.843 [2].

For example, Table 1 is an extract of the spreadsheet “BM_Table 1. Evaluation results for BMCase-1.xlsx” which is included in TR 38.843 [2]. Note not all the companies were extracted below.

Table 2.1-1: Part of extract from the spreadsheet of TR 38.843 on model inputs to inference of Tx beam or beam pair for BM Case 1

| Inference | Model input | Company |
| --- | --- | --- |
| Tx Beam | L1-RSRPs of Set B | Apple CMCC Google Huawei/HiSilicon NTT docomo Oppo Qualcomm Vivo |
|  | L1-RSRPs of Set B and implicit beam information | CATT Ericsson Nokia Samsung |
|  | L1-RSRP, implicity Tx beam ID | Futurewei MediaTek |
| Pair | L1-RSRPs of Set B | CMCC Huawei/HiSilicon NTT docomo Oppo Vivo |
|  | L1-RSRPs of Set B and implicit beam information | CATT Ericsson Nokia Samsung |
|  | L1-RSRP, implicty Tx beam ID, and Rx beam ID | Futurewei |

As can be seen in the table above, there are multiple companies mentioning only L1 RSRPs of Set B as the AI model input while there are companies using L1-RSRPs and beam information. And in some cases, it was not clear that companies using L1-RSRPs of Set B and implicit beam information indicate whether that the beam information is Tx and/or Rx beam information. Also, although we checked the FL (Focus Lead) summary of RAN1 #119 from November [3], it appears that RAN1 has not concluded types of AI model input as of now. Therefore, to move forward the RAN4 discussion on the AI/ML beam management test setup, we think we should firstly clarify what the AI model input is.

Observation 2: AI model inputs varied per company in the evaluation results of RAN1 [2] and there are companies which used only L1-RSRPs of Set B as the model input.

Observation 3: From the summary in RAN1 [3], it appears that RAN1 has not concluded types of AI model input.

Proposal 1: To move forward the discussion on the AI/ML beam management test setup, RAN4 first clarify the types of AI model inputs for the prediction of beams.

Option 1: Mandatory AI model inputs for the beam prediction are L1-RSRP of set B and implicit Tx beam ID

Option 2: Mandatory AI model inputs for the beam prediction are L1-RSRP of set B and Tx beam ID

Option 3: Mandatory AI model inputs for the beam prediction are L1-RSRP of set B, Tx beam ID and Rx beam information (FFS for the necessary information of Rx beam)

Option 4: Others

In a case option 1 or 2 is valid, then it means following 1 AoA setup B in sub-clause 2.3 will be able to use either a standardized UE Rx beam pattern or the actually measured beam pattern par UE. In a case option 3 is valid, it means it is mandatory that the setup B measures the actual UE Rx beam pattern.

In addition to the issue above, though there were reports in TR 38.843 [2] on the evaluation results of AI models, it appears that a comparison of prediction performance between different types of model inputs in [2] has not been made yet. Hence, it is preferred that we analyze those aspects and study if there is any prediction performance differences between different types of model inputs.

Observation 4: No study has been done yet regarding if there are any prediction performance differences between different types of model inputs.

Proposal 2: RAN4 to study if there are any prediction performance differences between different types of model inputs.

# 2.2. Views on the applicability of simplified CDL for AI model evaluation

At the previous meetings, there were several companies which proposed applying the simplified CDL for AI model tests with a multiple-AoA test setup [9]. At the same time multiple companies proposed that we should compare the prediction performance in a case of simplified CDL, legacy CDL and channels based on the actual field data [5,6,7,8]. Considering the previous contributions in RAN4, we assume it is premature that we conclude simplified CDL can be applied for the evaluation tests from the viewpoint of securing the field performance. Therefore, it is suggested that the decision to apply the simplified CDL be deferred until this study has been conducted and until the prediction performance has been confirmed within RAN4.

Observation 5: It is necessary to ensure that tests with simplified CDL are done and show appropriate prediction performance of the AI model in the field.

Proposal 3: The decision to apply the simplified CDL should be deferred until the study has been conducted and until the prediction performance has been confirmed within RAN4.

# 2.3. Comparison of test setup for the AI/ML beam management prediction performance

In this sub-clause, we provide our views on pros and cons of test setups which were proposed up to the last meeting for the AI/ML beam management prediction performance tests as follows. Generally speaking, we assume there is a trade-off among these test setups, and we anyway need to further study to conclude what types of setups are applicable for the evaluation of beam prediction performance.

Setup A) Extended MPAC (Multi-AoA for example 10 test probes)

Setup B) 1AoA + Fading simulator with wireless (virtual) cable connection

Setup C) 1 or a few AoA without wireless (virtual) cable connection

Setup A) Extended MPAC (Multi-AoA for example 10 test proves)

Pros

Presumably shorter test time is expected compared to setup B.

Possible to use an actual UE Rx beam pattern without an additional measurement by TE side.

There is a possiblility of physically providing DL channels from spatially different directions.

Cons

There is a limitation of channel models, i.e. Simplified CDL needs to be used to reduce the system complexity.

Further study is necessary to ensure the applicability of simplified CDL.

At this moment this setup has no usages other than the BM tests.

A complex system and a huge footprint are required.

Setup B) 1AoA + Fading simulator with wireless (virtual) cable connection

Pros

Though the number of test probe is physically one and DL signals are transmitted from one direction, it is possible to test with CDL channel models by using a fading simulator.

There are no limits to channel models (AoDs, AoAs, clusters, UE positions, etc.), and even possible to use the actual field data for the tests.

Possible to reuse the existing 1AoA OTA test system. i.e. Simpler test system compared to Setup A.

Cons

There is a need to include UE Rx beam pattern to a fading simulator, either standardized one or need to obtain actual UE beam patterns.

If the actual UE Rx beam patterns are necessary, there is a need to measure at least one beam pattern before the test to include it in the fading simulator. The number of necessary beam patterns are FFS. i.e. Longer test time is expected in a case the actual Rx beam pattern is necessary.

In a case the UE autonomously aims the Rx beam and then measures the RSRP and inputs it to the AI model in the field, it may not be possible to simulate the behaviour of the UE.

The measurements are made while beam-locked by the UBF (UE Beamlock Function) command, so they do not necessarily reproduce the UE in operation on the real network.

Setup C) 1 or a few AoA without wireless (virtual) cable connection

Pros

Possible to use an actual UE Rx beam pattern without an additional measurement by TE side.

Presumably shorter test time is expected compared to setup B.

Possible to reuse the existing 1AoA or 2 AoA OTA test system. i.e. Simpler test system compared to Setup A.

Cons

Cannot reproduce all channel conditions.

Observation 6: There is a trade-off between multi-AoA and 1AoA test setups.

Observation 7: Further studies are necessary regarding applicable channel models and types of model inputs to conclude setups to be used for the AI/ML beam management tests.

# 2.4. Supplemental explanation regarding the Setup B: 1AoA IFF test setup + Fading simulator with wireless (virtual) cable connection

Last in this sub-clause, we would like to provide a supplemental explanation on the 1AoA test setup + Fading simulator with wireless (virtual) cable connection, which we provided the concept at the last meeting [4]. Here we would like to enphasize that this system is already providing solutions to the issues listed in the WF [1] and in other contributions at the last meeting.

Observation 8: 1AoA test setup + Fading simulator with wireless (virtual) cable connection is already providing solutions to the issues listed in the WF and in other contributions.

Following figure depicts the concept of test with 1AoA test setup + fading simulator with wireless (virtual) cable connection. Actual test procedures are explained in the following Appendix A at the end of this contribution.

Figure 2.4-1: Concept of test setup of AI/ML beam management performance evaluation by 1AoA +Fading simulator with wireless (virtual) cable connection

Table 2.4-1: Extract of system information from the previous contribution in RAN4#113 [4]

BTS simulator Legacy features as the system simulator are necessary. Fading simulator CDL channel model + multiple UE antenna beam steering patterns + UE orientations are all contained in its simulation. A difference between the legacy MPAC and this test setup is that the CDL models in the MPAC tests do not include the antenna beam steering patterns and UE orientations in their tests. It is preferred that the antenna beam steering patterns are also standardized. TBD if we need to measure antenna beam patterns of each DUT independently. OTA chamber The legacy 1AoA DFF/IFF antenna can be reused. A probe number can be 1 if we consider the rank 2 test. FFS on a need of 2AoA 4-layer tests. DUT Same as demodulation tests, a UE beam is locked at bore sight direction and the UE is connected to the test system by the wireless (virtual) cable connection. An interface and/or signalling are necessary to report inferred results to the test system based on a measurement of DL signals from the fading simulator.

This time we would like to describe the characteristics of this test system comparing with the issues identified until the last meeting.

- Channel model to be achieved

As mentioned in above sub-clause, it is not clear yet whether the simplified CDL can be applied for this evaluation of AI/ML beam management. On the other hand, since this 1AoA test system has no limitation on the channel model to simulate, which means that it is even possible to simulate actual field channel condition by the fading simulator, we don’t need to use the simplified CDL. In other words it means that we have no concern on the degradation of evaluation or training possibly caused by the simplified CDL.

Observation 9: 1AoA test setup + Fading simulator with wireless (virtual) cable connection has no limitation to simulate channel model, which solves the concern of the degradation of evaluation or training possibly caused by the simplified CDL.

And though it might be difficult to imagine that this 1AoA setup + fading simulator can conduct the test with CDL model by 1AoA, since this setup is achieving the connection between the DUT and test equipment by wireless (virtual) cable connection, it is not the simple 1AoA direct connection from the view point of simulation and possible to conduct the test with CDL models, which emulates the environment of multiple AoAs even under the 1AoA condition. Corresponding descriptions can be found in the previous technical report to study on test methods, at sub clause 7.2 and 8.2 [10].

Observation 10: By connecting between the DUT and test equipment by wireless (virtual) cable connection, it is possible to emulate the environment of multiple AoAs even with the 1AoA setup.

- Need of Rx beam information as the AI model input

It should be noted that this aspect is related to an understanding of RAN4 on the need of Rx beam information as the AI model input. But we would anyway discuss the solution to achieve the associated test by 1AoA.

Though it is not possible to measure actual Rx beam patterns without having ATF (Antenna Test Function) in DUT UEs, since the associated duplex mode for FR2 is TDD, we can obtain the Rx beam patterns by measuring the Tx beam patterns of the DUT assuming the reciprocity between Tx and Rx. And once we have obtained the necessary actual beam patterns which correspond to the direction of UE to be measured, we can use those beam patterns in the fading simulator and can simulate the dedicated CDL channel models per DUT. And as far as an actual UE Rx beam patterns are aligned with the calculated channel model in the fading simulator during the test, UE can use its own Rx beam information for the AI model inputs. FFS on the necessary number of Rx beam patterns (or UE positions) to be measured. By the way, regarding the prediction of the associated Tx beam or beam pair, our understanding is that the UE uses only a measurement result with one UE Rx beam in its AI/ML algorithm in order to predict the Tx beam or beam pair once, i.e. beam prediction by AI is made once per one mesurement result with one Rx beam direction with the UE and one Set B condition. Otherwise if the UE needs to sweep its Rx beam for the prediction of Tx beam or beam pair, we assume its behaviour is same with the legacy procedures to find best beam conditions and there is no meaning to use the AI model.

Observation 11: By measuring the actual UE Rx beam patterns by the test system and using them in the fading simulator, as far as the UE beam patterns are aligned with the calculated channel model in the fading simulator, UE can use its own Rx beam information for the AI model inputs.

Observation 12:  Regarding the prediction of the associated Tx beam or beam pair, the UE uses only a measurement result with one UE Rx beam in its AI/ML algorithm in order to predict the Tx beam or beam pair once. FFS on the necessary number of Rx beam patterns (or UE positions) to be measured.

Now most of the descriptions in this sub-clause 2.4 b) are discussed based on the case that the Rx beam information is input in the AI model. However, as mentioned at the beginning of this sub-clause 2.4 b), depending on the needs of the Rx beam information as the AI model inputs, use of UE Rx beam pattern in the fading simulator can be separately considered as follows.

Observation 13: For setup B, use of UE Rx beam pattern in the fading simulator can be separately considered as follows.

AI model input case 1: UE Rx beam information is AI model input

A measurement of actual UE Rx beam pattern is necessary.

AI model input case 2: UE Rx beam information is not AI model input

Either of the following two options can be applied.

- Use a standardized UE Rx beam pattern in the fading simulator.

- Use an actually measured UE Rx beam pattern in the fading simulator, though it is not mandatory.

Comparing cases between the AI model input case 2 a) and 2 b), AI model input case 2 b) has a benefit that the fading simulator can use the actual UE Rx beam pattern in its channel simulation with a cost of an additional measurement time, though it is not mandatory to obtain the UE Rx beam pattern.

- Conclusion

In this contribution we shared our findings of the situation regarding the associated discussion on AI/ML beam management evaluation among RAN1 and RAN4, and we provided our views for them.

Observation 1: It is necessary to clarify characteristics of the UE Rx beam knowledge and their necessity for AI model input since they have a huge impact on the 1AoA test procedure and setup with wireless (virtual) cable connection.

Observation 2: AI model inputs varied per company in the evaluation results of RAN1 [2] and there are companies which used only L1-RSRPs of Set B as the model input.

Observation 3: From the summary in RAN1 [3], it appears that RAN1 has not concluded types of AI model input.

Proposal 1: To move forward the discussion on the AI/ML beam management test setup, RAN4 first clarify the types of AI model inputs for the prediction of beams.

Option 1: Mandatory AI model inputs for the beam prediction are L1-RSRP of set B and implicit Tx beam ID

Option 2: Mandatory AI model inputs for the beam prediction are L1-RSRP of set B and Tx beam ID

Option 3: Mandatory AI model inputs for the beam prediction are L1-RSRP of set B, Tx beam ID and Rx beam information (FFS for the necessary information of Rx beam)

Option 4: Others

Observation 4: No study has been done yet regarding if there are any prediction performance differences between different types of model inputs.

Proposal 2: RAN4 to study if there are any prediction performance differences between different types of model inputs.

Observation 5: It is necessary to ensure that tests with simplified CDL are done and show appropriate prediction performance of the AI model in the field.

Proposal 3: The decision to apply the simplified CDL should be deferred until the study has been conducted and until the prediction performance has been confirmed within RAN4.

Observation 6: There is a trade-off between multi-AoA and 1AoA test setups.

Observation 7: Further studies are necessary regarding applicable channel models and types of model inputs to conclude setups to be used for the AI/ML beam management tests.

Observation 8: 1AoA test setup + Fading simulator with wireless (virtual) cable connection is already providing solutions to the issues listed in the WF and in other contributions.

Observation 9: 1AoA test setup + Fading simulator with wireless (virtual) cable connection has no limitation to simulate channel model, which solves the concern of the degradation of evaluation or training possibly caused by the simplified CDL.

Observation 10: By connecting between the DUT and test equipment by wireless (virtual) cable connection, it is possible to emulate the environment of multiple AoAs even with the 1AoA setup.

Observation 11: By measuring the actual UE Rx beam patterns by the test system and using them in the fading simulator, as far as the UE beam patterns are aligned with the calculated channel model in the fading simulator, UE can use its own Rx beam information for the AI model inputs.

Observation 12:  Regarding the prediction of the associated Tx beam or beam pair, the UE uses only a measurement result with one UE Rx beam in its AI/ML algorithm in order to predict the Tx beam or beam pair once. FFS on the necessary number of Rx beam patterns (or UE positions) to be measured.

Observation 13: For setup B, use of UE Rx beam pattern in the fading simulator can be separately considered as follows.

AI model input case 1: UE Rx beam information is AI model input

A measurement of actual UE Rx beam pattern is necessary.

AI model input case 2: UE Rx beam information is not AI model input

Either of the following two options can be applied.

- Use a standardized UE Rx beam pattern in the fading simulator.

- Use an actually measured UE Rx beam pattern in the fading simulator, though it is not mandatory.

# 4.	References

[1] R4-2420513, “WF on AI/ML air interface part 1”, Moderator (Qualcomm), RAN4 #113, Orlando, USA

[2] TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, V18.0.0

[3] R1-2410892, “FL summary #5 for AI/ML in beam management”, Moderator (Samsung), RAN1 #119, Orlando, USA

[4] R4-2417673, “FR2 test setup for AI/ML beam management evaluation”, Anritsu, RAN4 #113, Orlando, USA

[5] R4-2414930, “Discussion on testability issues of AI/ML BM use case”, MediaTek Inc., RAN4 #112-bis, Hefei, China

[6] R4-2418522, “Discussion on Beam Management: Testability and Interoperability Issues”, Apple, RAN4 #113, Orlando, USA

[7] R4-2418900, “Discussion on testability and interoperability issues for beam management”, Ericsson, RAN4 #113, Orlando, USA

[8] R4-2417894, “Discussion on testability issues of AI/ML BM use case”, MediaTek Inc., RAN4 #113, Orlando, USA

[9] R4-2418151, “Topic summary for [113][129] NR_AIML_air_part1”, Moderator (Qualcomm), RAN4 #113, Orlando, USA

[10] TR 38.810, “Study on test methods”, v16.6.1

# Appendix A: Measurement procedures of Setup B) 1AoA + Fading simulator with wireless (virtual) cable connection

In this appendix, we provide supplemental explanations on the measurement procedures of setup B in sub-clause 2.3 and 2.4. As mentioned at the observation 13 in sub-clause 2.4, measurement procedures vary depending on an assumption whether the UE Rx beam pattern is the AI model input or not.

AI model input case 1: UE Rx beam information is AI model input

A measurement of actual UE Rx beam pattern is necessary.

AI model input case 2: UE Rx beam information is not AI model input

Either of the following two options can be applied.

- Use a standardized UE Rx beam pattern in the fading simulator.

- Use an actually measured UE Rx beam pattern in the fading simulator, though it is not mandatory.

We will explain the associated measurement procedure for each case as follows.

AI model input case 1: UE Rx beam information is AI model input

- Set a UE on a positioner in an OTA chamber.

- Establish a connection between the UE and a BTS simulator that is connected to a test antenna equipped in the OTA chamber.

- Change the UE orientation in the OTA chamber using the positioner while letting the UE adjust the Rx-beam direction autonomously to receive the DL signal from the test antenna.

- Stop the positioner at a certain position and set the UE in the Rx-beam-lock mode by UBF test command. The Rx-beam-lock mode needs to be kept in step 5, 6 and 7.

- Rotate the UE orientation by the positioner to measure a 3-dimensional Tx-antenna pattern of the UE, while assuming reciprocity between Tx and Rx antenna pattern. (Without ATF: Antenna Test Function being equipped in the UE, it wouldn’t be possible to directly measure Rx-antenna pattern.)

- Set the UE orientation to the same position as in step 4 and establish a wireless (virtual) cable connection. By this connection, DL signals based on the simulated channel model will be distributed virtually directly to an antenna connector of the UE.

- Set the measured Tx/Rx-antenna pattern to the CDL channel model in the fading simulator to perform UE testing for AI/ML beam management, where the Rx-beam ID in UE recognition should be aligned with the Rx-antenna pattern in the fading simulator.  (If necessary, we can perform testing several times with various UE orientations in the simulation while fixing the Rx-beam ID.)

Cross-polarized antenna

UE

Positioner

Step 4. : Stop changing UE orientation and set Rx-beam-lock mode

Cross-polarized antenna

UE

Positioner

Step 5. : Rotate UE while measuring Tx-antenna pattern

Cross-polarized antenna

UE

Positioner

Step 6. : Set UE orientation to the one in step4 to establish wireless-cable connection

Wireless cable connection

Rotate

If it is required to perform testing with multiple Tx/Rx-antenna patterns or Rx-beam IDs, repeat from step 3 to step 7.

AI model input case 2 a): UE Rx beam information is not AI model input and use a standardized UE Rx beam pattern in the fading simulator

- Set a UE on a positioner in an OTA chamber.

- Establish a connection between the UE and a BTS simulator that is connected to a test antenna equipped in the OTA chamber.

- Change the UE orientation in the OTA chamber using the positioner while letting the UE adjust the Rx-beam direction autonomously to receive the DL signal from the test antenna.

- Stop the positioner at the position where the UE Rx beam directs towards the bore sight direction and set the UE in the Rx-beam-lock mode. The Rx-beam-lock mode needs to be kept in step 5 and step 6.

- Establish a wireless (virtual) cable connection.

- Set a standardized Rx-antenna pattern to the CDL channel model in the fading simulator to perform UE testing for AI/ML beam management.  (If it is required for the AI model to correctly output inferring results with any Rx-antenna steering patterns, it would be necessary to perform testing multiple times with various combinations of a UE orientation and a standardized Rx-antenna pattern being loaded into the fading simulator.)

AI model input case 2 b): UE Rx beam information is not AI model input and use an actually measured UE Rx beam pattern in the fading simulator

The measurement procedure for “AI model input case 2 b)” would be similar to the one in “AI model input case 1”.  The difference would be as the following.

- The same measurement procedure for “AI model input case 1” can be used for “AI model input case 2 b)”, but it is not required that the Rx-beam recognition in the UE and the Rx-beam pattern used in the fading simulator are aligned.  So, in the case of “AI model input case 2 b)”, it should be allowed to change the Rx-beam direction between the time when measuring the Tx/Rx-beam pattern and the time when establishing the wireless connection.  If necessary, the wireless connection can be established with UE Rx-beam directed towards the bore sight direction.

Comparing cases between the AI model input case 2 a) and 2 b), AI model input case 2 b) has a benefit that the fading simulator can use the actual UE Rx beam pattern in its channel simulation with a cost of an additional measurement time.