3GPP TSG-RAN WG4 Meeting # 114	R4- 2500169

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM impacts of UE-to-UE CLI handling

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on the remain open issues on RRM impacts of UE-to-UE CLI handle described in WF [1].

Discussion

Issue 1-1-3: Rx beam - Agreements (from Thu AH session) - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec. - Rx beam sweeping is not needed for L1 CLI measurement.

Regarding the Rx beam configuration/determination for L1 based UE-to-UE CLI measurement, RAN1 agreed as below:

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

RAN4 can discuss how to capture the Rx beam determination for L1 CLI measurement. We think the statements of RAN1 agreements are well summarized the core meaning. So, RAN4 can capture the original statements of RAN1 agreements. If necessary minor change based on the statement can be considered. And additionally, RAN4 needs to the spec. structure also. Currently the CLI measurement spec. structure is as below:

9.7 Cross Link Interference measurements 9.7.1 Introduction 9.7.2 SRS-RSRP measurements 9.7.3 CLI-RSSI measurements 9.7.4 Scheduling availability of UE during CLI measurements 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements <- can be one candidate option.

The measurement items L1 CLI-RSSI/CLI-SRS-RSRP are introduced in Rel-19. So, we think new clause under 9.7 Cross Link Interference measurements can be considered. For example, 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements. In this case the contents of 9.7.1 should be updated to include the L1 CLI-RSSI/CLI-SRS-RSRP measurements. Or new clause 9.7X also can be considered for CLI measurement for SBFD. We need to discuss the spec. structure for CLI measurement of the SBFD.

Proposal 1: RAN4 can capture the original statements of RAN1 agreements. If necessary minor change based on the statement of RAN1 agreements can be considered.

Proposal 2: RAN4 need to discuss the spec. structure for CLI measurement of the SBFD.

- Option 1: Sub clause under 9.7. (e.g. 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements)

- Option 2: New clause for CLI measurement of the SBFD. (E.g. 9.7X Cross Link Interference measurements for SBFD)

Issue 1-1-4: Measurement resources - Proposal 1 (CATT): - Configuration with two discontinuous DL subbands should be taken into account when simulating and defining measurement accuracy requirements for CLI RSSI measurements - Proposal 2 (LGE, HW, MTK, Ericsson): - RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements. - Proposal 3 (Nokia): - RAN4 should define new requirements for the sub-band CLI-RSSI reporting if this feature is supported by RAN1. - Proposal 4 (Samsung): - RAN4 focuses on wideband L1-CLI-RSRI and L1-SRS-RSRP for L1 CLI measurement. Recommended WF - RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

During the last RAN1#119 meeting some CLI measurement resource related issues are agreed as below:

Agreement Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement - Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis) - FFS: Other configurations Define SRS-RSRP-MeasurementResource with the following parameters: - Legacy SRS Resource IE - FFS: Other parameters Agreement Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement. - Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis) Define CLI-RSSI-MeasurementResource with the following parameters: - CLI-RSSI measurement resource ID - Starting PRB index - Number of PRBs - Starting symbol of the CLI-RSSI resource within a slot - Number of symbols of the CLI-RSSI resource within a slot - Periodicity and slot offset for the CLI-RSSI resource - FFS: Other parameters

According to the agreement, RAN1 is still discussing about measurement resource and has not reached the conclusion yet. So, RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether there is an impact on the requirements.

Proposal 3: RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

Issue 1-1-7: Measurement reporting - Proposal 1 (vivo): - Define requirements for periodic, semi-persistent reporting. - Proposal 2 (Samsung, CTC, HW, MTK): - Focus on aperiodic CSI reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1 - Proposal 3 (CATT, Nokia, HW, E///): - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused, and wording can be adapted when reporting scheme design is stable. - Proposal 4 (MTK): - FFS whether L1-RSRP measurement reporting requirement can be re-used pending on RAN1 agreement.

RAN1 decided to support aperiodic CSI reporting mode for Rel19 CLI measurement. But regarding other reporting modes, RAN1 doesn’t reach any conclusion yet. So, we prefer to focus on aperiodic CSI reporting mode for Rel-19 CLI measurement, and for other reporting modes hold on the discussion pending on further progress in RAN1.

Proposal 4: Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

Issue 1-1-12: Requirement applicability - Proposal 1 (LGE): - L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - SRS-RSRP measurement resource is fully confined within BW of UL sub-band - CLI-RSSI measurement resource is configured within DL sub-band - Proposal 2 (QC): - The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

According to the agreed measurement methods from RAN1, the additional applicability for L1 based UE-to-UE CLI measurement is necessary.

Proposal 5: The additional applicability for L1 based UE-to-UE CLI measurement need to be considered as follow:

L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of UL sub-band - when CLI-RSSI measurement resource is configured within DL sub-band

Summary

Proposal 1: RAN4 can capture the original statements of RAN1 agreements. If necessary minor change based on the statement of RAN1 agreements can be considered.

Proposal 2: RAN4 need to discuss the spec. structure for CLI measurement of the SBFD.

- Option 1: Sub clause under 9.7. (e.g. 9.7.5 L1 CLI-RSSI/CLI-SRS-RSRP measurements)

- Option 2: New clause for CLI measurement of the SBFD. (E.g. 9.7X Cross Link Interference measurements for SBFD)

Proposal 3: RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the

requirements.

Proposal 4: Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

Proposal 5: The additional applicability for L1 based UE-to-UE CLI measurement need to be considered as follow:

L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of UL sub-band - when CLI-RSSI measurement resource is configured within DL sub-band

Reference

[1] R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei