3GPP TSG-RAN WG4 Meeting # 114	R4- 2500173

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues of RRM core requirements for NR LP-WUS/WUR based on listed open issues in approved WF [1].

Discussion

- Topic #1: RRM core requirements for LP-WUS/WUR

General aspects

### Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

RAN4 has been agreed to support case#1 and #3. But regarding case#2 scenario, we can follow the RAN2’s decision. During last RAN2 meeting they didn’t reach conclusion.

RAN2#128 (Nov.2024) meeting RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

From the RAN2’s discussion, if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation, then it means not to support case#2. Else, it means to support case#2.

So, regarding case#2 scenario, we think RAN4 need to wait for RAN2’s conclusion.

Proposal 1: Regarding case#2 scenario, RAN4 need to follow RAN2’s conclusion.

For criteria (entry/exit) related issues such as fully offloading, MR RRM measurement relaxation, and LP-WUS monitoring, RAN2 is still under disussing. Therefore, it would be up to RAN2’s discussion. RAN4 can further discuss after RAN2’ conclusion if needed.

Proposal 2: Criteria related issues are up to RAN2’s consideration.

Issue 1-1-8: LP-WUR operating carrier frequency

During last meeting RAN4 discussed on the reply LS, but doesn’t reached to conclusion as follow:

- Option 1: Include RAN4 112bis RF session’s agreement in the reply LS, and RAN4 112bis RRM session’s agreement without FFS - Option 2: Include RAN4 112bis RF session’s agreement in the reply LS, and RAN4 112bis RRM session’s agreement with FFS: - no specific RF work related to RAN1 LS. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. - Option 2a: FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. - Option 2b: Note: FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. - Option 2c Include reference of RAN4 112bis RRM WF R4-2417112 or attach it in the reply LS. - Option 3: Include only RAN4 112bis RF session’s agreement in the reply LS (to be handled in RF session) - no specific RF work related to RAN1 LS.

Regarding this issue, the WF is endorsed during RANP#106 meeting as follow:

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Detailed discussions and final solution selection is left up to WGs.

According to the agreement of RANP#106, for Rel-19, RAN4 do not need to consider different carrier frequency case. And RAN4 do not need to send reply LS because there are agreement in the RANP.

Proposal 3: RAN4 do not need to send reply LS.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offlading(case 1) case/RRM relaxation (case 3) case)

At legacy status, the initial LP-WUR status can be OFF status. And to turn on the LP-WUR the MR measurement results should be reference because the LP-WUR measurement results are not available.

MR power and LR power are correlated, but it doesn’t quarantee exact specific LR quality Pth1 at Pth1 of MR quality. So, the evaluation step can be necessary for confirm the LR quality which is on discussion at issue1-2-3. From the figure, to turn on the LR at Pth1 LR quality, the LR should be turn on at Pth1+Δ of MR quality. The Δ value is added to compensate the difference between MR and LR quality. It can be ignorable or not. The Δ value can be impacted by various factors including UE implementation. So, if it is guaranteed that LR turn on at LR quality is higher than or equal to Pth1, then when or how to turn on LR can be UE implementation issue.

Proposal 4: If UE guarantee that the LR turn on at LR quality is higher than or equal to specific quality, then when or how to turn on LR can be UE implementation.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

### Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

In the last RAN4#113 meeting, the periodicity for SSB based LP-WUR measurement delay requirements are discussed as follow:

Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

When LP-SS is configured, the LP-SS periodicity is not important for SSB based LP-WUR measurement delay requirements. Even the LO periodicity is smaller than LP-SS periodicity, the UE measure SSB during DRX cycle which has same periocity with LO. So, when the LO periodicity is smaller than LP-SS periodicity, it doesn’t impact to the SSB based LP-WUR measurement delay requirements. We think RAN4 do not need to care whether LO periodicity is smaller than LP-SS periodicity or not for SSB based LP-WUR measurement delay requirements.

Proposal 5: RAN4 do not need to care whether LO periodicity is smaller than LP-SS periodicity or not for SSB based LP-WUR measurement delay requirements.

### Issue 1-2-3: On requirements for LP-WUR for entry/exit criteria(threshold) evaluation

The moderator’s recommendation in WF[1] is captured as below:

Recommendations: Encourage companies firstly to consider whether to differentiate requirements based on cases Note: For entry condition evaluation requirements, requirements from legacy state to case 1 or case 3 or LP-WUS monitoring will be discussed separately. If No Same requirements will be defined on entry criteria(threshold) evaluation from one case to the other case (excluding scenarios in the note). Same requirements will be defined on exit criteria(threshold) evaluation from one case to the other case. If Yes, consider the following cases one by one Issue 1-2-3a: On requirements for entry/exit criteria(threshold) evaluation for Case 1 Issue 1-2-3b: On requirements for entry/exit criteria(threshold) evaluation for Case 3 Issue 1-2-3c: On requirements for entry/exit criteria(threshold) evaluation for LP-WUS monitoring On detailed evaluation requirements, the following options are available - Option 1: Evaluation requirements for entry/exit evaluation is the same as that of LP-WUR measurement period. - Option 2: Evaluation requirements for entry/exit conditions is longer than that of LP-WUR measurement period. - Option 3: Evaluation requirements for entry evaluation is longer than that of LP-WUR measurement period; evaluation requirements for exit evaluation is the same as that of the LP-WUR measurement period.

For entry condition evaluation requirements, we think each scenarios require different environments. So, the requirements from legacy state to case 1 or case 3 or LP-WUS monitoring should be discussed separately.

### Issue 1-2-3a: On requirements for entry/exit criteria(threshold) evaluation for Case 1

According to the RAN2 agreement, the entry condition for serving cell RRM relaxation is as follow:

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

Similar to RRM relaxation case, the fully offloading case also can be considered MR results based threshold. And we think in this case the evaluation is not necessary.

Proposal 6: The entry condition to case 1 can be determined based on MR results. And the evaluation is not necessary.

Regarding exit condition from MR fully offloading case, it also can be decided by MR based measurements. But in this case there are no available MR results. So, indirectly LR results can be referred. If the LR result is higher than configured threshold, then MR can be turn on. And then the evaluation procedure can be considered based on the MR results.

Proposal 7: The exit condition from case 1, indirectly LR results can be referred. If the LR result is higher than configured threshold, then MR can be turn on. And then the evaluation procedure can be necessary based on MR results.

### Issue 1-2-3b: On requirements for entry/exit criteria(threshold) evaluation for Case 3

Based on RAN2 agreements, the entry condition to case 3 can be based on MR results. And the evaluation is not necessary.

Proposal 8: The entry condition to case 3 can be determined based on MR results. And the evaluation is not necessary.

Regarding exit condition for serving cell RRM relaxation, RAN4 need to RAN2’s further discussion.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

Proposal 9: Regarding exit condition for serving cell RRM relaxation, RAN4 need to RAN2’s further discussion.

### Issue 1-2-3c: On requirements for entry/exit criteria(threshold) evaluation for LP-WUS monitoring

Regarding LP-WUS monitoring entry/exit critera, RAN2 made an agreement at RAN2#127bis meeting as below:

- If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

- The LPWUS monitoring exit condition does not include MR measurements.

So, the LP-WUS monitoring exit criteria(threshold) should be decided based on LR results. But when entry case, if the LR status was OFF before entry the LP-WUS monitoring, then LR can be turn on based on the MR results and after that, the evaluation can be necessary by LR results. Else if the LR status was ON before entry the LP-WUS monitoring, then both MR and LR results can be used for checking entry condition.

Proposal 10: For LP-WUS monitoring entry case, if the LR status is OFF before entry the LP-WUS monitoring, then LR can be temporary turn on based on the MR results and after that, the evaluation can be necessary by LR results based on RAN2 agreement. Else if the LR status was ON before entry the LP-WUS monitoring, then both MR and LR results can be used for checking entry condition based on RAN2 agreement.

### Issue 1-2-4: On interruption related requirements for LP-WUR

- Proposals - P1: Wait for RAN1 further progress or based on RAN1’s conclusion. (CATT CMCC oppo Huawei) - P2: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. (Apple) - P3: Suggest to use the term “MR activation” time to replace “interruption” time; Defining interruption requirement for the normal case and worst case where a cell reselection is included in the worst case, ramping duration is based on RAN1 conclusion, detailed conditions are FFS. (vivo) - “MR activation” duration = Ramping duration + (2 + 2) samples * SSB_ periodicity - “MR activation” duration = Ramping duration + (2 + 2 + 23) samples * SSB_ periodicity + TSI-NR + 2*Ttarget_cell_SMTC_period - P4: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (Samsung) - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g. 1~2 SSB sufficient - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

When LR include the indication for MR activation, the MR should wake-up after wake-up delay. The wake-up delay include the ramp up duration and MR sync procedure time. The ramp up duration can be UE capability and the required time for MR sync procedure depends on the MR conditions. The MR wake-up delay requirement can be discussed based on Proposal 4.

Proposal 11: MR wake-up delay requirement can be discussed based on P4 as below.

  - P4: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (Samsung)

    - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g.  1~2 SSB sufficient

    - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

In the last RAN4 #113 meeting we made below agreements,

Agreement: Scaling factor(s) is/are >= 4. Further down-select from - Option 1: 1 fixed scaling factor - Option 2: multiple values configured by NW - Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo) DRX cycle length [s]; Scaling factor. 0.32; TBD. 0.64; TBD. 1.28; TBD. 2.56; TBD.

Based on the LP-WUR operation, UE can further save power consumption in IDLE/INACTIVE mode. So, in extreme case, MR based measurement can be off in cell center area (i.e., fully offloading case). If UE moves toward to cell edge region, normal MR based measurement should be performed. It means that different scaling factors for measurement relaxation could be considered based on the UE location not DRX cycle length. And now, RAN2 does not consider UE mobility condition (e.g., low or high UE mobility) for measurement relaxation in Rel-19. Therefore, network can configure multiple thresholds (e.g., two or three thresholds), and different sacling factors can be applied depending on the configured threshold to ensure UE power saving.

Proposal 12: For measurement relaxation, network can configure multiple thresholds (e.g., two or three thresholds), and different sacling factors can be applied depending on the configured thresholds.

Conclusion

In this contribution, we provide views on issues listed in WF [1], and we propose

Proposal 1: Regarding case#2 scenario, RAN4 need to follow RAN2’s conclusion.

Proposal 2: Criteria related issues are up to RAN2’s consideration.

Proposal 3: RAN4 do not need to send reply LS.

Proposal 4: If UE guarantee that the LR turn on at LR quality is higher than or equal to specific quality, then when or how to turn on LR can be UE implementation.

Proposal 5: RAN4 do not need to care whether LO periodicity is smaller than LP-SS periodicity or not for SSB based LP-WUR measurement delay requirements.

Proposal 6: The entry condition to case 1 can be determined based on MR results. And the evaluation is not necessary.

Proposal 7: The exit condition from case 1, indirectly LR results can be referred. If the LR result is higher than configured threshold, then MR can be turn on. And then the evaluation procedure can be necessary based on MR results.

Proposal 8: The entry condition to case 3 can be determined based on MR results. And the evaluation is not necessary.

Proposal 9: Regarding exit condition for serving cell RRM relaxation, RAN4 need to RAN2’s further discussion.

Proposal 10: For LP-WUS monitoring entry case, if the LR status is OFF before entry the LP-WUS monitoring, then LR can be temporary turn on based on the MR results and after that, the evaluation can be necessary by LR results based on RAN2 agreement. Else if the LR status was ON before entry the LP-WUS monitoring, then both MR and LR results can be used for checking entry condition based on RAN2 agreement.

Proposal 11: MR wake-up delay requirement can be discussed based on P4 as below.

  - P4: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (Samsung)

    - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g.  1~2 SSB sufficient

    - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

Proposal 12: For measurement relaxation, network can configure multiple thresholds (e.g., two or three thresholds), and different sacling factors can be applied depending on the configured thresholds.

Reference

[1] R4-2420100, “WF on RRM requirements for NR_LPWUS,” vivo