3GPP TSG RAN WG4 #114	R4-2500202

Athens, Greece, Feb 17 – 21, 2025

Agenda item:		7.13.4.2 (Identification and reduction of BS OTA test scope)

Source:	Nokia

Title:	On BS OTA test scope reduction

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#113 regarding the performance part of OTA test enhancements. Listed in the WF [2] is the work plan for the work item. According to the schedule, for RAN4#114, discussion is to be continue on the BS OTA test scope reduction. This contribution presents further elaborates Nokia’s views on the topic.

# Discussion

## BS OTA test scope reduction

It was agreed in RAN4#110bis in [2]:

| RAN4#114 (2/2025) | Core: - Continue to discuss the applicability of existing co-location reference antenna and identify the problems if any. Perf: - Continue to identify BS excessive OTA test scope cases and try to reduce it with the aim to shorten the test duration time for selected RF requirements - Continue to discuss how to simplify BS TRP test methods for FR1 and FR2 by improving the applicability - Continue to discuss the conformance testing requirement for co-location reference antenna if any |
| --- | --- |

Following agreements on OTA scope reductions are included in the agreed Way forward [1]:

Issue 1-1: test scope reduction for Tx requirement - Recommended WF: - Focus on test scope reduction for Tx IMD requirement - Further discuss the applicable interference freq offset for ACLR, OBUE and spurious emission requirements; - Further discuss the test scope reduction of test configuration. Issue 1-2: Out of band blocking - Recommended WF: - Focus on test scope reduction for Rx OOBB requirement - Focus on the measurement step size for Rx OOBB requirement;

### OTA Transmitter Intermodulation (TX IM)

The OTA transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station. The requirement applies during the transmitter ON period and the transmitter transient period.

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to restrict the generation of intermodulation products in its nonlinear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station to below specified levels.

The transmitter intermodulation level shall not exceed the TRP unwanted emission limits specified for OTA transmitter spurious emission in clause 6.7.5 (except co-location with other base stations), OTA out-of-band emissions in clause 6.7.4 and OTA ACLR 6.7.3. Naturally due to TRP test method and CLTA usage, execution of these cases requires quite a much of test effort.

Currently there are 6 interferer frequency offsets in test case, 3 on each side of the carrier defined as the following:

, for n=1, 2 and 3

For NC operation this is twice more so it is 12. Historically this approach was introduced in UTRA specification and later was reused in E-UTRA, MSR and NR. However currently number of the offsets should be reconsidered which would effectively impact the test efforts while still keeping the requirement coverage.

- Observation 1: Currently, for TX IM there are 6 interferer offsets, 3 on each side of the carrier.

- Proposal 1: To reconsider some of the interferer offsets while still keeping the requirement coverage.

### OTA out-of-band blocking

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

The test stresses the ability of the receiver unit associated with the RIB under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

The OTA general blocking interferer is injected via test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. CW interfering signal is swept with a step size of 1 MHz within the frequency range corresponding to 30 MHz to FUL_low - ΔfOOB and from FUL_high + ΔfOOB up to 12750 MHz for FR1. Whilst for FR2, 1 MHz step size applies for frequency range 30 MHz to 6000 MHz, and above 6000MHz bigger step sizes is defined.

The OTA co-location blocking interferer for FR1 is injected via the CLTA. The CLTA is fed with the specified co-location blocking interferer power per supported polarization. CW interfering signal is swept with a step size of 1 MHz within the frequency range corresponding to downlink operating bands related to co-located systems

- Observation 2: Interfering signal step size is 1 MHz in frequency range 30-12750MHz for FR1 and in frequency range 30-6000MHz for FR2.

- Observation 3: Out of all test requirements of radiated conformance test specification TS 38.141-2, the OTA out-of-band blocking case is one of the cases which requires the largest test efforts.

This is largely due to 1 MHz step size used and, therefore, the number of measurements points is significantly large. Increase of step size from 1 MHz to X MHz effects test efforts proportionally.

Observation 4: Increase of interferer step size from 1 MHz to X MHz effects test efforts proportionally.

To reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity, the measurement step size for interfering signal can be set according to the minimum supported BS channel bandwidth.

- Proposal 2: To increase interferer step size of 1 MHz to X MHz, where X is depends on channel bandwidth.

# Conclusion

This document shares Nokia’s views on simplification of BS TRP test methods. Following observations and proposals were introduced:

- Observation 1: Currently, for TX IM there are 6 interferer offsets, 3 on each side of the carrier.

- Proposal 1: To reconsider some of the interferer offsets while still keeping the requirement coverage.

Observation 2: Interfering signal step size is 1 MHz in frequency range 30-12750MHz for FR1 and in frequency range 30-6000MHz for FR2.

Observation 3: Out of all test requirements of radiated conformance test specification TS 38.141-2, the OTA out-of-band blocking case is one of the cases which requires the largest test efforts.

Observation 4: Increase of interferer step size from 1 MHz to X MHz effects test efforts proportionally.

- Proposal 2: To increase interferer step size of 1 MHz to X MHz, where X is depends on channel bandwidth.

# References

- R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE

- R4-2406088 “Way Forward for [110bis][311] NR_BS_RF”, RAN4#110bis, ZTE Corporation